| 3GPP TR 28.811 V0.5.0 (2021-05) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Management and orchestration; Network Slice Management Enhancement (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

Introduction	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	8

3.3	Abbreviations	8

4	Concepts and overview	8

4.1	Network slice management terms	8

4.2	Multi-operator relationships in network slice management	9

4.3	End to end management of network slice	10

4.4	Multi-operator concepts	10

4.4.1	National roaming concept	10

5	Scenarios for Network Slicing Management Enhancements	11

5.1	Network Slice use in multiple networks scenario 1	11

5.1.1	Description RAN sharing	11

5.1.2	Identified problems and general issues	12

5.2	Network slice use in multiple networks scenario 2	12

5.2.1	Description national roaming	12

5.2.2	Identified problems	12

5.3	Use case – Isolated deployment of multiple network slices of same customer	13

5.3.1	Introduction	13

5.3.2	Issue and gaps	14

5.4	Deployment of edge application service	14

5.4.1	Description	14

5.4.2	Identified problems	14

5.5	Use case – Support network slice specific authentication	14

5.5.1	Description	14

5.5.2	Issue and gaps	15

5.6	use case – support network slice protection on N6 interface	15

5.6.1	Description	15

5.6.2	Issue and gaps	15

6	Potential requirements for Network Slicing Management Enhancements	16

6.1	Potential requirements for Network Slicing Management to support roaming	16

6.2	Potential requirements for Network Slicing Management to support network slice isolation	16

6.3	Potential requirements for Network Slicing Management to support edge computing	16

6.4	Potential requirements for Network Slicing Management to support network slice specific authentication	16

6.5	Potential requirements for Network Slicing Management to support network slice protection on N6 interface	17

7	Possible solutions for Network Slicing Management Enhancements	17

7.1	Possible solutions for network slice isolation	17

7.2	Possible solutions for network slice covering multiple networks	18

7.2.1 Alternative 1: solution based on the existing NRM with related exposure	18

7.2.1.1 Description	18

7.2.1.2 Solution analysis	19

7.2.2 Alternative 2: solution based on NRM extension - dual views of network slice	20

7.2.3 Alternative 3: solution based on NRM extension - recursive network slice	20

7.2.4 Alternative 4: solution based on - support network slice subnet as a service	20

7.2.5	Solution for national roaming	22

7.3	Possible solutions for network slice specific authentication	22

8	Conclusions and recommendations	23

Annex A (informative): Change history	24

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

can		indicates that something is possible

cannot		indicates that something is impossible

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

# Introduction

# 1	Scope

The present document describes potential enhancements to network slice management.

The present document considers the following issues:

- The potential network slice information model and management service enhancements to support cross-operator network slice management use case.

- The potential new management capabilities to support end to end network slicing.

- The potential new management capabilities to support security management of network slice.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 23.501 v16.6.0: "System architecture for the 5G System (5GS); Stage 2"

[2]	3GPP TS 28.541 v17.0.0: "5G Network Resource Model (NRM); Stage 2 and stage 3"

[3]	3GPP TS 28.531 v16.7.0: "Management and orchestration; Provisioning"

[4]	3GPP TS 28.530 v16.3.0: "Management and orchestration; Concepts, use cases and requirements"

[5]	3GPP TS 28.532 v16.6.0: "Management and orchestration; Generic management services"

[6]	3GPP TS 28.545 v16.1.0: "Management and orchestration; Fault Supervision (FS)"

[7]	3GPP TS 28.550 v16.7.0: "Management and orchestration; Performance assurance"

[8]	3GPP TS 28.554 v17.1.0: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)"

[9]	3GPP TS 23.558 v1.2.0: "Architecture for enabling Edge Applications"

[10]	3GPP TS 22.011 v17.3.0 “Service accessibility”

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

<defined term>: <definition>.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<Abbreviation>	<Meaning>

# 4	Concepts and overview

## 4.1	Network slice management terms

The following terms are defined by SA2 in 3GPP specifications

| Term | Reference | Definition |
| --- | --- | --- |
| Network Slice | TS 23.501 [1] | A logical network that provides specific network capabilities and network characteristics. |
| Network Slice instance | TS 23.501 [1] | A set of Network Function instances and the required resources (e.g. compute, storage and networking resources) which form a deployed Network Slice. |
| NSI ID | TS 23.501 [1] | An identifier for identifying the Core Network part of a Network Slice instance when multiple Network Slice instances of the same Network Slice are deployed, and there is a need to differentiate between them in the 5GC. |

The following terms are defined by SA5 in 3GPP specifications

| Term | Reference | Definition |
| --- | --- | --- |
| network slice | TS 28.530 [4] | A logical network that provides specific network capabilities and network characteristics, supporting various service properties for network slice customers. |
| <<InformationObjectClass>> NetworkSlice | TS 28.541 [2] | This IOC represents the properties of a network slice instance in a 5G network. |
| NetworkSlice instance | TS 28.530 [4] | A Managed Object Instance (MOI) of NetworkSlice IOC. |
| NetworkSlice identifier | TS 28.531 [3] | To identify a NetworkSlice instance defined in TS 28.530, it is DN of a managed object instance of NetworkSlice IOC [2]. |
| cNSIIdList | TS 28.541 [2] | It is a set of NSI ID. NSI ID is an identifier for identifying the Core Network part of a Network Slice instance when multiple Network Slice instances of the same Network Slice are deployed, and there is a need to differentiate between them in the 5GC. |
| network slice subnet | TS 28.530 [4] | A representation of a set of network functions and the associated resources (e.g. compute, storage and networking resources) supporting network slice. |
| <<InformationObjectClass>> NetworkSliceSubnet | TS 28.541 [2] | This IOC represents the properties of a network slice subnet instance in a 5G network. |
| NetworkSliceSubnet instance | TS 28.530 [4] | A Managed Object Instance (MOI) of NetworkSliceSubnet IOC. |
| NetworkSliceSubnet identifier | TS 28.531 [3] | To identify a NetworkSliceSubnet instance defined in TS 28.530, it is DN of a managed object instance of NetworkSliceSubnet IOC [2]. |

## 4.2	Multi-operator relationships in network slice management

Multi-operator network slice management refers to scenarios where multiple network operators are involved in providing a network slice.

The following figure is a subset of 28.530 [4] Figure 4.8.1, with focus on the relationship between the Communication Service Provider and Network Operator.

Provider

Client

Communication Service Provider

Network Operator

Figure 4.2.1: High-level model of roles

However, 28.530 [4] does not describe the fact that multiple Network Operators may be involved. The following figure considers a scenario where Network Operator A may source network resources from another Network Operator B. The sourced network resources are packaged as a Communication Service, therefore Network Operator A behaves as a Communication Service Customer and Network Operator B behaves as a Communication Service Provider.

Provider

Provider

Client

Client

Communication Service Customer

Communication Service Provider A

Communication Service Customer A

Network
Operator B

Communication Service Provider B

Network
Operator A

Figure 4.2.2: Multi-operator scenario

## 4.3	End to end management of network slice

Management of network slices is the set of management and orchestration activities that allow the deployment and operation of network slices across multiple administrative and network management domains. According to 28.530 [4], a network slice includes all the network function instances, with their supporting resources, to provide a certain set of communication services to serve a certain business purpose.

## 4.4	Multi-operator concepts

In this clause different Multi-operator concepts are described.

### 4.4.1	National roaming concept

National Roaming is defined in TS 22.011 [10] as a service whereby a UE of a given PLMN is able to obtain service from another PLMN of the same country, anywhere, or on a regional basis. It is used today e.g between EPS networks. In the following figure, Network operator A and B have a national roaming agreement and subscribers of network operator A can use the 5GS network of network operator B, while using the communication services of network operator A.

Provider

Provider

Client

Client

Communication Service Customer

Communication Service Provider A

Communication Service Provider B

Network
Operator A

Communication Service Customer

Client

Provider

Radio Network
 A

Radio Network
 B

Communication Service Provider B

Network
Operator B

Figure 4.4.1.1: National roaming concept

Management interactions between and by network operators involved here: both the network operator A and B have instantiated network slices. Network operator B determines which network slices are allowed to be used by inbound roamers from network operator A, and if needed, will configure its network to support mapping of S-NSSAI as defined in TS 23.501 [1].

If subscribers of network operator A are visiting the network of network operator B the roaming principles as standardized in TS 23.501 [1] apply, i.e., the UE of network operator A will register into 5GC with an AMF in network operator B, and will receive Allowed NSSAI and other network slicing related information as needed. In case of home routing, PDU sessions with an S-NSSAI are established with V-SMF in network operator B and H-SMF in network operator A.

# 5	Scenarios for Network Slicing Management Enhancements

## 5.1	Network Slice use in multiple networks scenario 1

### 5.1.1	Description RAN sharing

CSP wishes to provide a communication service in multiple regions. The CSP has selected NOP-A as network slice provider. NOP-A does not have RAN coverage in all of the geographical areas that are required by the CSP.

NOP-A selects NOP-B to provide the missing RAN coverage for one or more geographical areas.

Editor’s Note: It is assumed that the geographical areas are located in a single country and that a single PLMN ID is used. International roaming scenario is for further study and may be more similar to the national roaming scenario in clause 5.2.

The roles of CSP and NOP-A may be played by either same or different companies. Further details how CSP interacts with NOP-A to order, provision or monitor a network slice is out the scope of this use case. It is expected to follow general network slice use cases and not be specific to the RAN sharing scenario.

It is assumed that NOP-A and NOP-B roles are played by different companies, hereafter referred to as Company-NA and Company-NB. This is illustrated in figure 5.1.1-1. For interaction between Company-NA and Company-NB for RAN sharing, the applicability and meaning of additional roles such as NSC and NSP  will be considered as part of potential solutions.

Figure 5.1.1-1 Interaction between Company-NA and Company-NB

As pre-condition for this scenario, Company-NA may need a general RAN sharing agreement with Company-NB. Details of how to establish such an agreement is out of scope for this use case.

To support request from CSP, Company-NA orders or modifies a service from BSS of Company-NB which will provide the required RAN coverage. Company-NB may in turn issue one or more requests to its own 3GPP management system from its own BSS based on the agreed order from Company-NA.

NOTE 1: Details of how Company-NA orders service product from BSS of Company-NB and negotiates SLA is out the scope of this use case.

During operation phase, Company-NA may further provision or monitor the RAN coverage service via BSS of Company-NB, depending on agreed level of management capability exposure

The ServiceProfile corresponding to the CSP communication service will be associated with a NetworkSlice instance in NOP-A’s management system. NOP-A’s management system will also contain a corresponding top NetworkSliceSubnet instance as well as additional objects representing constituents that are part of NOP-A’s network. The modelling of NOP-B network constituents in NOP-B’s management system, as well as how they are referred from NOP-A’s management system is partly solution dependent and will be further discussed for each potential solution.

### 5.1.2	Identified problems and general issues

There is no guidance for a RAN service provider on the management capabilities that should be exposed to allow different management levels for resources related to the RAN service.

Solutions need to consider potential business and security impact as well as technical gaps. See also analysis of solution alternative 1 in clause 7.2.1.2.

The service provided by Company-NB is only RAN part. It is important that terminology in 3GPP specifications is consistent and understandable. For network slice the primary SA5 definitions are found in TS 28.530 clause 3.1 This also includes reference to TS 23.501. Here a network slice is described as always being e2e, i. e. containing both CN and RAN. Thus potential solutions should also consider use and impact on terminology in addition to other aspects.

Editor’s Note: In this use case Company-NA and Company-NB interact with each other via their respective BSS, more details of this interactions are being studied in SI FS_MNSAC and FS_NSCE

## 5.2	Network slice use in multiple networks scenario 2

### 5.2.1	Description national roaming

CSP-A wishes to provide a communication service using NOP-A. NOP-A does not have RAN coverage in all of the geographical areas that are required by the CSP-A. CSP-A has a national roaming agreement with CSP-B to allow subscribers of NOP-A to roam into NOP-B.

NOTE: National roaming is an agreement between two MNOs, and both may have the roles of CSP, NOP, and NSP.

As part of the agreement with CSP-B, CSP-A requests CSP-B to support a communication service for inbound roamers with certain capabilities. CSP-A need to inform CSP-B about the S-NSSAI(s) used by inbound roamers and for the capabilities required for this communication service. CSP-B needs this information first of all to check (with NOP-B) on the NetworkSlice instance(s) to be used by the inbound roamers and whether there is a need to map the CSP-A S-NSSAI to CSP-B S-NSSAI. CSP-A may issue the request to CSP-B’s 3GPP management system, or CSP-B may issue the request to its own 3GPP management system.

For the NetworkSlice instance in NOP-A, this exists in NOP-A’s management system, including the information that the NetworkSlice instance is used for outbound roaming. For the NetworkSlice instance in NOP-B, this exists in NOP-B’s management system, including the information that the NetworkSlice instance is used for inbound roaming.

### 5.2.2	Identified problems

The national roaming agreement between CSP-A and CSP-B needs to be enhanced for 5G to also include required NetworkSlice instance information (identify needed enhancement).

For national roaming, 3GPP management enhancements may be needed to support required exchanges between CSP-A/NOP-A (as well as between CSP-B/NOP-B).

The NetworkSlice instance information in NOP-A and NOP-B needs to be updated to support outbound/inbound roaming. This is not currently supported in the NetworkSlice IOC defined in 28.541 [2].

## 5.3	Use case – Isolated deployment of multiple network slices of same customer

### 5.3.1	Introduction

In this use case, one network slice customer (e.g. a public safety department, or NSC_PS-1) requests two network slices from a network slice provider to support mIoT services of two regions, e.g. in two cities, and other service requirements (besides coverage area) of the two slices can be different also.  The isolation requirements of the two mIoT network slices are same, both require to physically isolate user plane of the network slice from network slices of other network slice customers (no restriction on control plane). In addition, to save total cost, the network slice customer NSC_PS-1 agrees that the two mIoT network slices can share resources with each other without physical isolation.  As shown in below, user plane of both NS_mIoT-1 and  NS_mIoT-2 are separated from NS_x1 and NS_x2, e.g. NS_mIoT-1 and  NS_mIoT-2 are deployed on different physical nodes (e.g. physical gNBs, physical UPFs or servers to accommodate virtualized UPF instances) than nodes of NS_x1 and NS_x2. However, NS_mIoT-1 and NS_mIoT-2 could share same physical resources, e.g. physical UPF or server/host OS in where the virtualized UPF instances are deployed.

Figure 5.3.1-1 Example of isolated deployment of two network slices of same customer

In other words, NS_mIoT-1 and NS_mIoT-2 of NSC_PS-1 are deployed in one security domain (refer to 33.210), which is isolated from other security domains. The NS_mIoT-1 and NS_mIoT-2 share isolation requirements, as well as resources.

To optimize operational procedures, an MNO/NOP (as internal customer) may create network slices for their own eMBB service subscribers in different cities. e.g. NS_eMBB-1 and NS_eMBB-2, and other service requirements (besides coverage area) of the two slices can be different also. To better utilize resource, these two eMBB services can share same resources. As shown in below, resource of both NS_eMBB-1 and  NS_eMBB-2 are separated from NS_x1 and NS_x2, but  NS_eMBB-1 and NS_eMBB-2 could share same resource.

Figure 5.3.1-2 Example of isolated deployment of two network slices of operator

### 5.3.2	Issue and gaps

The resourceSharingLevel attribute is defined in ServiceProfile dataType - see Network Slice IOC defined in 5G Network Resource Model (NRM). This attribute only specifies whether the resources to be allocated to the network slice can be shared with other network slice(s). The existing 3GPP management system capabilities cannot support the use cases and potential requirements described above, which implied not only granular isolation requirements for network slice (e.g. isolation attribute specified in GSMA PRD NG.116), but also flexible inclusion/exclusion rules between network slices (e.g. group network slices with same isolation requirement and security level together, and isolate the network slices in the group from network slices in other groups).

## 5.4	Deployment of edge application service

### 5.4.1	Description

NOP wishes to provide an edge application service which is to be deployed as part of a network slice.

NOP allocates a network slice for the new service, which includes Edge Application Server (EAS) and Edge Enablement Server (EES) as Core Network NFs. An S-NSSAI identifies the allocated network slice.

If NOP is the owner of the Edge Configuration Server (ECS), NOP configures ECS with the relevant S-NSSAI for the new edge application service. Otherwise, NOP informs the owner of the ECS of the relevant S-NSSAI and the owner of the ECS configures the ECS.

When a UE registers on the network, the UE sends a service provisioning request to ECS. The response from ECS includes the S-NSSAI for the edge application service (ref 23.558 [9] clause 8.3.3).

### 5.4.2	Identified problems

When allocating a network slice, it is not possible to specify in ServiceProfile that the network slice should contain EAS and EES, and that the network slice should support a particular edge application service.

NRM for Edge Configuration Server should include S-NSSAI for the edge application service.

## 5.5	Use case – Support network slice specific authentication

### 5.5.1	Description

Network Slice Specific Authentication and Authorization (NSSAA) attribute is defined in GSMA GST (NG.116 v4.0) to specify whether for the network slice, devices need to be also authenticated and authorized by a AAA server using additional credentials different than the ones used for the primary authentication. This feature was defined in clause 5.15.10 of 3GPP TS 23.501(see [1]), and related procedures were specified in TS 23.502 (see [y]).

According to Network Slice-Specific Authentication and Authorization procedure described in clause 4.2.9 of TS 23.502 (see [y]), the NSSAA  is triggered for an S-NSSAI requiring Network Slice-Specific Authentication and Authorization with an AAA Server (AAA-S) which may be hosted by the H-PLMN operator or by a third party which has a business relationship with the H-PLMN. An AAA Proxy (AAA-P) in the HPLMN may be involved e.g. if the AAA Server belongs to a third party.

This procedure is triggered by the AMF during a Registration procedure when some Network Slices require Slice-Specific Authentication and Authorization, when AMF determines that Network Slice-Specific Authentication and Authorization is requires for an S-NSSAI in the current Allowed NSSAI (e.g. subscription change), or when the AAA Server that authenticated the Network Slice triggers a re-authentication.

The AMF performs the role of the EAP Authenticator and communicates with the AAA-S via the Network Slice Specific Authentication and Authorization Function (NSSAAF). The NSSAAF undertakes any AAA protocol interworking with the AAA protocol supported by the AAA-S.

### 5.5.2	Issue and gaps

To support NSSAA feature and corresponding procedures, the operator may provision and configure following objects and attributes for a network slice, based on requirement of the network slice, operator policies and network deployment options:

- The indication whether the network slice is subject to Network Slice-Specific Authentication and Authorization

- Functions to support NSSAA, e.g. AMF, NSSAAF, AAA servers and Proxy, etc.

- Credentials for NSSAA (especially when AAA-S is hosted by the H-PLMN operator)

The management capabilities described above were not supported in existing 3GPP specifications.

## 5.6	use case – support network slice protection on N6 interface

### 5.6.1	Description

With network slicing, various types of users/customers can enjoy connectivity and data processing of mobile networks tailored to their specific requirements (e.g. latency, throughput, reliability, security, etc.) that adhere to a Service Level Agreement (SLA). Providing secure inter- and intra- network slice communication and secure internet access (e.g. prevention of attacks at the edge of network slices) are major requirements from mobile network operators (MNOs) or industry verticals. User data access attribute defined in GSMA GST (NG.116 v4.0) describes how the network slice should handle the user data towards data network according to requirement for the network slice. There are several options, e.g. no access to internet, all data traffic is routed to the private network through secure tunnel, no access to either internet or private network. Deploying security function, e.g. firewalls, at the CN edge (e.g. N6 interface) could protect the network slice from the attack from internet. The network slice with edge protection could be looked like Figure 5.x.1-1.

Figure 5.6.1-1 Example of edge protect for a network slice

### 5.6.2	Issue and gaps

3GPP TS 22.101 (see [x]) defines Flexible Mobile Service Steering (FMSS) requirements to steer the subscriber’s traffic to appropriate enablers (e.g. NAT, antimalware, parental control, Firewall, DDoS protection) in the N6-LAN. The implementation of the requirements in core network could improve the user’s QoE, apply the security control, etc., for the 5G network. Further, service function chaining (SFC) requirements were defined in clause 6.35 of 22.261 [see [y]], which enables operator to define and modify service function chaining policies for steering traffic, create/modify/delete a service function, create/configure/control a chain of service functions, etc. The requirements are for 5G network, which could be applicable to network slice as well to improve security of a network slice, especially at the N6 interface towards internet.

However the existing management services (including MnS components A, B, C) specified in 3GPP can't support either requirement of protection N6 interface for a network slice, or FMSS and SFC requirements defined in SA1.

# 6	Potential requirements for Network Slicing Management Enhancements

## 6.1	Potential requirements for Network Slicing Management to support roaming

- The 3GPP management system should have the capability to allow a specific S-NSSAI to be used for outbound roaming to a specific PLMN.

- The 3GPP management system should have the capability to allow a specific S-NSSAI to be used for inbound roaming from a specific PLMN.

## 6.2	Potential requirements for Network Slicing Management to support network slice isolation

- The 3GPP management system should have the capability to allocate network slice or network slice subnetbased on granular isolation requirements on the network slice.

Note: For example, instead of binary sharing/non-sharing indicator, granular isolation requirements could be multi- levels requirement, such as isolation attribute specified in GSMA PRD NG.116

- The 3GPP management system should have the capability to, based on isolation requirements associated with the service profile and slice profile, group network slices or network slice subnet with same isolation requirements and isolation level.

- The 3GPP management system should have the capability to support authorized network slice or network slice subnet related management service consumer to query the available isolation groups for the network slice or network slice subnet to be deployed.

- The 3GPP management system should have the capability to support authorized network slice related management service consumer to select the appropriate isolation group for the network slice to be deployed.

Note: For example, the authorized network slice or network slice subnet related management service consumer could be another high privileged management system or administrator of the MNP.

## 6.3	Potential requirements for Network Slicing Management to support edge computing

- The 3GPP management system should have the capability to allow a ServiceProfile <<dataType>> (cf. TS 28.541) to be extended with edge computing related service requirements, for the cases where the corresponding communication service includes one or several edge application services.

## 6.4	Potential requirements for Network Slicing Management to support network slice specific authentication

- REQ-NSSA-1 The 3GPP management system may have the capability to support NSSAA requirement for a network slice

- REQ-NSSA-2 The 3GPP management system may have the capability to support configuring NSSAA indication for a network slice

- REQ-NSSA-3 The 3GPP management system may have the capability to support provisioning AMF for NSSAA

- REQ-NSSA-4 The 3GPP management system may have the capability to support provisioning NSSAAF for NSSAA

- REQ-NSSA-5 The 3GPP management system may have the capability to support provisioning AAA-S and AAA-P for NSSAA

- REQ-NSSA-6 The 3GPP management system may have the capability to support managing and provisioning  NSSAA credentials for network slice

## 6.5	Potential requirements for Network Slicing Management to support network slice protection on N6 interface

- REQ-NSP-1 The 3GPP management system may have the capability to support service level requirement to protect N6 interface of a network slice.

Note 1: The service level requirement to protect N6 interface of a network slice could be captured in service profile (or SLS) of a network slice.

- REQ-NSP-2 The 3GPP management system may have the capability to support translating service level requirement to protect N6 interface of a network slice to requirements on network slice subnets.

Note 1: For example, the requirements on network slice subnet to protect N6 interface could be captured in a slice profile of a core network slice subnet.

- REQ-NSP-3 The 3GPP management system may have the capability to support translating requirements of network slice subnet to protect N6 interface to configuration parameters of network functions or security functions, to facilitate deploying and configuring security function chain to protect N6 interface for a network slice.

Note 1: The configuration parameters of network functions could be traffic routing information configured in PCF, SMF or UPF. Please refer PCC rule of 23.503 and related NRM fragment defined in 28.541

Note 2: For example, according to N6 interface protection requirement, a firewall could be deployed and configured in core network slice subnet to protect N6 interface of the network slice. 3GPP management system may coordinate with other systems, e.g. management system defined in NFV or IETF, for security function deployment and configuration.

# 7	Possible solutions for Network Slicing Management Enhancements

## 7.1	Possible solutions for network slice isolation

It is proposed to enhance the 5G NRM to support network slice isolation

- Define isolation group in NRM to represent a group of network slices/network slice subnets sharing same isolation requirements and resources. The group can be based on tenant, SST, region, security level, etc.

- Define isolation profile in NRM to represent a set of isolation requirements.

- Associate a network slice/network slice subnet to an isolation group

- Associate an isolation group to an isolation profile

- Isolation requirements, associated with service profiles and slice profile, are used to guide the allocation of network slices and network slice subnets

Figure 7.1-1 NRM fragment proposal for network slice isolation

## 7.2	Possible solutions for network slice covering multiple networks

### 7.2.1 Alternative 1: solution based on the existing NRM with related exposure

#### 7.2.1.1 Description

This clause describes how 3GPP management capabilities may be used to allow Company-NA to manage a network slice which is provided by Company-NB.

As part of the provisioning phase, the top NetworkSliceSubnet MOI in Company-NA’s management system should be configured with a reference to the top RAN NetworkSliceSubnet MOI in Company-NB’s management system. To obtain this reference, Company-NA’s management system may read the NetworkSlice instance which was returned by Company-NB after allocateNsi operation (cf. 28.531 [3], clause 6.5.1), and read the attribute NetworkSliceSubnetRef. This assumes that Company-NA has read access to the generic provisioning management service of Company-NB, and has the authority to read the NetworkSlice instance.

Therefore, to allow Company-NA to manage the network slice using a 3GPP Management System, Company-NB may allow Company-NA read access to the managed object NetworkSlice via the getMOIAttributes operation, defined in 28.532 [5]. This will allow Company-NA to read the operationalState and administrativeState of the NetworkSlice instance, and also the networkSliceSubnetRef of the NetworkSliceSubnet.

If Company-NB wishes to allow Company-NA to control the adminstrativeState of the NetworkSlice instance, Company-NB may allow Company-NA write access to the managed object NetworkSlice via the modifyMOIAttributes operation, defined in 28.532 [5]. However, this would also allow Company-NA to alter the ServiceProfileList, which may not be desirable. Therefore, Company-NB must implement checks on any change to the attribute ServiceProfileList, see “Procedure of Network Slice Instance Modification” in 28.531 [3] and any undesirable changes should be rejected.

If Company-NB wishes to allow Company-NA to view alarms related to the NetworkSlice instance, Company-NB may expose the “FS Data Report for NSI” Service, as described in 28.545 [6]. Company-NB should only allow operations by Company-NA where the baseObjectInstance is equal to the DN of the NetworkSlice instance.

If Company-NB wishes to allow Company-NA to manage alarms related to the NetworkSlice instance, Company-NB may expose the “FS Control for NSI” Service, as described in 28.545 [6]. Company-NB should only allow operations by Company-NA where the baseObjectInstance is equal to the DN of the NetworkSlice instance.

If Company-NB wishes to allow Company-NA to view performance measurements related to the NetworkSlice instance, Company-NB may expose the operations and notificationsdescribed in 28.550 [7]. Company-NB should only expose measurements related to the S-NSSAI of the NetworkSlice instance.

If Company-NB wishes to allow Company-NA to view KPIs related to the NetworkSlice, Company-NB may expose the KPIs as described in 28.554 [8]. Company-NB should only expose KPIs related to the NetworkSlice instance or KPIs related to the S-NSSAI of the NetworkSlice instance.

#### 7.2.1.2 Solution analysis

The solution in 7.2.1.1 mainly based on existing management capabilities could have potential business, security issues, as well as technical gap. E.g. The solution proposes the top NetworkSliceSubnet MOI in NOP-A’s management system could aggregate RAN NetworkSliceSubnet MOI in NOP-B’s management system. The network slice topology resulting from the provisioning according to this solution would look like figure 7.2.1.2-1.

Figure 7.2.1.2-1 Topology view between Company-NA and Company-NB

The problems/limitations of the solution include:

1. It breaks the layering principles for a Telecommunications Management Network defined by ITU-T, where different layers have different views (scope, abstraction level, exposure etc.) to the network.

2. Generally, the service provider (SP) offers service to its customer (SC) as a Blackbox without exposing detailed network/resource topology (encapsulation/abstraction principle) in order to avoid leaking unnecessary sensitive information (need to know principle) and potential mis-use and mis-operation. Therefore, in a normal case, only service level object and related attributes would be exposed to the service consumer instead of exposing lower layer resources to external customer, especially when the resource is shared by multiple services of difference customers. In other words, generally an SC is not allowed to “decompose” or "reverse-engineer" the service (e.g. network slice) it gets from the SP and to use just a "building block" of the consumed service, such as partial resource of the service (i.e. a single network slice subnet or network function out of the entire network slice as a service). The service decomposition or reverse-engineering is usually explicitly prohibited (by the terms of the contract / SLA) as causing threat/risk to SP’s network.

3. Company-NA’s management system holds a reference to the top NetworkSliceSubnet MOI in Company-NB’s management system, and therefore the related SliceProfile. But there is no similar reference to the NetworkSlice MOI in Company-NB’s management system, and the related ServiceProfile. This implies that Company-NA is unable to track the SLA/SLS related to the ServiceProfile. Note that a reference to the NetworkSlice MOI in Company-NB’s management system is returned in the response to the allocateNsi operation, but the 3GPP NRM does not standardize how this should be stored in Company-NA’s management system.

4. The ServiceProfile is currently not suitable for expressing RAN only requirements. Attributes are expected to express e2e requirements. See descriptions as well as use of sources such as GSMA GST and TS 22.261. The ServiceProfile also doesn’t allow providing PLMN ID or S-NSSAI information. These values will be selected by Company-NA/NOP-A, and for RAN level must be part of input requirements.

### 7.2.2 Alternative 2: solution based on NRM extension - dual views of network slice

Company-NA sends to Company-NB an AllocateNsi operation request (cf. TS 28.531 clause 6.5.1) to ask to allocate a NetworkSlice instance to satisfy its network slice related requirements;

Company-NB sends back the AllocateNsi operation result to Company-NA. This result includes the relevant network slice instance information (an existing NetworkSlice instance may have been modified or a new NetworkSlice instance may have been created to satisfy the network slice related requirements - see TS 28.531 clause 6.5.1);

Company-NA can then create its end-to-end network slice based on local network slices provided by Company-NB.

Suggest to model network slice as combination (dual views) of service and resource. From one perspective, network slice can be treated as a service (commercial exposure) when it offered by network slice provider (NSP) to network slice customer (NSC). From another perspective, a network slice (either provided by the same operator in network slice as NOP internal mode or offered by a different operator in network slice as a service (NSaaS) mode) can be treated as a resource and therefore, stitched to a resource collection object (e.g. network slice subnet) to build another network slice. That implies a NetworkSlice MOI could be aggregated by a resource collection object, e.g. NetworkSliceSubnet MOI.

Editor's Note: the model details can be further elaborated.

### 7.2.3 Alternative 3: solution based on NRM extension - recursive network slice

Company-NA sends to Company-NB an AllocateNsi operation request (cf. TS 28.531 clause 6.5.1)  to ask to allocate a NetworkSlice instance to satisfy its network slice related requirements;

Company-NB sends back the AllocateNsi operation result to Company-NA. This result includes the relevant network slice instance information (an existing NetworkSlice instance may have been modified or a new NetworkSlice instance may have been created to satisfy the network slice related requirements - see TS 28.531 clause 6.5.1);

Company-NA can then create its end-to-end network slice based on local network slices provided by Company-NB.

Suggest to update TS 28.541 Figure 6.2.1-1 to represent that a NetworkSlice IOC instance can be composed of other NetworkSlice IOC instances

### 7.2.4 Alternative 4: solution based on - support network slice subnet as a service

This clause describes how 3GPP management capabilities may be used to allow Company-NA to manage a network slice subnet service which is provided by Company-NB.

When receiving request for RAN coverage service, Company-NB BSS can issue a corresponding allocateNssi request to its own 3GPP management system with SliceProfile for RAN based on based on parameters received from Company-NA, also taking into account any existing agreements. The pLMNInfoList of the SliceProfile will capture PLMN ID of NOP-A as well as S-NSSAI within that PLMN selected by Company-NA/NOP-A. If successful, relevant and needed identifiers associated with the provided service are returned to Company-NA. Company-NA/NOP-A can then include this information as part of network slice representation in its own management system. This high-level ordering and provisioning procedure is illustrated in figure 7.2.4-1.

Editor’s note: Details of how to model an external constituent in NOP-A management system is for further study.

NOTE: The NetworkSliceSubnet instance allocated in NOP-B’s management system can be newly created or an existing instance can be used following procedure for allocateNssi, see TS 28.531 [x] clause 6.5.2 and clause 7.3. Compared to use cases without sharing, it is possible that for some NetworkSliceSubnet instances there will be no reference from other NetworkSliceSubnet instance or indirectly any NetworkSlice instance within NOP-B’s management system.

Figure 7.2.4-1 Provisioning phase

For basic support of the RAN sharing use case, it is not necessary for Company-NB to expose any capabilities beyond service lifecycle operations. But based on agreement Company-NB may expose additional management capabilities in provisioning or monitoring areas. This is illustrated in figure 7.2.4-2.

Such exposure should be setup to ensure that only capabilities related to the service provided to Company-NA are exposed. And if any provisioning operations are allowed, these must not interfere with use of network resources by other users.

Figure 7.2.4-2 Operation phase

If Company-NB wishes to allow Company-NA to view operationalState or administrativeState of the NetworkSliceSubnet instance, Company-NB may allow Company-NA to interact with the BSS, the BSS has read access to the managed object NetworkSliceSubnet via the getMOIAttributes operation, defined in 28.532 [5].

If Company-NB wishes to allow Company-NA to control the adminstrativeState of the NetworkSliceSubnet instance, Company-NB may allow Company-NA to interact with the BSS, the BSS has write access to the managed object NetworkSliceSubnet via the modifyMOIAttributes operation, defined in 28.532 [5].

If Company-NB wishes to allow Company-NA to view alarms related to the NetworkSliceSubnet instance, Company-NB may expose management capabilities to the “FS Data Report for NSSI” Service via BSS, as described in 28.545 [6]. BSS in this case is allowed operations on behalf of Company-NA where the baseObjectInstance is equal to the DN of the NetworkSliceSubnet instance.

If Company-NB wishes to allow Company-NA to manage alarms related to the NetworkSliceSubnet instance, Company-NB may expose management capabilities to the “FS Control for NSSI” Service, via BSS as described in 28.545 [6]. BSS in this case is allowed operations on behalf of Company-NA where the baseObjectInstance is equal to the DN of the NetworkSliceSubnet instance.

If Company-NB wishes to allow Company-NA to view performance measurements related to the NetworkSliceSubnet instance, Company-NB may expose management capabilities to the operations and notifications described in 28.550 [7] via BSS. BSS should only expose measurements related to the S-NSSAI of the NetworkSliceSubnet instance.

If Company-NB wishes to allow Company-NA to view KPIs related to the NetworkSliceSubnet, Company-NB may expose the KPIs as described in 28.554 [8] via BSS. Company-NB should only expose KPIs related to the NetworkSliceSubnet instance or KPIs related to the S-NSSAI of the NetworkSliceSubnet instance.

### 7.2.5	Solution for national roaming

Editor’s Note: The solution for national roaming is FFS

## 7.3	Possible solutions for network slice specific authentication

It is proposed to enhance the 5G NRM to support network slice specific authentication:

- Add NSSAA related requirements in ServiceProfile and SliceProfile for CN

- Add NSSAA related parameter in AMFFunction IOC

- Add new IOCs to support NSSAAF, AAA-S and AAA-P

# 8	Conclusions and recommendations

## 8.1 	Conclusions

The following issues are identified in this study

- Depending on agreements between Network Operators, one Network Operator may expose management capabilities to the other Network Operator. This issue is in the scope of Work Item FS_NSCE.

- The NetworkSlice IOC defined in 28.541 [2] should be updated to support outbound/inbound roaming.

- The Network Resource Model defined in 28.541 [2] should be updated to support granular isolation requirements on the network slice.

- The Network Resource Model defined in 28.541 [2] should be updated to support grouping of network slices with same isolation requirements and security level.

- NRM for Edge Configuration Server should include S-NSSAI for the edge application service. This issue is in the scope of Work Item FS_Eedge_Mgt.

- The NetworkSlice IOC defined in 28.541 [2] should be updated to support service level edge protection requirements.The NetworkSlice IOC defined in 28.541 [2] should be updated to support Network Slice Specific Authentication and Authorization (NSSAA) requirements.

## 8.2 	Recommendations

The improvements to the Network Resource Model (as described in clause 7) are not currently in the scope of any existing Work Item. It is proposed that the scope of existing Work Item adNRM should be extended to add the following issues

- Network slice support for outbound/inbound roaming

- Improved isolation for a network slice

- Grouping of network slices

- Support for service level edge protection requirements

- Support for Network Slice Specific Authentication and Authorization (NSSAA) requirements

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2019-07 | - | n/a | - | - | - | Initial skeleton | 0.0.0 |
| 2020-08 | SA5#132e | S5-204041 | - | - | - | 28.811 skeleton | 0.1.0 |
| 2020-08 | SA5#132e | S5-204582 | - | - | - | Add concepts | 0.1.0 |
| 2020-08 | SA5#132e | S5-204583 | - | - | - | Add scope | 0.1.0 |
| 2020-11 | SA5#134e | S5-206065 | - | - | - | Update scope to include security aspects | 0.2.0 |
| 2020-11 | SA5#134e | S5-206399 | - | - | - | Add cross-operator concept | 0.2.0 |
| 2020-11 | SA5#134e | S5-206400 | - | - | - | Add end to end concept | 0.2.0 |
| 2020-11 | SA5#134e | S5-206401 | - | - | - | Update references and terms | 0.2.0 |
| 2020-11 | SA5#134e | S5-206402 | - | - | - | Deploy two network slices of same customer | 0.2.0 |
| 2020-11 | SA5#134e | S5-206403 | - | - | - | Add cross-operator network slicing scenario | 0.2.0 |
| 2021-02 | SA5#135e | S5-211434 | - | - | - | Add deployment of edge application service | 0.3.0 |
| 2021-02 | SA5#135e | S5-211435 | - | - | - | Add description of cross-operator management of network slice | 0.3.0 |
| 2021-02 | SA5#135e | S5-211476 | - | - | - | Update to end to end network slice concept | 0.3.0 |
| 2021-02 | SA5#135e | S5-211477 | - | - | - | Multi-operator concepts | 0.3.0 |
| 2021-03 | SA5#136e | S5-212424 | - | - | - | Addition of national roaming scenario | 0.4.0 |
| 2021-05 | SA5#137e | S5-213506 | - | - | - | 28.811 use case - support isolation in network slice subnet | 0.5.0 |
| 2021-05 | SA5#137e | S5-213561 | - | - | - | pCR 28.811 Add potential requirements | 0.5.0 |
| 2021-05 | SA5#137e | S5-213562 | - | - | - | 28.811 use case - support Network Slice Specific Authentication | 0.5.0 |
| 2021-05 | SA5#137e | S5-213563 | - | - | - | Update to multi-operator scenario | 0.5.0 |
| 2021-05 | SA5#137e | S5-213576 | - | - | - | 28.811 use case - support network slice protection on N6 interface | 0.5.0 |
| 2021-05 | SA5#137e | S5-213577 | - | - | - | pCR 28.811 Add conclusions and recommendations | 0.5.0 |