3GPP TSG SA WG5 Meeting 137-e		TDoc S5-213089

# electronic meeting, online, 10 - 19 May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.541 | CR | 0478 | rev | - | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update table L.2.1 to introduce PM info |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | EMA5SLA |  |  |  |  |  | Date: |  |  | 2021-04-30 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Some of the information (as shown in Table L.2.1) in 5GC SliceProfile and NG-RAN SliceProfile is translated to configurable parameters related to network function behaviour for the control plane SLA support purpose. The related KPIs and performance measurements are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update table L.2.1 to introduce information of the related KPIs and performance measurements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex L.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st modified section

L.2	GSMA GST, ServiceProfile and sliceProfile

The GSMA GST is used as the SLA information for the communication between the NSC (e.g. vertical industry) and the NSP. The SLA requirements can be fulfilled from management aspect and control aspect in a coordinated way. The SLS includes ServiceProfile information model.

As shown in figure L.2.1, the GST [50] is translated and used as input to NRM ServiceProfile, the ServiceProfile can be translated to corresponding requirements for dedicated domains. For example, 5GC SliceProfile is used to carry 5GC domain requirements, NG-RAN SliceProfile is used to carry NG-RAN domain requirements, and TN requirements are translated and provide to TN domain. Some of the information in 5GC SliceProfile and NG-RAN SliceProfile translated to configurable parameters of network function for the control plane SLA support purpose.

Some of the information (as shown in Table L.2.1) in 5GC SliceProfile and NG-RAN SliceProfile is translated to configurable parameters related to network function behaviour for the control plane SLA support purpose. While other information (e.g. delay tolerance, deterministic communication support) in 5GC SliceProfile and NG-RAN SliceProfile are kept at OAM domain and is used to determine the overall behaviour of the network slice.

Table L.2.1: GST translation related to control plane SLA support purpose

| GST Attributes | ServiceProfile Parameter | SliceProfile Parameter |  | Configurable Parameter | KPIs and Performance Measurements |
| --- | --- | --- | --- | --- | --- |
|  |  | RANSliceSubnetProfile |  | CNSliceSubnetProfile |  |  |  |
| Maximum number of UEs | maxNumberofUEs | maxNumberofUE | maxNumberofUE |  |  | TBD | FFS |
| Maximum number of PDU sessions | maxNumberofPDUSessions | Not applicable | maxNumberofPDUSessions | TBD | See clause 6.4.5 Maximum number of PDU sessions of network slice in TS 28.554 [27]. |
| Downlink maximum throughput per UE | dLThptPerUE | dLThptPerUEPerSubnet | dLThptPerUEPerSubnet | TBD | See clause 5.1.1.3.1 Average DL UE throughput in gNB in TS 28.552 [69]. |
| Uplink maximum throughput per UE | uLThptPerUE | uLThptPerUEPerSubnet | uLThptPerUEPerSubnet | TBD | See clause 5.1.1.3.3 Average UL UE throughput in gNB in TS 28.552 [69]. |

Editors note: The list of exact configurable parameters is to be revisted depending on the requirements from SA2 and RAN WGs.

NOTE:	Void.

Figure L.2.1 Relation between GSMA GST, ServiceProfile and SliceProfile

End of modified section