3GPP TSG-SA5 Meeting #137e	TDoc S5-213097

# electronic meeting, online, 10 - 19 May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.310 | CR | 0013 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to definition for domain centralized SON |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | EE_5G |  |  |  |  |  | Date: |  |  | 2021-04-12 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Centralized SON function includes Domain-Centralized SON and Cross Domain-Centralized SON functions. These functions shall not be specified separately. Distributed energy saving solution should not include both D-SON and Domain Centralized SON |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the wrong description for Distributed energy saving solution |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Describing Centralized SON and Domain-Centralized SON functions separately are misleading and not correct |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of 1st modification

### 6.2.3	Distributed energy saving solution

#### 6.2.3.0	Management service components used for D-SON ES solution

The MnS components used for D-SON (Distributed SON) ES solution are listed in the following clauses 6.2.3.1.1, 6.2.3.1.2 and 6.2.3.1.3.

#### 6.2.3.1	Management services

##### 6.2.3.1.1	MnS component type A

| MnS Component Type A | Note |
| --- | --- |
| Operations defined in clause 11.1.1 of TS 28.532 [16]: - createMOI - getMOIAttributes - modifyMOIAttributes - deleteMOI | Supported by the Provisioning MnS for NF, as defined in TS 28.531 [6]. |
| Notifications defined in clause 11.1.1 of TS 28.532 [16]: - notifyMOICreation - notifyMOIAttributeValueChanges - notifyMOIDeletion | Supported by the Provisioning MnS for NF, as defined in TS 28.531 [6]. |

##### 6.2.3.1.2	MnS Component Type B

###### 6.2.3.1.2.1	Objective and targets

The objective of ES is to automatically set parameters so as to maximize NG-RAN data energy efficiency - see Table 6.2.3.1.2.1-1.

Table 6.2.3.1.2.1-1.  Energy Saving targets

| Targets | Definition | Legal Values |
| --- | --- | --- |
| NG-RAN data Energy Efficiency | Data Volume (DV) divided by Energy Consumption (EC) of the considered network elements. | In bit/J. |

###### 6.2.3.1.2.2	Control information

The parameters in DESManagementFunction IOC, which is defined in TS 28.541 [11], are used to control the D-SON (Distributed SON) ES functionality.

##### 6.2.3.1.3	MnS Component Type C

###### 6.2.3.1.3.1	Parameters to be optimized

This is out of the scope of the present document.

###### 6.2.3.1.3.2	Performance measurements

Performance measurements related to D-SON (Distributed SON) ES are captured in Table 6.2.3.1.3.2-1:

Table 6.2.3.1.3.2-1.  Energy saving management related performance measurements

| Performance measurements | Description | Related targets |
| --- | --- | --- |
| DRB.PdcpSduVolumeDL_Filter | Data Volume (amount of PDCP SDU bits) in the downlink delivered to PDCP layer – see clause 5.1.2.1.1.1 of TS 28.552 [15], per configured PLMN ID and per QoS level (mapped 5QI) and per S-NSSAI. In case of non-split gNBs. | NG-RAN data Energy Efficiency |
| DRB.PdcpSduVolumeUL_Filter | Data Volume (amount of PDCP SDU bits) in the uplink delivered from PDCP layer to higher layers – see clause 5.1.2.1.2.1 of TS 28.552 [15], per configured PLMN ID and per QoS level (mapped 5QI) and per S-NSSAI. In case of non-split gNBs. | NG-RAN data Energy Efficiency |
| DL Cell PDCP SDU Data Volume on X2 Interface | Data Volume (amount of PDCP SDU bits) in the downlink delivered on X2 interface in DC-scenarios – see clause 5.1.2.1.1.2 of TS 28.552 [15], per PLMN ID and per QoS level (mapped 5QI or QCI in NR option 3). In case of split gNBs. | NG-RAN data Energy Efficiency |
| DL Cell PDCP SDU Data Volume on Xn Interface | Data Volume (amount of PDCP SDU bits) in the downlink delivered on Xn interface in DC-scenarios scenarios – see clause 5.1.2.1.1.3 of TS 28.552 [15], per PLMN ID and per QoS level (mapped 5QI) and per S-NSSAI. In case of split gNBs. | NG-RAN data Energy Efficiency |
| UL Cell PDCP SDU Data Volume on X2 Interface | Data Volume (amount of PDCP SDU bits) in the uplink delivered on X2 interface in NSA scenarios – see clause 5.1.2.1.2.2 of TS 28.552 [15], per PLMN ID and per QoS level (mapped 5QI or QCI in NR option 3). In case of split gNBs. | NG-RAN data Energy Efficiency |
| UL Cell PDCP SDU Data Volume on Xn Interface | Data Volume (amount of PDCP SDU bits) in the uplink delivered on Xn interface in SA scenarios – see clause 5.1.2.1.2.3 of TS 28.552 [15], per PLMN ID and per QoS level (mapped 5QI) and per S-NSSAI. In case of split gNBs. | NG-RAN data Energy Efficiency |
| PNF Energy consumption | Energy consumed – see clause 5.1.1.19.3 of TS 28.552 [15] | NG-RAN data Energy Efficiency |

#### 6.2.3.2	Procedures

##### 6.2.3.2.1	Energy saving activation

Figure 6.2.3.2.1-1 depicts a procedure that describes how MnS producer of Distributed ES management makes the NR capacity booster cell enter the energySaving state.

Figure 6.2.3.2.1-1: Distributed energy saving activation

It is assumed that all relevant MOIs have been created.

Energy saving activation:

The MnS producer for distributed ES management consumes the management service for NF provisioning with modifyMOIAttributes operation to:

- Configure the cell overlaid relations for NR capacity booster cells, and macro cells as candidate cells

- Configure the ES policy that includes the thresholds for the energy saving activation and deactivation for NR capacity booster cells and candidate cells

- Enable the distribute energy saving function for intra-RAT or inter-RAT.

NOTE: Void

The distributed ES function makes decision for the NR capacity booster cell to enter the energySaving state based on the cell traffic load information (see clause 15.4.2 in TS 38.300 [13]).

The distributed ES function changes to the energySaving state, leading to a notifyMOIAttributeValueChanges (see clause 5.1.9 in TS 28.532 [16]) being sent to the MnS producer for distributed ES management to indicate the NR capacity booster has entered the energySaving state.

##### 6.2.3.2.2	Energy saving deactivation

Figure 6.2.3.2.2-1 depicts a procedure that describes how Distributed ES function makes the NR capacity booster cell leave the energySaving state.

Figure 6.2.3.2.2-1: Distributed energy saving deactivation

Energy saving deactivation:

The distributed ES function monitors the traffic load of candidate cell, and decides to re-activate the NR capacity booster cell when it detects that additional capacity is needed (see clause 15.4.2 in TS 38.300 [13]).

The distributed ES function changes to the notEnergySaving state, leading to a notifyMOIAttributeValueChanges being sent to the MnS producer for distributed ES management to indicate the NR capacity booster has been re-activated.

End of modification