# 3GPP TSG-SA5 Meeting #137-e	S5-213155

# electronic meeting, online, 10 May - 19 May 2021

# Source:	CATT

# Title:	pCR Updating description and requirements for Direct Communication charging

# Document for:	Approval

# Agenda Item:	7.5.3

1	Decision/action requested

The group is asked to discuss and agree on the proposal.

2	References

[1]	3GPP TR 32.846: “Study on charging aspects of Proximity-based Services in 5GS”.

3	Rationale

This contribution is to update the general description and reqirement for ProSe Direct Communication.

4	Detailed proposal

1st Modified Section

6.2.1	General description and assumptions

5G ProSe Direct Communication is defined as a communication between two or more UEs in proximity that are ProSe-enabled, by means of user plane transmission using NR technology via a path not traversing any network node.

5G ProSe Direct Communication over NR based PC5 reference point supports broadcast mode, groupcast mode, and unicast mode. Each communication mode is supported when the UE is served by NR and when the UE is outside of NR coverage.

5G ProSe Direct Communication supports both the case of public safety and commercial service.

5G ProSe Direct Communication supports both event based and session based charging;

To perform ProSe direct communication over PC5 reference point, the UE is configured with the related information as described in TS23.304[11] clause 5.1.3. The procedures for service authorization and provisioning to UE may be initiated by the PCF, by the UEa, or by the AF, as defined in TS23.304[11] clause 6.2, the parameters could be provided/updated by ProSe Application Server or PCF, provisioned in the ME or configured in the UICC.

The 5GS could re-use the Service authorization and provisioning mechanism, and the UE could be configured with information to be included in the usage information report.

Editor’s Note:	The support for Direct Communication via UE-to-Network is FFS.

Next Modified Section

6.2.2	Potential charging requirements

The following are potential high-level charging requirements for ProSe services in 5GS, derived from the requirements in TS 22.115 [9], and TS 23.303 [8].

REQ-CH_ PROSE _5GS_DC - 01: The 5GS should support converged charging and charging information reporting for ProSe Communication including:

-	ProSe Broadcast mode Direct Communication;

-	ProSe Groupcast mode Direct Communication;

-	ProSe Unicast Direct Communication, including UE-to-Network Relay;

REQ-CH_ PROSE _5GS_DC - 02: The 5GS should support identifying chargeable events and collecting charging information from UE.

REQ-CH_ PROSE _5GS_DC - 03:	The 5GS should support identifying chargeable events and collecting charging information via 5G ProSe service.

Editor’s note: These requirements is FFS based on use cases.

Next Modified Section