3GPP TSG SA WG5 Meeting 137-e		S5-213160

# electronic meeting, online, 10 - 19 May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.541 | CR | 0491 | rev | - | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Improve the readability of EP_Transport |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | S5 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei |  |  |  |  |  |  |  |  |  |
| Work item code: | adNRM |  |  |  |  |  | Date: |  |  | 2021-04-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Transport EP NRM fragment relationship is descrirbed in Figure 6.2.1-2, and EP_Transport IOC is introduced in Clause 6.3.17 to describe the logical transport interface or endpoin, However, it is not clear how to utilize such IOC when deploy a network slice, for example, which parameters need to be configured to 3gpp management system, and which parameters need to be sent to transport management system from 3gpp management system. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Improve the readability of EP_Transport IOC and epTransportRef attribute in NetworkSlice IOC |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The utilization of EP_transport IOC is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.1, 6.3.17.1, 6.3.18.1,6.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

6.3.2	NetworkSliceSubnet

6.3.2.1	Definition

This IOC represents the properties of a network slice subnet in a 5G network. For more information about the network slice subnet instance, see 3GPP TS 28.530 [70].

The attribute epTransportRef is used to specify a list of EP_Transport instance to be associated to the NetworkSliceSubnet. The MnS consumer determines the EP_Transport instance(s) associated to the NetworkSliceSubnet and request the MnS producer to configure the attribute epTransportRef of the NetworkSliceSubnet IOC.

The EP_Transport is name contained by SubNetwork, and an EP_Transport instance can be a new instance created for the NetworkSliceSubnet instance or an existing instance reused for the NetworkSliceSubnet instance.

6.3.2.2	Attributes

The NetworkSliceSubnet IOC includes attributes inherited from SubNetwork IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| operationalState | M | T | F | F | T |
| administrativeState | M | T | T | F | T |
| nsInfo | CM | T | F | F | T |
| sliceProfileList | M | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| managedFunctionRef | M | T | F | F | T |
| networkSliceSubnetRef | M | T | F | F | T |
| epTransportRef | O | T | T | F | T |

6.3.2.3	Attribute constraints

| Name | Definition |
| --- | --- |
| nsInfo Support Qualifier | Condition: It shall be supported if the NSS instance is realized in the virtualized environment. Otherwise this attribute shall be absent. |

6.3.2.4	Notifications

The common notifications defined in subclause 6.5 are valid for this IOC, without exceptions or additions.

2nd Change

6.3.17	EP_Transport

6.3.17.1	Definition

This IOC represents the logical transport interface or endpoint which including transport level information, e.g. transport address, reachability information and QoS profiles, etc.

MnS consumer can request MnS producer to create an EP_transport instance for one or multiple NetworkSliceSubnet(s).

The attribute logicInterfaceId and qosProfile is determined and requested by MnS consumer to configure the logicInterfaceId and qosProfile.

The attribute “epApplicationRef” is used to specify a list of EP_N3 and EP_NgU associated to the EP_transport.

MnS consumer can obtain all the information of the EP_Transport associated to a NetworkSliceSubnet from MnS producer and send to corresponding TN Management System.

6.3.17.2	Attributes

The EP_Transport IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| ipAddress | M | T | F | F | T |
| logicInterfaceId | M | T | T | F | T |
| nextHopInfo | O | T | F | F | T |
| qosProfile | O | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| epApplicationRef | M | T | T | F | T |

6.3.17.3	Attribute constraints

None.

6.3.17.4	Notifications

The common notifications defined in subclause 6.5 are valid for this IOC, without exceptions or additions.

6.3.18	EP_Application <<ProxyClass>>

6.3.18.1	Definition

This represents  <<IOC>>EP_N3 or <<IOC>>EP_NgU associated to EP_Tranport.

6.3.18.2	Attributes

See that defined in <<IOC>>EP_N3 or <<IOC>>EP_NgU.

6.3.18.3	Attribute constraints

See that defined in <<IOC>>EP_N3 or <<IOC>>EP_NgU.

.

6.3.18.4	Notifications

See that defined in <<IOC>>EP_N3 or <<IOC>>EP_NgU.

3rd Change

6.4.1	Attribute properties

| …… |  |  |
| --- | --- | --- |
| serviceType | An attribute specifies the standardized network slice type. allowedValues: eMBB, URLLC, MIoT, V2X. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: True |
| epApplicationRef | This parameter specifies a list of application level EPs (i.e. EP_N3 or EP_NgU) associated with the logical transport interface. | type: DN multiplicity: * isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| epTransportRef | This parameter specifies a list of transport level EPs associated with the application level EP (i.e. EP_N3 or EP_NgU) or network slice subnet. | type: DN multiplicity: * isOrdered: N/A isUnique: True defaultValue: None isNullable: True |
| sliceSimultaneousUse | This attribute describes whether a network slice can be simultaneously used by a device together with other network slices and if so, with which other classes of network slices. allowedValues: “0”, “1”, “2”, “3”, “4”. “0”: Can be used with any network slice “1”: Can be used with network slices with same SST value “2”: Can be used with any network slice with same SD value “3”: Cannot be used with another network slice “4”: Cannot be used by a UE in a specific location | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| NOTE 1: There is no direct relationship between localAddress/remoteAddress in EP_RP and ipAddress in EP_transport. While the localAddress/remoteAddress in EP_RP could be exchanged as part of signalling between GTP-u tunnel end points, ipAddress in EP_transport is used for transport routing. NOTE 2: void |  |  |

End of Change