3GPP TSG SA WG5 Meeting 137-e		TDoc S5-213285

# electronic meeting, online, 10 - 19 May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.251 | CR | 0518 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of PSCell ID in SGW CDR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDCE5-CH, TEI17 |  |  |  |  |  | Date: |  |  | 2021-04-30 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Based on the SA5 reply LS (S5-197481) to ETSI TC LI, SA2 has been requested (in S5‑202022 SA5 was in copy) to make the PSCELL ID available at the SGW in EPC scenario supporting 5G New Radio via Dual Connectivity. The objective was to capture this information in SGW CDR to comply with Retained Data regulations. TS 23.401 has been updated for the MME to pass PSCell ID to SGW in different procedures, which can be captured in SGW CDRs |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce PSCell ID in SGW CDR |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Retained data regulations cannot be fulfilled in EPC scenario supporting 5G New Radio via Dual Connectivity |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1.11.1, 6.1.2, 6.3.1.2, 6.3.1.4, 6.3.2, 6.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

##### 5.2.1.11.1	General

Volume reporting for Secondary RAT is an optional capability in the S-GW and P-GW that provides accounting functionality when a Secondary RAT is used in conjunction with E-UTRAN. This is valid for both HPLMN and VPLMN.

To align with existing per IP-CAN bearer accounting procedures, the following principles are used:

-	The reporting of Secondary RAT Data Volume is controlled by the E-UTRAN.

-	The uplink and downlink data volumes for the Secondary RAT are reported (from E-UTRAN to EPC) on a per EPS bearer basis. The report contains Secondary RAT (e.g. NR) resources used for transport of user data and indicated separately for uplink and downlink per EPS bearer and per time interval. The time interval used for the measurements reported (from RAN) may be partitioned to indicate usage that occurred before respectively after an absolute time (that occurs while measurement for secondary RAT usage report is ongoing). For example, the RAN is configured to partition reports in usage, that occurred before respectively after midnight.

-	The reporting (performed by E-UTRAN) in association with UE-related control signaling and via standalone reporting is internally triggered by E-UTRAN.

When available by the S-GW, the PSCELL Id associated to the Secondary RAT is collected for SGW-CDR.

Next change

### 6.1.2	IP-CAN bearer charging data in S-GW (SGW-CDR)

If the collection of CDR data is enabled then the S-GW data specified in table 6.1.2.1 shall be available for each IP-CAN bearer.

Table 6.1.2.1: S-GW IP-CAN bearer data (SGW-CDR)

| Field | Category | Description |
| --- | --- | --- |
| Record Type | M | S-GW IP-CAN bearer record. |
| Retransmission | OC | This parameter, when present, indicates that information from retransmitted Charging Data Request has been used in this CDR. |
| Served IMSI | C | IMSI of the served party, if available. |
| IMSI Unauthenticated Flag | OC | This field indicates the provided served IMSI is not authenticated (emergency bearer service situation). |
| Served IMEI | OC | The IMEI or IMEISV of the ME, if available. It is used for identifying the user in case Served IMSI is not present during emergency bearer service. |
| S-GW Address used | M | The control plane IP address of the S-GW used. |
| S-GW Address IPv6 | OC | The control plane IPv6 address, in case of IPv4v6 dual stack, of the S-GW. |
| Charging ID | M | IP-CAN bearer Charging identifier used to identify this IP-CAN bearer in different records created by PCNs |
| PDN Connection Charging Id | OM | This field holds the explicit Charging Id for the PDN connection if received from the P-GW. Otherwise, the field holds the Charging Id of the EPS default bearer in GTP case, or the unique Charging Id of the IP-CAN session in PMIP case. This field is used to identify different records belonging to same PDN connection. |
| Serving Node Address | M | List of serving node control plane IP addresses (e.g. S4-SGSN, MME, …) used during this record. |
| Serving Node IPv6 Address | OC | List of serving node control plane IPv6 addresses, in case of IPv4v6 dual stack, (e.g. S4-SGSN, MME, …) used during this record. |
| Serving node Type | M | List of serving node types in control plane. The serving node types listed here map to the serving node addresses listed in the field "Serving node Address" in sequence. |
| S-GW Change | OC | Present if this is first record after a change from another serving node (i.e. SGW, ePDG, HSGW). |
| PGW PLMN Identifier | Oc | PLMN identifier (MCC MNC) of the P-GW used. |
| Access Point Name Network Identifier | OM | The logical name of the connected access point to the external packet data network (network identifier part of APN). |
| PDP/PDN Type | OM | This field indicates PDN type (i.e. IPv4, IPv6 or IPv4v6). |
| PDP/PDN Type extension | OM | This field indicates Non-IP PDN type. |
| Served PDP/PDN Address | OC | IP address allocated for the PDP context / PDN connection, if available, i.e. IPv4 address when PDN Type is IPv4 or IPv6 prefix when PDN Type is IPv6 or IPv4v6. For non-IP PDN connection, this field is not present |
| Served PDP/PDN Address prefix length | OC | PDP/PDN Address prefix length of an IPv6 typed Served PDP Address. The field needs not available for prefix length of 64 bits. For non-IP PDN connection, this field is not present. |
| Served PDP/PDN Address extension | OC | This field holds IPv4 address of the served IMSI, if available, when PDN type is IPv4v6. For non-IP PDN connection, this field is not present. |
| Dynamic Address Flag | OC | Indicates whether served PDP/PDN address is dynamic, which is allocated during IP-CAN bearer activation, initial attach (E-UTRAN or over S2x) and UE requested PDN connectivity. This field is missing if IPv4 address is static when PDN Type is IPv4, or if IPv6 address is static when PDN Type is IPv6 or IPv4v6. |
| Dynamic Address Flag extension | OC | Indicates whether served IPv4 PDP/PDN address is dynamic, which is allocated during IP-CAN bearer activation, initial attach (E-UTRAN or over S2x) and UE requested PDN connectivity with PDP/PDN type IPv4v6. This field is missing if IPv4 address is static. |
| List of Traffic Data Volumes | OM | A list of changes in charging conditions for this QCI/ARP pair, each change is time stamped. Charging conditions are used to categorize traffic volumes, such as per tariff period. Initial and subsequently changed QoS and corresponding data values are also listed. |
| Record Opening Time | M | Time stamp when IP-CAN bearer is activated in this S-GW or record opening time on subsequent partial records. |
| MS Time Zone | OC | This field contains the MS Time Zone the MS is currently located as defined in TS 29.060 [203], if available. |
| Last MS Time Zone | OC | This field contains the UE Time Zone the UE is located at IP-CAN bearer deactivation, or at default bearer deactivation when IP-CAN session charging is active, when available. |
| Duration | M | Duration of this record in the S-GW. |
| Cause for Record Closing | M | The reason for the release of record from this S-GW. |
| Diagnostics | OM | A more detailed reason for the release of the IP-CAN bearer, when a single cause is applicable. |
| Enhanced Diagnostics | OC | This field holds a more detailed reason for the release of the IP-CAN bearer, when a set of causes is applicable. |
| Record Sequence Number | C | Partial record sequence number, only present in case of partial records. |
| Node ID | OM | Name of the recording entity. |
| Local Record Sequence Number | OM | Consecutive record number created by this node. The number is allocated sequentially including all CDR types. |
| APN Selection Mode | OM | An index indicating how the APN was selected. |
| Served MSISDN | OC | The primary MSISDN of the subscriber, if available. |
| User Location Information | OC | This field contains the User Location Information of the MS as defined in TS 29.060 [203] for GPRS case, and in TS 29.274 [210] for EPC case, if available. |
| User Location Information Time | OC | This field contains the time at which the user location information was acquired. |
| PSCell Information | OC | This field holds the PSCell information: Primary SCG (Secondary Cell Group) Cell information |
| Last User Location Information | OC | This field contains the User Location Information of the UE at IP-CAN bearer deactivation, or at default bearer deactivation when IP-CAN session charging is active, when available. |
| User CSG information | OC | This field contains the User CSG information of the UE, if available, including CSG ID, access mode and CSG membership indication. |
| Presence Reporting Area Information | OC | This grouped field contains the Presence Reporting Area Identifierfor the UE PDN connection and the initial status of the UE presence in the Presence Reporting Area, if available. This field is not applicable to multiple PRA(s) |
| Charging Characteristics | M | The Charging Characteristics applied to the IP-CAN bearer. |
| Charging Characteristics Selection Mode | OM | Holds information about how Charging Characteristics were selected. |
| IMS Signalling Context | OC | Included if the IM-CN Subsystem Signalling Flag is set, see TS 23.060 [201] IP-CAN bearer is used for IMS signalling. |
| P-GW Address used. | OC | This field is the P-GW IP Address for the Control Plane |
| P-GW Address IPv6 | OC | This field is the P-GW IPv6 Address, in case of IPv4v6 dual stack, for the Control Plane |
| Serving Node PLMN Identifier | OC | This field holds the PLMN Identifier (MCC and MNC) serving the UE during this record, as received in the "Serving Network” IE over S4/S11, if available. |
| CN Operator Selection Entity | OC | This field indicates whether the Serving Network identified by the "Serving Node PLMN Identifier” has been selected by the UE or by the network, as defined in TS 29.060 [203] for GPRS, and in TS 29.274 [210] for EPC, if available. |
| RAT Type | OC | This field indicates the Radio Access Technology (RAT) type currently used by the Mobile Station as defined in TS 29.061 [205], when available. |
| Start Time | OC | This field holds the time when User IP-CAN session starts, available in the CDR for the first bearer in an IP-CAN session. |
| Stop Time | OC | This field holds the time when User IP-CAN session is terminated, available in the CDR for the last bearer in an IP-CAN session. |
| Low Priority Indicator | OC | This field indicates if this IP-CAN session has a low priority, i.e. for Machine Type Communication. |
| CP CIoT EPS Optimisation indicator | OC | This field indicates whether CP CIoT EPS Optimisation is used during data transfer with the UE (i.e. Control Plane NAS PDU via S11-U between SGW and MME) or not (i.e.User Plane via S1-U between SGW and eNB), if enabled. |
| UNI PDU CP Only Flag | OC | This field indicates whether this PDN connection is applied with "Control Plane Only Flag" for UNI PDU transfer, i.e. transfer only using Control Plane NAS PDU (Control Plane CIoT EPS optimisation). |
| Serving PLMN Rate Control | Oc | This field holds the Serving PLMN Rate Control used by the MME during this record. |
| MO exception data counter | Oc | This field holds the MO exception data counter value with the timestamp indicating the time at which the counter value increased from 0 to 1, as defined in TS 29.274 [210]. |
| Record Extensions | OC | A set of network operator/manufacturer specific extensions to the record. Conditioned upon the existence of an extension. |
| List of RAN Secondary RAT Usage Reports | OC | A list of containers with volumes reported from RAN for secondary RAT usage, each container is time stamped. |

Next change

#### 6.3.1.2	Definition of the PS information

PS specific charging information is provided within the PS Information. The fields of the PS Information are indicated with the node (MME, S-GW, ePDG, TWAG, P-GW, TDF) from which the information is sent.

The detailed structure of the PS Information can be found in table 6.3.1.2.1.

Table 6.3.1.2.1: Structure of the PS Information

| Information Element | Category | Description |
| --- | --- | --- |
| Supported Features | OC | This field holds the list of features supported by the PCN, CDF or OCF as defined in clause 6.3.1.x. |
| Charging Id | OC | This field holds the Charging Id for this IP-CAN bearer (this together with the P-GW Address constitutes a unique identifier for the IP-CAN bearer). This field holds the unique Charging Id in the PMIP case. When charging per IP-CAN session is active, PGW use this field to hold the Charging Id of the EPS default bearer. It is the same as the PDN Connection Charging Id. |
| Charging per IP-CAN Session Indicator | Oc | This field indicates whether charging per IP-CAN session is active. If this field is not present, charging per IP-CAN session is not active. |
| Node Id | OC | This fields holds the name of the Node |
| PDN Connection Charging Id | OC | This field holds the explicit Charging Id for the PDN connection. When NBIFOM is supported, the PGW will assign an explicit PDN Connection Charging ID for the PDN connection. Otherwise the field holds the Charging Id of the EPS default bearer. This field is used to identify different records belonging to same PDN connection. |
| PDP/PDN Type | OC | This field holds the type of IP-CAN bearer, e.g. IP or PPP, or PDN type (i.e. IPv4, IPv6 or IPv4v6, or Non-IP). |
| SGi PtP Tunnelling Method | OC | This field indicates whether SGi PtP tunnelling method based on UDP/IP or other methods are used for this PDN connection when a non-IP PDN type. |
| SCS/AS Address | OC | This field holds the Address of SCS/AS for SGi PtP tunnelling. |
| PDP/PDN Address | OC | This field holds the IP address of the served IMSI allocated for the PDP context / PDN connection, i.e. IPv4 address or IPv6 prefix, or assigned IP address if any when PDP/PDN Type is Non-IP. This parameter shall be present except: - when both the PDP type is PPP and dynamic PDP address assignment is used or, - when PDP/PDN Type is Non-IP, and no IP address has been assigned by the PGW for the PDN connection. It may occur twice within the PS Information field when PDN type is IPv4v6: first occurrence with IPv6 prefix, second occurrence with IPv4 address. |
| PDP/PDN Address prefix length | OC | PDP/PDN Address prefix length of an IPv6 typed Served PDP Address. The field needs not available for prefix length of 64 bits. |
| Dynamic Address Flag | OC | This field indicates whether served PDP/PDN address is dynamically allocated. This field is missing if address is static. |
| Dynamic Address Flag Extension | OC | Indicates whether served IPv4 PDP/PDN address is dynamic, which is allocated during IP-CAN bearer activation, initial attach (E-UTRAN or over S2x) and UE requested PDN connectivity with PDP/PDN type IPv4v6. This field is missing if IPv4 address is static. |
| Negotiated QoS Profile | OC | This field holds the authorized QoS applied to IP-CAN bearer. See NOTE. This field indicates the bandwith limitation, applied to TDF session in TDF case. When charging per IP-CAN session is active in P-GW, this field indicates the APN-AMBR applied to the IP-CAN session. |
| Serving Node Address | OC | This field holds the SGSN/S-GW/TWAG IP address that is used by the control plane for the handling of control messages, or the AGW IP address, or the ePDG address, or the MME address. It may be used to identify the PLMN to which the user is attached. It may occur twice in the case when the serving node has IPv4v6 dual stack control plane. |
| Serving Node Type | OC | This field holds the type of the serving node (SGSN/S-GW/ePDG/AGW/TWAG from PGW/TDF, or SGSN/MME from SGW). |
| SGW Change | OC | This field is present if this is first Charging Data Request after a change from another serving node (from SGW/ePDG/ TWAG/HSGW). |
| PGW Address | OC | This field holds the IP-address of the P-GW that generated the Charging Id. It may occur twice in the case when the P-GW has IPv4v6 dual stack control plane. |
| TDF Address | OC | The control plane IP address of the TDF used. It may occur twice in the case when the TDF has IPv4v6 dual stack control plane. |
| SGW Address | OC | This field holds the IP-address of the S-GW used. It may occur twice in the case when the P-GW has IPv4v6 dual stack control plane. |
| ePDG Address | OC | This field holds the IP-address of the ePDG used. It may occur twice in the case when the ePDG has IPv4v6 dual stack control plane. |
| TWAG Address | OC | This field holds the IP-address of the TWAG used. It may occur twice in the case when the TWAG has IPv4v6 dual stack control plane. |
| MME Number for MT SMS | OC | This field holds the MME international PSTN/ISDN E.164 number used for "SMS over MME Charging". |
| MME Name | OC | This field holds the operation Identity of the MME used for "SMS over MME Charging". |
| MME Realm | OC | This field holds the operation Realm Identity of the MME used for "SMS over MME Charging". |
| CG Address | OC | This field holds the Charging Gateway IP address if available. Only used in EPC Online Charging. |
| IMSI MCC MNC | OC | This field holds the MCC and MNC extracted from the user's IMSI (first 5 or 6 digits, as applicable from the presented IMSI). |
| IMSI Unauthenticated Flag | OC | This field is present when IMSI is provided in Subscription Id and this IMSI is unauthenticated (due to emergency bearer service situations). |
| PGW MCC MNC | OC | This field holds the MCC-MNC of the network the P-GW belongs to. |
| TDF MCC MNC | OC | PLMN identifier (MCC MNC) of the TDF. |
| NSAPI | OC | This field Identifies a particular IP-CAN bearer for the associated PDN and MSISDN/IMSI from creation to deletion. See NOTE. |
| Called Station Id | OC | This field contains the identifier of the access point (APN) the user is connected to. |
| Session Stop Indicator | OC | This field indicates to the OCS that the last IP-CAN bearer of a session is released and that the IP-CAN session has been terminated. This field only indicates to the OCS that the TDF session is terminated in TDF case. |
| Selection Mode | OC | This field contains the Selection mode for this APN received in the Create PDP context request, on E-UTRAN initial attach and UE requested PDN connectivity, or Attach in WLAN on GTP S2a. |
| Charging Characteristics | OC | This field contains the Charging Characteristics for this IP-CAN bearer received in the Create IP-CAN bearer Request Message (only available in R99 and later releases). |
| Charging Characteristics Selection Mode | OC | This field holds information about how the "Charging Characteristics" was selected. |
| Serving Node MCC MNC | OC | This field holds the MCC and MNC serving the UE: extracted from the RAI within the IP-CAN bearer activation or Update messages received over Gn/Gp, or provided by the SGW in the "Serving Network" IE over S5/S8 or by AGW/TWAG (For TWAN, PLMN identifier used for UE authentication, i.e. the VPLMN in roaming case, and the HPLMN in non-roaming case). |
| CN Operator Selection Entity | OC | This field indicates whether the Serving Network identified by the "Serving Node MCC MNC" has been selected by the UE or by the network, as defined in TS 29.060 [203] for GPRS, and in TS 29.274 [210] for EPC, if available. |
| MS Time Zone | OC | This field indicates the offset between universal time and local time in steps of 15 minutes of where the MS currently resides. |
| Charging Rule Base Name | OC | This field indicates the name of a pre-defined group of PCC rules residing at the PCEF. It may occur several times within the PS Information field. |
| ADC Rule Base Name | OC | This field indicates the name of a pre-defined group of ADC rules in TDF case. |
| User Location Info | OC | This field indicates details of where the UE is currently located (e.g. SAI, TAI, RAI, CGI, ECGI or access-specific user location information). |
| User Location Info Time | OC | This field contains the time at which the user location information was acquired, if available. |
| PSCell Information | OC | This field holds the PSCell information: Primary SCG (Secondary Cell Group) Cell information |
| User CSG Information | OC | This field indicates details of the User CSG Information of the UE, if available, including CSG ID, access mode and CSG membership indication. |
| 3GPP2 User Location Info | OC | This field holds the 3GPP2 User Location Info of where the UE is currently located (i.e. 3GPP2 BSID: Cell-Id, SID, NID), as defined in TS 29.212 [216] |
| TWAN User Location Information | OC | This field contains the UE location in a Trusted WLAN Access Network (TWAN) , if available: - as "TWAN identifier" defined in TS 29.274 [210] for S2a GTP-based case, or, - as "Access Network Identifier Option IE" defined in 29.275 [211] for S2a PMIP-based case. |
| UWAN User Location Information | OC | This field contains the UE location in an Untrusted Wireless Access Network (UWAN) which includes the UE local IP address and optionally UDP or TCP source port number (if NAT is detected) as defined in TS 29.274 [210]. It may also include WLAN location information the ePDG may have received from the 3GPP AAA server about the UE as "WLAN location information" defined in TS 29.274 [210]. |
| Presence Reporting Area Information | OC | This field contains part of the Presence Reporting Area Information of UE as defined in TS 29.212 [216], comprising the Presence Reporting Area identifier(s) the Presence Reporting Area(s) status (UE presence or not, inactive), and which Node subscribed-to, or the Presence Reporting Area is set to inactive, if available. It may occur several times within the PS Information field. |
| RAT Type | OC | This field indicates which Radio Access Technology (RAT) is currently serving the UE as defined in TS 29.061 [205]. When NBIFOM is accepted by PCRF, this field holds the first default bearer’s RAT type when charging per IP-CAN session is active. |
| Furnish Charging Information | OC | This field contains the PS Furnish Information Elements. |
| Offline Charging | OC | This field contains the Offline Charging parameters to control offline charging. |
| PDP Context Type | OC | This field indicates the type of a PDP context (i.e. Primary or Secondary). This field is present when using the Gn/Gp reference point only. |
| Traffic data volumes | OC | This field holds the containers associated to a charging condition change on an IP-CAN bearer. This is included when triggers conditions are met (Qos change, tariff time change ...). It may occur several times within the PS Information field. |
| Service data container | OC | This field holds the container associated to a service condition change on a service data flow (categorized per rating group or per combination of the rating group and service id) within this IP-CAN bearer. For TDF case, this field holds the container associated to a charging condition change on an application traffic (categorized per rating group or per combination of the rating group and service id) within this TDF session. It may occur several times within the PS Information field. |
| User Equipment Info | OC | This field holds the identification of the terminal (IMEI or IMEISV…) It is used for identifying the user in case IMSI is not present during emergency bearer service. Editor's Note: Alignement with online charging is needed. |
| Terminal Information | OC | This field holds the identification of the terminal (IMEI or IMEISV, 3GPP2-MEID..).It is used for identifying the user in case IMSI is not present during emergency bearer service. Editor's Note : this parameter is only for offline charging and usage for online charging instead of User-Equipment-info is ffs |
| Start Time | OC | Timestamp when User IP-CAN/TDF session starts. |
| Stop Time | OC | Timestamp when User IP-CAN/TDF session terminates. |
| Change Condition | OC | This field holds the reason for sending Charging Data Request from the PCN Nodes. |
| Diagnostics | OC | This field holds a more detailed reason for the release of theIP-CAN bearer, when a single cause is applicable, and complements the "Change Condition" information. When charging per IP-CAN session is active, PGW uses this field to hold a more detailed reason for the release of the IP-CAN session, when a single cause is applicable. |
| Enhanced Diagnostics | OC | This field holds a more detailed reason for the release of the connection, when a set of causes is applicable. When charging per IP-CAN session is active, PGW uses this field to hold a more detailed reason for the release of the IP-CAN session, when a set of causes is applicable. |
| Low Priority Indicator | OC | This field indicates if this IP-CAN session has a low priority, i.e. for Machine Type Communication. |
| NBIFOM Support | OC | This field indicates that NBIFOM was requested by the UE, supported and accepted by the network for the IP-CAN session or if NBIFOM is not supported for the IP-CAN session. |
| NBIFOM Mode | OC | This field indicates the NBIFOM mode selected for the PDN connection, when NBIFOM is supported for the IP-CAN session. This field is only present when NBIFOM is accepted. |
| CP CIoT EPS Optimisation indicator | OC | This field indicates whether CP CIoT EPS Optimisation is used during data transfer with the UE (i.e. Control Plane NAS PDU via S11-U between SGW and MME) or not (i.e. User Plane via S1-U between SGW and eNB), if enabled. |
| UNI PDU CP Only Flag | OC | This field indicates whether this PDN connection is applied with "Control Plane Only Flag" for UNI PDU transfer, i.e. transfer only using Control Plane NAS PDU (Control Plane CIoT EPS optimisation). |
| Serving PLMN Rate Control | Oc | This field holds the Serving PLMN Rate Control used by the MME. |
| MO exception data counter | Oc | This field holds the MO exception data counter value with the timestamp indicating the time at which the counter value increased from 0 to 1, as defined in TS 29.274 [210]. |
| APN Rate Control | Oc | This field holds the list of APN Rate Controls enforced in the PGW. |
| 3GPP PS Data Off Status | Oc | This field holds the 3GPP Data off Status when UE’s 3GPP Data Off status is Activated or Deactivated. |
| Unused Quota Timer | Oc | This field holds the threshold for the time period without any quota granted or any quota being consumed for any rating group belonging to the Gy session. It holds either the value configured in PCEF, if it is supported, or the value to be used as received from the OCS. A value of zero indicates that this mechanism shall not be used. |

NOTE:	For network requested secondary IP-CAN bearer activation, the CCR[Initial] does not contain a value of NSAPI, whilst the Negotiated QoS profile reflects the requested QoS profile used in the activation request.

Editor’s Note: The handling of  "CP CIoT EPS Optimisation" when charging per IP-CAN session applies is FFS.

Next change

#### 6.3.1.4	Supported features

The Supported features information that is used for PS domain charging is based on the information defined for the supported features mechanism specified in clause 6.5.10 TS 32.299 [50].

The following table defines the features applicable for the feature lists with a Feature-List-ID of 1.

# Table 6.3.1.4: Features of Feature-List-ID 1 used in PS charging

| Feature bit | Feature | Description | Online/Offline |
| --- | --- | --- | --- |
| 0 | EDCE5-CH | This feature indicates the support of 5G New Radio via Dual Connectivity based on clause 5.2.1.11 Data Volume Reporting for Secondary RAT usage. | Offline |
| 1 | eFMSS_CH | This feature indicates the support of Flexible Mobile Service Steering based on clause 5.1.6. | Both |
| 2 | WAEPC_CH_Untrusted | This feature indicates the support of extensions of UWAN location information | Both |
| 3 | WAEPC_CH_trusted | This feature indicates the support of extensions of TWAN location information | Both |
| 4 | EDCE5-CH_Extended-BW-NR | This feature indicates the support of 5G New Radio extended QoS. | Both |
| 5 | EDCE5-CH_PSCELL_ID | This feature indicates the support of PSCELL ID when Dual Connectivity is supported | Offline |
| Feature bit: The order number of the bit within the Feature-List AVP where the least significant bit is assigned number "0". Feature: A short name that can be used to refer to the bit and to the feature, e.g. "EPS". Feature in this table is always mandatory since it relates to the charging functionality of a network feature. Description: A clear textual description of the feature. |  |  |  |

Next change

### 6.3.2	Detailed message format for offline charging

The following clause specifies per Operation Type the charging data that are sent by MME, S-GW, ePDG, TWAG, P-GW and TDF.

The Operation Types are listed in the following order: S (Start)/I (Interim)/S (Stop)/E (Event). Therefore, when all Operation Types are possible it is marked as SISE. If only some Operation Types are allowed for a node, only the appropriate letters are used (i.e. SIS or E) as indicated in the table heading. The omission of an Operation Type for a particular field is marked with "-" (i.e. SI-E). Also, when an entire field is not allowed in a node the entire cell is marked as "-".

Table 6.3.2.1 illustrates the basic structure of the supported fields in the Charging Data Request message for PS offline charging.

Table 6.3.2.1: Supported fields in Charging Data Request message

| Information Element | Node Type | S-GW | ePDG | P-GW | TWAG | TDF |
| --- | --- | --- | --- | --- | --- | --- |
|  | Supported Operation Types | S/I/S/E | S/I/S/E | S/I/S/E | S/I/S/E | S/I/S/E |
| Session Identifier |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Originator Host |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Originator Realm |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Destination Domain |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Operation Type |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Operation Number |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Operation Identifier |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| User Name |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Destination Host |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Operation Interval |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Origination State |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Origination Timestamp |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Proxy Information |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Route Information |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Operation Token |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Service Information with PS and IMS Information |  |  |  |  |  |  |
| Supported Features |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Subscription Id |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| IMSI Unauthenticated Flag |  | SIS- | SIS- | SIS- | SIS- | - |
| Node Functionality |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Charging Id |  | SIS- | SIS- | SIS- | SIS- | - |
| Charging per IP-CAN Session Indicator |  | - | - | S--- | - | - |
| Node Id |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| PDN Connection Charging Id |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| PDP/PDN Type |  | SIS- | SIS- | SIS- | SIS- | - |
| SGi PtP Tunnelling Method |  | - | - | SIS- | - | - |
| SCS/AS Address |  | - | - | SIS- | - | - |
| PDP/PDN Address |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| PDP/PDN Address prefix length |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Dynamic Address Flag |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Dynamic Address Flag Extension |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Serving Node Address |  | SIS- | - | SIS- | - | SIS- |
| Serving Node Type |  | SIS- | - | SIS- | - | SIS- |
| SGW Change |  | S--- | S--- | - | S--- | - |
| P-GW Address |  | SIS- | - | SIS- | SIS- | SIS- |
| TDF Address |  | - | - | - | - | SIS- |
| SGW Address |  | SIS- | - | - | - | - |
| ePDG Address |  | - | SIS- | - | - | - |
| TWAG Address |  | - | - | SIS- | - | - |
| CG Address |  | - | - | - | - | - |
| IMSI MCC MNC |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| PGW MCC MNC |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| TDF MCC MNC |  | - | - | - | - | SIS- |
| NSAPI |  | - | - | SIS- | - | - |
| Called Station Id |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Session Stop Indicator |  | - | - | --S- | - | --S- |
| Selection Mode |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Charging Characteristics |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Charging Characteristics Selection Mode |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Serving Node MCC MNC |  | SIS- | - | SIS- | - | SIS- |
| MS Time Zone |  | SIS- | - | SIS- | - | SIS- |
| Charging Rule Base Name |  | - | - | SIS- | - | - |
| ADC Rule Base Name |  | - | - | - | - | SIS- |
| User Location Info |  | SIS- | - | SIS- | - | SIS |
| User Location Info Time |  | SIS- | - | SIS- | - | - |
| PSCell Information |  | SIS- | - | - | - | - |
| User CSG Information |  | SIS- | - | SIS- | - | SIS- |
| 3GPP2 User Location Info |  | SIS- | - | SIS- | - | SIS- |
| TWAN User Location Information |  | - | - | SIS- | SIS- | - |
| UWAN User Location Information |  | - | SIS- | SIS- | - | - |
| Presence Reporting Area Information |  | SIS- | - | SIS- | - | - |
| RAT Type |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Furnish Charging Information |  | - | - | SIS- | - | - |
| Offline Charging |  | - | - | - | - | - |
| QoS Information |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Traffic data volumes |  | -IS- | -IS- | - | -IS- | - |
| Service data container |  | - | - | -IS- | - | -IS- |
| User Equipment Info |  | - | - | - | - | S--- |
| Terminal-Information |  | S--- | S--- | S--- | S--- | S--- |
| Start time |  | S--- | S--- | S--- | S--- | S--- |
| Stop time |  | --S- | --S- | --S- | --S- | --S- |
| Change Condition |  | -IS- | -IS- | -IS- | -IS- | -IS- |
| Diagnostics |  | --S- | --S- | --S- | --S- | --S- |
| Enhanced Diagnostics |  | --S- | --S- | --S- | --S- | - |
| Low Priority Indicator |  | S--- | - | S--- | - | - |
| CN Operator Selection Entity |  | SIS- | - | SIS- | - | - |
| NBIFOM Support |  | - | - | S--- | - | - |
| NBIFOM Mode |  | - | - | S--- | - | - |
| CP CIoT EPS Optimisation Indicator |  | SIS- | - | - | - | - |
| UNI PDU CP Only Flag |  | SIS- | - | SIS- | - | - |
| Serving PLMN Rate Control |  | SI-- | - | SI-- | - | - |
| APN Rate Control |  | - | - | SI-- | - | - |
| MO exception data counter |  | SIS- | - | SIS- | - | - |
| 3GPP PS Data Off Status |  | - | - | SIS- | - | - |
| RAN Secondary RAT Usage Reports |  | -IS- | - | -IS- | - | - |

Table 6.3.2.1a illustrates the basic structure of the supported fields in the Charging Data Request message for "SMS over MME Charging".

Table 6.3.2.1a: Supported fields in Charging Data Request message for "SMS over MME Charging"

| Information Element | Node Type | MME |
| --- | --- | --- |
|  | Supported Operation Types | S/I/S/E |
| Session Identifier |  | ---E |
| Originator Host |  | ---E |
| Originator Realm |  | ---E |
| Destination Domain |  | ---E |
| Operation Type |  | ---E |
| Operation Number |  | ---E |
| Operation Identifier |  | ---E |
| User Name |  | ---E |
| Destination Host |  | ---E |
| Operation Interval |  | ---E |
| Origination State |  | ---E |
| Origination Timestamp |  | ---E |
| Proxy Information |  | ---E |
| Route Information |  | ---E |
| Operation Token |  | ---E |
| Service Information with PS, IMS, MMS and SMS Information |  |  |
| Subscription Id |  | ---E |
| Node Functionality |  | ---E |
| Node Id |  | ---E |
| MME Number for MT SMS |  | ---E |
| MME Name |  | ---E |
| MME Realm |  | ---E |
| Serving Node Type |  | ---E |
| SGSN Address |  | ---E |
| Charging Characteristics |  | ---E |
| Charging Characteristics Selection Mode |  | ---E |
| 3GPP User Location Info |  | ---E |
| User Location Info Time |  | ---E |
| RAT Type |  | ---E |
| Terminal Information |  | ---E |
| Recipient Address |  | ---E |
| SMSC Address |  | ---E |
| Originator Address |  | ---E |
| Submission Time |  | ---E |
| Message Id |  | ---E |
| CN Operator Selection Entity |  | ---E |

Table 6.3.2.2 illustrates the basic structure of the supported fields in the Charging Data Response message for PS offline charging, and "SMS over MME Charging".

Table 6.3.2.2: Supported fields in Charging Data Response message

| Information Element | Node Type | S-GW | ePDG | TWAG | P-GW | TDF |
| --- | --- | --- | --- | --- | --- | --- |
|  | Supported Operation Types | S/I/S/E | S/I/S/E | S/I/S/E | S/I/S/E | S/I/S/E |
| Session Identifier |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Operation Result |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Originator Host |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Originator Domain |  |  |  |  |  |  |
| Operation Type |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Operation Number |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Operation Identifier |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Operation Interval |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Error Reporting Host |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Origination State |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Origination Timestamp |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Proxy Information |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Route Information |  | SIS- | SIS- | SIS- | SIS- | SIS- |
| Service Information with PS Information |  |  |  |  |  |  |
| Supported Features |  | SIS- | SIS- | SIS- | SIS- | SIS- |

Next change

### 6.3.3	Detailed message format for online charging

The following table specifies per Operation type the charging data that are sent P-GW and TDF network element for:

The Operation types are listed in the following order: I [initial]/U [update]/T [terminate]/E [event]. Therefore, when all Operation types are possible it is marked as IUTE. If only some Operation types are allowed for a node, only the appropriate letters are used (i.e. IUT or E) as indicated in the table heading. The omission of an Operation type for a particular field is marked with "-" (i.e. IU-E). Also, when an entire filed is not allowed in a node the entire cell is marked as "-".

Note that not for all structured fields the individual field members are listed in the table. Detailed descriptions of the fields are provided in TS 32.299 [50].

Table 6.3.3.1 illustrates the basic structure of the supported fields in the Debit / Reserve Units Request for PS online charging.

Table 6.3.3.1: Supported fields in Debit / Reserve Units Request message

| Information Element | Node Type | P-GW | TDF |
| --- | --- | --- | --- |
|  | Supported Operation Types | I/U/T/E | I/U/T/E |
| Session Identifier |  | IUT- | IUT- |
| Originator Host |  | IUT- | IUT- |
| Originator Domain |  | IUT- | IUT- |
| Destination Domain |  | IUT- | IUT- |
| Operation Identifier |  | IUT- | IUT- |
| Operation Token |  | IUT- | IUT- |
| Operation Type |  | IUT- | IUT- |
| Operation Number |  | IUT- | IUT- |
| Destination Host |  | IUT- | IUT- |
| User Name |  | IUT- | IUT- |
| Origination State |  | IUT- | IUT- |
| Origination Timestamp |  | IUT- | IUT- |
| Subscriber Identifier |  | IUT- | IUT- |
| Termination Cause |  | --T- | --T- |
| Requested Action |  | IUT- | IUT- |
| Multiple Operation |  | IU-- | IU-- |
| Multiple Unit Operation |  | IUT- | IUT- |
| Subscriber Equipment Number |  | IUT- | IUT- |
| Route Information |  | IUT- | IUT- |
| Service Information |  | IUT- | IUT- |
| Service Information with IMS and PS Information |  |  |  |
| Supported Features |  | IUT- | IUT- |
| Node Functionality |  | IUT- | IUT- |
| Charging Id |  | IUT- | - |
| Node Id |  | IUT- | IUT- |
| PDN Connection Charging Id |  | IUT- | IUT- |
| PDP/PDN Type |  | IUT- | - |
| SGi PtP Tunnelling Method |  | IUT- | - |
| SCS/AS Address |  | IUT- | - |
| PDP/PDN Address |  | IUT- | IUT- |
| PDP/PDN Address prefix length |  | IUT- | IUT- |
| Dynamic Address Flag |  | IUT- | IUT- |
| Dynamic Address Flag Extension |  | IUT- | IUT- |
| QoS Information |  | IUT- | IUT- |
| Serving Node Address |  | IUT- | IUT- |
| Serving Node Type |  | IUT- | IUT- |
| SGW Change |  | - | - |
| P-GW Address |  | IUT- | IUT- |
| TDF Address |  | - | IUT- |
| CG Address |  | IUT- | IUT- |
| IMSI MCC MNC |  | IUT- | IUT- |
| IMSI Unauthenticated Flag |  | IUT- | - |
| PGW MCC MNC |  | IUT- | IUT- |
| TDF MCC MNC |  | - | IUT- |
| NSAPI |  | IUT- | - |
| Called Station Id |  | IUT- | IUT- |
| Session Stop Indicator |  | --T- | --T- |
| Selection Mode |  | IUT- | IUT- |
| Charging Characteristics |  | IUT- | IUT- |
| Charging Characteristics Selection Mode |  | IUT- | IUT- |
| Serving Node MCC MNC |  | IUT- | IUT- |
| MS Time Zone |  | IUT- | IUT- |
| Charging Rule Base Name |  | IUT- | - |
| ADC Rule Base Name |  | - | IUT- |
| User Location Info |  | IUT- | IUT- |
| User Location Info Time |  | IUT- | - |
| PSCell Information |  | - | - |
| User CSG Information |  | IUT- | IUT- |
| 3GPP2 User Location Info |  | IUT- | IUT- |
| TWAN User Location Information |  | IUT- | IUT- |
| UWAN User Location Information |  | IUT- | - |
| Presence Reporting Area Information |  | UT- | - |
| RAT Type |  | IUT- | IUT- |
| Furnish Charging Information |  | - | - |
| Offline Charging |  | IUT- | IUT- |
| PDP Context Type |  | IUT- | - |
| Traffic data volumes |  | - | - |
| Service data container |  | - | - |
| User Equipment Info |  | - | - |
| Terminal-Information |  | - | - |
| Start time |  | - | - |
| Stop time |  | - | - |
| Change Condition |  | - | - |
| Diagnostics |  | --T- | --T- |
| Enhanced Diagnostics |  | --T- | - |
| CN Operator Selection Entity |  | - | - |
| NBIFOM Support |  | I--- | - |
| NBIFOM Mode |  | I--- | - |
| CP CIoT EPS Optimisation Indicator |  | -- | - |
| UNI PDU CP Only Flag |  | IUT- | - |
| Serving PLMN Rate Control |  | IU-- | - |
| APN Rate Control |  | IU-- | - |
| MO exception data counter |  | - | - |
| 3GPP PS Data Off Status |  | IUT- | - |
| Unused Quota Timer |  | IU-- | - |

Table 6.3.3.2 illustrates the basic structure of the supported fields in the Debit / Reserve Units Response for PS online charging.

Table 6.3.3.2: Supported fields in Debit / Reserve Units Response Message

| Information Element | Node Type | P-GW | TDF |
| --- | --- | --- | --- |
|  | Supported Operation Types | I/U/T/E | I/U/T/E |
| Session Identifier |  | IUT- | IUT- |
| Operation Result |  | IUT- | IUT- |
| Originator Host |  | IUT- | IUT- |
| Originator Domain |  | IUT- | IUT- |
| Operation Identifier |  | IUT- | IUT- |
| Operation Type |  | IUT- | IUT- |
| Operation Number |  | IUT- | IUT- |
| Operation Failover |  | IUT- | IUT- |
| Multiple Unit Operation |  | IUT- | IUT- |
| Operation Failure Action |  | IUT- | IUT- |
| Redirection Host |  | IUT- | IUT- |
| Redirection Host Usage |  | IUT- | IUT- |
| Redirection Cache Time |  | IUT- | IUT- |
| Route Information |  | IUT- | IUT- |
| Failed parameter |  | IUT- | IUT- |
| Service Information |  | IUT- | IUT- |
| Service Information with PS Information |  |  |  |
| Supported Features |  | I--- | I--- |
| Furnish Charging Information |  | IUT- | IUT- |
| Offline Charging |  | I---- | I---- |
| Presence Reporting Area Information |  | IU-- | - |
| Unused Quota Timer |  | IU-- | - |

End of changes