3GPP TSG SA WG5 Meeting 137-e		S5-213317

# E-meeting, online, 10 - 19 May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.310 | CR | 0014 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-16 CR TS28.310 Update on energy saving management services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | EE_5G |  |  |  |  |  | Date: |  |  | 2021-04-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - In MnS component type A the notification of “notifyMOIChanges” is missing; - In Table 6.2.3.1.3.2-1 the reference of 5.1.2 of TS 28.552 for split gNB is not correct. Clause 5.1.2 is only valid for non-split gNB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add “notifyMOIChanges” to the notification of MnS component type A; - In Table 6.2.3.1.3.2-1 the reference of 5.1.2 of TS 28.552 for split gNB is wrong. Clause 5.1.2 is only valid for non-split gNB. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | “notifyMOIChanges” will be missing and the reference for split gNB is not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.1.1 6.2.3.1.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First modified section

6.2.3.1	Management services

6.2.3.1.1	MnS component type A

| MnS Component Type A | Note |
| --- | --- |
| Operations defined in clause 11.1.1 of TS 28.532 [16]: - createMOI - getMOIAttributes - modifyMOIAttributes - deleteMOI | Supported by the Provisioning MnS for NF, as defined in TS 28.531 [6]. |
| Notifications defined in clause 11.1.1 of TS 28.532 [16]: - notifyMOICreation - notifyMOIAttributeValueChanges - notifyMOIDeletion - notifyMOIChanges | Supported by the Provisioning MnS for NF, as defined in TS 28.531 [6]. |

End of first modified section

Second modified section

6.2.3.1.3.2	Performance measurements

Performance measurements related to D-SON (Distributed SON or Domain-Centralized) ES are captured in Table 6.2.3.1.3.2-1:

Table 6.2.3.1.3.2-1.  Energy saving management related performance measurements

| Performance measurements | Description | Related targets |
| --- | --- | --- |
| DRB.PdcpSduVolumeDL_Filter | Data Volume (amount of PDCP SDU bits) in the downlink delivered to PDCP layer – see clause 5.1.2.1.1.1 of TS 28.552 [15], per configured PLMN ID and per QoS level (mapped 5QI) and per S-NSSAI. In case of non-split gNBs. | NG-RAN data Energy Efficiency |
| DRB.PdcpSduVolumeUL_Filter | Data Volume (amount of PDCP SDU bits) in the uplink delivered from PDCP layer to higher layers – see clause 5.1.2.1.2.1 of TS 28.552 [15], per configured PLMN ID and per QoS level (mapped 5QI) and per S-NSSAI. In case of non-split gNBs. | NG-RAN data Energy Efficiency |
| DL Cell PDCP SDU Data Volume on X2 Interface | Data Volume (amount of PDCP SDU bits) in the downlink delivered on X2 interface in DC-scenarios – see clause 5.1.2.1.1.2 of TS 28.552 [15], per PLMN ID and per QoS level (mapped 5QI or QCI in NR option 3). In case of non-split gNBs. | NG-RAN data Energy Efficiency |
| DL Cell PDCP SDU Data Volume on Xn Interface | Data Volume (amount of PDCP SDU bits) in the downlink delivered on Xn interface in DC-scenarios scenarios – see clause 5.1.2.1.1.3 of TS 28.552 [15], per PLMN ID and per QoS level (mapped 5QI) and per S-NSSAI. In case of non-split gNBs. | NG-RAN data Energy Efficiency |
| UL Cell PDCP SDU Data Volume on X2 Interface | Data Volume (amount of PDCP SDU bits) in the uplink delivered on X2 interface in NSA scenarios – see clause 5.1.2.1.2.2 of TS 28.552 [15], per PLMN ID and per QoS level (mapped 5QI or QCI in NR option 3). In case of non-split gNBs. | NG-RAN data Energy Efficiency |
| UL Cell PDCP SDU Data Volume on Xn Interface | Data Volume (amount of PDCP SDU bits) in the uplink delivered on Xn interface in SA scenarios – see clause 5.1.2.1.2.3 of TS 28.552 [15], per PLMN ID and per QoS level (mapped 5QI) and per S-NSSAI. In case of non-split gNBs. | NG-RAN data Energy Efficiency |
| PNF Energy consumption | Energy consumed – see clause 5.1.1.19.3 of TS 28.552 [15] | NG-RAN data Energy Efficiency |

End of second modified section