3GPP TSG-SA5 Meeting #136e 	S5-213320

# e-meeting 10 - 19 May 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.313 | CR | 0022 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of PCI configuration for C-SON |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | SON_5G |  |  |  |  |  | Date: |  |  | 22/04/2021 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the current specification TS28.313 C-SON PCI configuration for Domain Centralized SON is failed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To make it clear that: C-SON PCI configuration of PCI list is for Domain-Centralized SON |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The definition of the C-SON PCI configuration is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.1.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of the first change

7.2	Management services for C-SON

7.2.1	PCI configuration

7.2.1.1	MnS component type A

Table 7.2.1.1-1: PCI type A

| MnS Component Type A | Note |
| --- | --- |
| Operations and notifications defined in clause 11.1.1 of TS 28.532 [3]: - createMOI operation - getMOIAttributes operation --- modifyMOIAttributes operation - - deleteMOI operation - - notifyMOIAttributeValueChanges operation - notifyMOICreation - notifyMOIDeletion - notifyMOIChanges | It is supported by Provisioning MnS for NF, as defined in 28.531 [11]. |
| Operations defined in clause 11.3.1.1.1 in TS 28.532 [3] and clause 6.2.3 of TS 28.550 [12]: - notifyFileReady operation - reportStreamData operation | It is supported by Performance Assurance MnS for NFs, as defined in 28.550 [12]. |

7.2.1.2	MnS Component Type B definition

7.2.1.2.1	Control information

The parameter is used to control the C-SON PCI configuration function.

Table 7.2.1.2.1-1: PCI control

| Control parameter | Definition | Legal Values |
| --- | --- | --- |
| PCI configuration control | This attribute allows authorized consumer to enable/disable the C-SON PCI configuration functionality. | disable, enable |

7.2.1.2.2	Parameters to be updated

The table below lists the parameter related to the C-SON PCI configuration function.

Table 7.2.1.2.2-1: PCI dpdate

| Updated parameters | Definition | Legal Values |
| --- | --- | --- |
| NR PCI | This parameter contains the PCI of the NR cell used by Cross Domain-Centralized SON. | Integer |
| PCI list | The list of PCI values to be used by Domain-Centralized SON PCI configuration function to assign the PCI for NR cells. (See attribute pciList in TS 28.541 [13]). | List of integers |

7.2.1.3	MnS Component Type C definition

7.2.1.3.1	Notifications information

The table below lists the notifications related to PCI configuration are generated from the NR cells.

Table 7.2.1.3.1-1: PCI notification

| Notification information | Description | Note |
| --- | --- | --- |
| PCI collision notification | The collision notification is used to indicate two neighbouring cells are using the same PCIs. |  |
| PCI Confusion notification | The confusion notification is used to indicate that a serving cell has 2 neighbouring cells that are using the same PCI value. |  |

7.2.1.3.2	Performance measurements

Performance measurements related to the PCI configuration are collected from the NR cells.

Table 7.2.1.3.2-1. PCI related performance measurements

| Performance measurements | Description | Note |
| --- | --- | --- |
| PCI of candidate cells | The measurement contains cumulative counter with subcounters that is identified by the PCI value(s) of the candidate cells, and is derived from MeasResultListNR (see clause 6.3.2 in TS 38.331 [9]) where it contains PCI in PhysCellId, and RSRP/RSRQ in MeasQuantityResults of candidate cells. It is generated when the RSRP received from the candidate cells exceeds certain thresholds. |  |

End of the first change