- 3GPP TSG SA WG5 Meeting #138-e	S5-214065r1

- August 23 – 31, 2021, e-Meeting	revision of S5-21abcd

- Source:	Ericsson

- Title:	pCR 28.814 Add potential solutions for 5GS enhancements for Edge Computing

- Document for:	Approval

Agenda Item:	6.5.2

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

[1] 	TR 28.814 “Study on enhancements of edge computing management”

[2] 	3GPP TS 23.548: "5G System Enhancements for Edge Computing; Stage 2"

# 3	Rationale

This contribution proposes add potential solution for TR 28.814 [1] that is to support the for 5GS enhancements for Edge Computing [2].

# 4	Detailed proposal

1st modified section

# 7	Potential solutions

Editor’s note: this clause is to describe the potential solutions to support orchestration and management of edge computing.

## 7.x		EASDF lifecycle management using provisioning MnS

### 7.x.1	General

To support EASDF deployment via provisioning MnS, the following IOC should be defined:

-  EASDFFunction IOC that contains the following, but not limited to, attributes:

-  EASDFFunction IOC ID: Naming attribute inherited from ManagedFunction.

-  EASDF Traffic function ID: the unique identity of EASDF Instance.

-  EASDF profile information elements : for EASDF Discovery and Selection (see TS.23.548[X]), the detail content of nfprofile is defined in TS 29.510.

### 7.x.2	EASDF instantiation

The procedure of NF instance creation (see clause 7.10 in TS28.531 [5]) can be reused for EASDF instantiation.

### 7.x.3	EASDF termination

The procedure of NF instance deletion (see clause 7.12 in TS28.531 [5]) can be reused for EASDF termination.

End of modified section