3GPP TSG-SA5 Meeting #138-e	S5-214136

- August 23 – 31, 2021, e-Meeting								       		 							s5-2abcde

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.313 | CR | 0026 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add RRM related measurements info |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSON_5G |  |  |  |  |  | Date: |  |  | 2021-08-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Some RRM related measurments are missing references to TS 28.552. Only mean measurements are included to monitor “PDU Sessions requested to setup” and “Peak number of PDU Sessions requested to setup”, It is missing the peak measurements. The number of Active UEs measurments are also needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the missing references to TS 28.552, and the following measurements: - peak measurements for “PDU Sessions requested to setup, - Peak number of PDU Sessions requested to setup, - Mean number of Active UEs in the DL per cell, - Maximum number of Active UEs in the DL per cell, - Mean number of Active UEs in the UL per cell, - Maximum number of Active UEs in the UL per cell |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The RRM related measurements are not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.2.2, 7.2.2.3.1, 8.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

#### 6.4.2.2	RRM resources optimization for network slice instance(s)

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To enable the optimization of the RRM resources allocated among network slice instance(s) to ensure the RRM resources are efficiently used, while providing quality end-user experience and performance. |  |
| Actors and Roles | C-SON function to support RRM resource optimization for network slice instance(s). |  |
| Telecom resources | - gNB; - The producer of provisioning MnS |  |
| Assumptions | N/A |  |
| Pre-conditions | - 5G NR cells are in operation. - An AI/ML model has been created based on the RRM related performance measurements received previously. |  |
| Begins when | The C-SON function has been enabled. |  |
| Step 1 (M) | The C-SON function collects RRM related measurements on a per network slice instance basis (e.g. mean DL/UL PRB used for data traffic, mean number of DRBs successfully setup, and mean number of PDU Sessions successfully setup, … etc.), by consuming the MnS of performance assurance. |  |
| Step 2 (M) | The C-SON function analyzes the measurements to train the AI/ML model and determines the actions to optimize the RRM resources for network slice instance(s) that include consuming the MnS of provisioning to update the appropriate RRM policies. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The RRM resources for network slice instance(s) have been optimized. |  |
| Traceability | REQ-RRM-FUN-1, REQ-RRM-FUN-2 |  |

Next Modified Section

#### 7.2.2.3	MnS Component Type C definition

##### 7.2.2.3.1	Performance measurements

RRM related performance measurements are captured in Table 7.2.2.3.1-1:

Table 7.2.2.3.1-1. RRM related performance measurements

| Performance measurements | Description | Note |
| --- | --- | --- |
| Mean DL PRB used for data traffic | Provides the mean number of PRBs used in downlink for data traffic in the NRCellDU, with subcounters per sNSSAI (see clause 5.1.1.2.5 in TS 28.552 [5]). |  |
| Peak DL PRB used for data traffic | Provides the peak number of PRBs used in downlink for data traffic in the NRCellDU, with subcounters per sNSSAI |  |
| Mean UL PRB used for data traffic | Provides the mean number of PRBs used in uplink for data traffic in the NRCellDU, with subcounters per sNSSAI (see clause 5.1.1.2.7 in TS 28.552 [5]). |  |
| Peak UL PRB used for data traffic | Provides the peak number of PRBs used in uplink for data traffic in the NRCellDU, with subcounters per sNSSAI |  |
| Average DL UE throughput in gNB | Provides the average UE throughput in downlink in the NRCellDU, with subcounters per sNSSAI (see clause 5.1.1.3.1 in TS 28.552 [5]). |  |
| Distribution of DL UE throughput in gNB | Provides the distribution of the UE throughput in downlink in the NRCellDU, with subcounters per sNSSAI (see clause 5.1.1.3.2 in TS 28.552 [5]). |  |
| Average UL UE throughput in gNB | Provides the average UE throughput in uplink in the NRCellDU, with subcounters per sNSSAI (see clause 5.1.1.3.3 in TS 28.552 [5]). |  |
| Distribution of UL UE throughput in gNB | Provides the average UE throughput in uplink in the NRCellDU, with subcounters per sNSSAI (see clause 5.1.1.3.4 in TS 28.552 [5]). |  |
| Mean number of Active UEs in the DL per cell | Provides the mean number of active UEs in downlink in an NRCellDU, with subcounters per sNSSAI (see clause 5.1.1.23.1 in TS 28.552 [5]). |  |
| Maximum number of Active UEs in the DL per cell | Provides the maximum number of active UEs in downlink in an NRCellDU, with subcounters per sNSSAI (see clause 5.1.1.23.2 in TS 28.552 [5]). |  |
| Mean number of Active UEs in the UL per cell | Provides the mean number of active UEs in uplink in an NRCellDU, with subcounters per sNSSAI (see clause 5.1.1.23.3 in TS 28.552 [5]). |  |
| Maximum number of Active UEs in the UL per cell | Provides the maximum number of active UEs in uplink in an NRCellDU, with subcounters per sNSSAI (see clause 5.1.1.23.4 in TS 28.552 [5]). |  |
| Mean number of DRBs successfully setup | Provides the mean number of DRBs successfully setup in the PDU sessions in the NRCellCU, with subcounters per sNSSAI |  |
| Peak number of DRBs successfully setup | Provides the peak number of DRBs successfully setup in the PDU sessions in the NRCellCU, with subcounters per sNSSAI |  |
| Mean number of PDU Sessions requested to setup | Provides the mean number of PDU Sessions requested to setup in the NRCellCU, with subcounters per sNSSAI |  |
| Mean number of PDU Sessions successfully setup | Provides the mean number of PDU Sessions successfully setup in the NRCellCU, with subcounters per sNSSAI |  |

Next Modified Section

### 8.3.3	RRM resources optimization for network slice instance(s)

Figure 8.3.3-1 depicts an example of network slice instances that are created to support various services, such as URLLC, eMBB, or mMTC with different RRM resources requirements, where network slice instances sNSSAI #2-1 and sNSSAI #2-2 support a kind of RRM requirements, while network slice instances sNSSAI #1 supports different RRM requirements. It shows that the RRM resources provided by network functions of DU, CUUP, and CUCP are characterized by RRMPolicyRatio IOC with rRMPolicyMaxRatio, rRMPolicyMinRatio, and rRMPolicyDedicatedRatio attributes to define the shared resources, prioritized resources, and dedicated resources for network slice instance(s) (see clause 4.3.36.1 in TS 28.541 [13]).

End of Modified Sections