- 23GPP TSG-SA5 Meeting #138e	S5-214142rev2

- 23 August to 31 August 2021, E-meeting

- Source:	Intel, NEC, China Telecom

- Title:	pCR 28.104 Add common information elements of analytics outputs

- Document for:	Approval

- Agenda Item:	6.4.18

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1]     S5-214143 add structure of TS 28.104 Management Data Analytics (MDA).

# 3	Rationale

There are some information elements that are common for all analytics outputs, i.e., these common information elements form a subset of all analytic outputs.

It would be more organized and clearer if the common information elements can be defined commonly in a central place.

This pCR is to add common information of analytics outputs to a dedicated clause in draft TS 28.104 aligning with the structure in S5-214143 [1].

# 4	Detailed proposal

1st Modified Section

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[h]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[i]	3GPP TS 28.658: "Telecommunication management; Evolved Universal Terrestrial Radio Access Network (E-UTRAN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[j]	3GPP TS 28.662: "Telecommunication management; Generic Radio Access Network (RAN) Network Resource Model (NRM); Information Service (IS)".

[k]	3GPP TS 32.300: "Telecommunication management; Configuration Management (CM); Name convention for Managed Objects".

[l]	3GPP TS 32.156: "Telecommunication management; Fixed Mobile Convergence (FMC) model repertoire".

Next Modified Section

## 8.3	Common information elements of analytics outputs

There are some information elements that are common for all analytics outputs, i.e., these common information elements form a subset of all analytics outputs.

### 8.3.1	Common information element definitions

The common information elements of the analytics outputsare defined in table 8.3.1-1.

Table 7.3-1: Common information elements of analytics outputs

| Information element | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| MDAType | It indicates the type of MDA capability for analysis of the reported issue(s) (Note 1). The allowed values are the MDA type names defined for each MDA capability respectively, in format of “CategoryName.CapabilityName”. | M | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| AnalyticsId | The identifier of the analytics described in the MDA report. | M | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| RelatedMOIs | The MOIs that are: - affected affected by the ongoing issue described in the MDA report, - potentially affected affected by the potential issue described in the MDA report, or - relevant to the prediction described in the MDA report. | M | type: DN multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| RecommendedActions | The recommended actions pertaining to the analytics outputs described in the MDA report. | O | type: FFS multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

8.3.2	Information element constraints

| Name | Definition |
| --- | --- |

End of Modified Sections