- 3GPP TSG-SA5 Meeting #138e	S5-214146rev2

- 23 August to 31 August 2021, E-meeting

- Source:	Intel

- Title:	pCR 28.104 Add ML support for MDA

- Document for:	Approval

- Agenda Item:	6.4.18

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1]	TS 28.104 “Management and orchestration; Management Data Analytics”

# 3	Rationale

MDA may be supported by ML capabilities.

MDAS producer may or may not disclose the use of ML for MDA. When MDAS producer disclose the use of ML or MDA, the ML model training needs to be supported.

This pCR is to add specific aspects on ML support for MDA in draft TS 28.104 [1].

# 4	Detailed proposal

1st Modified Section

# 7.X	Machine Learning (ML) for MDA

### 7.X.1	Description

#### 7.X.1.1	MDA with ML support

MDA may be supported by ML capabilities.

When MDA is supported by ML capabilities, one deployment scenario of MDA MnS as illustrated in Figure 7.X.1.1-1.

Figure 7.X.1.1-1: MDA with ML support

1. 	The Management Function (MDA MnS producer) collects the analytics inputs for MDA.

2.	The MDA MnS consumer requests the MDA reports and controls the MDA reporting from the MDA MnS producer.

3.	The Management Function (MDA MnS producer) processes the analytics inputs for MDA based on the inference of the (trained) ML model, and generates analytics/inference output.

NOTE: How the Management Function (MDA MnS producer) interacts with and use the ML model for inference is out of scope of the present specification.

4.	The Management Function (MDA MnS producer) generates the MDA report(s) based on the analytics/inference output, and provides the MDA report(s) to the MDA MnS consumer.

7.X.1.2	ML model training

When ML is used for MDA, the ML model needs to be trained in order to provide inference output that can meet consumer’s expectations.

The ML model training capabilities are provided by a ML Model Training (MLMT) MnS producer to one or more consumer(s). The interactions for MLMT MnS is illustrated in Figure X.2-1.

Figure X.1.2-1: MLMT MnS interactions

1.	The MLMT MnS consumer requests the MLMT MnS producer to train the ML model with the specified data source(s) for training.

2.	The MLMT MnS producer provides a response to the consumer indicating whether the request was accepted.

3.	If the request was accepted, the MLMT MnS producer trains the ML model based the request, and provides the training result and information about the trained ML model to the MLMT MnS consumer.

7.X.1.3	Relation between ML and MDA

This clause provides the relation between ML and MDA for the case that the MDA is supported by ML capabilities.

Figure7. X.1.3-1: Relation between ML and MDA

The MLMT MnS producer trains the ML model for MDA per the request from the MLMT MnS consumer, as described in clause 7.X.1.2.

The MDA MnS producer uses the trained ML model to conduct ML inference for MDA, as described in clause 7.X.1.1.

The MDAS producer may also act as consumer of MLMT MnS to request training the ML model (as described in clause 7.X.1.2).

The MLMT MnS producer may also expose the MLMT MnS to another consumer (other than the MDAS producer, e.g., the same entity of MDAS consumer), and allow that consumer to request training the ML model.

The MDA MnS producer and MLMT MnS producer could be located in the same Management Function or different Management Functions.

The MDA MnS consumer and MLMT MnS consumer could be the same entity or different entities.

### 7.X.2	Use cases

#### 7.X.2.1	ML model training for MDA

When ML is used for MDA, the ML model needs to be trained in order to provide the inference output that can meet consumer’s expectations.

The ML model training capabilities are provided by an MLMT MnS producer to one or more consumer(s), as illustrated in clause X.2.

The MLMT MnS consumer requests the MLMT MnS producer to train the ML model with the specified data source(s) for training.

The MLMT MnS producer) trains the ML model based the request from the MLMT MnS consumer, and provides the training result and information of the trained ML model to the MLMT MnS consumer.

### 7.X.3	Requirements for ML model training for MDA

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-MDA_ML-FUN-1 | The MLMT MnS producer shall have a capability allowing the consumer to request ML model training. | ML model training for MDA |
| REQ-MDA_ML-FUN-2 | The MLMT MnS producer shall have a capability allowing the consumer to specify the expected data sources for ML model training. | ML model training for MDA |
| REQ-MDA_ML-FUN-3 | The MLMT MnS producer shall have a capability to provide the trained model and indication whether the training was successful to the consumer. | ML model training for MDA |

Next Modified Sections

# 7.Y	Feedback for MDA

### 7.Y.1	Description

#### 7.Y.1.1	MDA with feedback support

The MDA report may be evaluated in terms of the quality and accuracy, and the feedback of the evaluation may be used to optimize the MDA algorithm.

Figure 7.Y.1.1-1: Deployment example of MDA MnS and Feedback MnS

In this deployment scenario, where one Management Function acts as MDA MnS producer and Feedback MnS consumer, and the feedback for MDA is supported by the following steps:

1. 	The Management Function (Feedback MnS consumer) requests and controls the feedback on MDA reports from Feedback MnS producer.

2.	The Feedback MnS producer evaluates the MDA reports and provides the feedbacks to the Management Function (Feedback MnS consumer).

3.	The Management Function (MDA MnS producer) may use the received feedbacks to optimize the MDA algorithm.

#### 7.Y.1.2	ML model training with feedback support

The trained ML model may be evaluated, and feedback of the evaluation may be used for further training the ML model.

The ML model training may be triggered by the feedback on a trained ML model, one deployment scenario of MLMT MnS and Feedback MnS (see clause X) is illustrated in Figure 7.Y.1.2-1.

Figure 7.Y.1.2-1: Deployment example of ML Model Training (MLMT) MnS and Feedback MnS

In this deployment scenario, where one Management Function acts as MLMT MnS producer and Feedback MnS consumer, the ML model training may be trigged by the feedback on a trained ML model by the following steps:

1. 	The Management Function (Feedback MnS consumer) requests and controls the feedback on a ML model from Feedback MnS producer.

2.	The Feedback MnS producer evaluates the ML model and provides the feedbacks to the Management Function (Feedback MnS consumer).

3.	The Management Function (MLMT MnS producer) may use the received feedbacks to train the ML model.

7.Y.2	Use cases

#### 7.Y.2.1	Feedback for MDA and ML

The delivered result of an MnS, for instance the MDA report provided by an MDA MnS producer, or the trained ML model for MDA provided by an MLMT MnS producer, may be validated and evaluated, and a feedback may be provided as the result of the validation and evaluation.

The feedback is provided by a Feedback MnS producer to one or more consumer(s). The interactions for Feedback MnS is illustrated in Figure 7.Y.2.1-1.

Figure 7.Y.2.1-1: Feedback MnS interactions

1.	The Feedback MnS consumer requests the Feedback MnS producer to provide feedback on a delivered result of an MnS.

2.	The Feedback MnS producer provides a response to the consumer indicating whether the request was accepted.

3.	If the request was accepted, the Feedback MnS producer evaluates the delivered result of the specified MnS, and provide the feedback to the consumer according to the request.

### 7.Y.3	Requirements

MDA_FUN2heback n oderhave a aaty aing  cosm requstrained ML model by an MLnd control the fedreporng

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-FB-FUN-1 | The Feedback MnS producer shall have a capability allowing the consumer to request feedback on a delivered result of a specified MnS and control the feedback reporting. | Feedback for MDA and ML |
| REQ-FB-FUN-2 | The Feedback MnS producer shall have a capability to evaluate an delivered result of the specified MnS, and provide the feedback to the consumer. | Feedback for MDA and ML |
| REQ-MDA_FB-FUN-1 | The Feedback MnS producer shall have a capability allowing the consumer to request feedback on the MDA reports provided by an MDA MnS producer and control the feedback reporting. | Feedback for MDA and ML |
| REQ-ML_FB-FUN-1 | The Feedback MnS producer shall have a capability allowing the consumer to request feedback on a trained ML model provided by an MLMT MnS producer and control the feedback reporting. | Feedback for MDA and ML |

End of Modified Sections