- 3GPP TSG-SA5 Meeting #138e	S5-214168rev3

- 23 Aug to 31 Aug 2021, E-meeting

- Source:	China Telecom

- Title:	Add MDA capability for MDA assisted energy saving

- Document for:	Approval

- Agenda Item:	6.4.18

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1] 	3GPP TS 28.104-000 “Management and orchestration; Management Data Analytics”

[a]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[b]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[c]	3GPP TS 28.406: "Telecommunication management; Quality of Experience (QoE) measurement collection; Information definition and transport".

[d]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[e]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[f]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[g]	3GPP TS 28.532: "Management and orchestration; Generic management services".

# 3	Rationale

This pCR is to add the MDA capability for MDA assisted energy saving analysis for TS 28.104 based on the initial version (v0.0.0) [1].

# 4	Detailed proposal

Start of modifications (all new text)

## X	MDA assisted energy saving

### X.1	Energy saving issue analysis

#### X.1.1	MDA type

The MDA type for energy saving issue alaysis is: EnergySavingAnalysis.EnergySavingIssue.

#### X.1.2	Analytics inputs

The analytics inputs that can be used for energy saving issue analysis are provided in table X.1.2-1.

Table X.1.2-1: Analytics inputs for energy saving issue analysis

| Data category | Description | References |
| --- | --- | --- |
| Performance Measurements | PNF Power Consumption: Power consumed over the measurement period; | 5.1.1.19.2 of TS 28.552 [a] |
|  | PNF Energy consumption: The energy consumed; | 5.1.1.19.3 of TS 28.552 [a] |
|  | PNF Temperature: The temperature over the measurement period; | 5.1.1.19.4 of TS 28.552 [a] |
|  | PNF Voltage: The voltage; | 5.1.1.19.5 of TS 28.552 [a] |
|  | PNF Current: The current; | 5.1.1.19.6 of TS 28.552 [a] |
|  | PNF humidity: The percentage of humidity during the measurement period; | 5.1.1.19.7 of TS 28.552 [a] |
|  | PDCP Data Volume of NR cells: The transmitted PDCP data volume; | 5.1.2.1 and 5.1.3.6 of TS 28.552 [a] |
|  | Virtual resource usage of NF: The resource usage of virtual network functions; | 5.7.1 of TS 28.552 [a] |
|  | Data volume of UPF; | 5.4 of TS 28.552 [a] |
|  | Delay related measurements of UPF; | 5.4 of TS 28.552 [a] |
|  | Number of PDU sessions with SSC mode 1 on UPF. | TS 28.552 [a] |
|  | Traffic load variation: PRB utilization rate, RRC connection number, etc | 5.1.1.2 and 5.1.1.4 of TS 28.552 [a] |
|  | Network data: UE throughput, the mobility management data (e.g., rate of successful handover executions), packet delay data (e.g., air-interface delay), etc. | 5.1.1.1, 5.1.1.3 and 5.1.1.6 of TS 28.552 [a] |
| QoE Data | The measurements that are collected are DASH and MTSI measurements. | TS 28.406 [c], TS 26.247 [d], TS 26.114 [e] |
| NRM | MOIs of the cells, UPFs and SMFs. | TS 28.541 [f] |
| Network analytics data | The control plane analysis result from the NWDAF, e.g., observed service experience related network data analytics. | TS 23.288 [b] |
| ML model information | The information of the ML model, which may include the ML model file address. It can be used by MDA to derive predictions of the energy saving related information (e.g., base station energy-saving scenarios, traffic load). . | Editor’s note: to be defined in MDA capability specific procedures in TS 28.104 |

#### X.1.3	Analytics output

The specific information elements of the analytics output (MDA report) for energy saving issue analysis, in addition to the common information elements of the MDA reports, are provided in table X.1.3-1.

Table X.1.3-1: Analytics output for energy saving issue analysis

| Information element | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| EnergySavingIssueArea | The indication of NR cells, or the NFs (e.g., UPFs) instance where the unreasonable energy consumption exists; | M | FFS |
| Root cause | The root cause of the part of the energy consumption that may be conserved. The allowed value is one of the enumerated values: Ultra-lowTrafficLoadAreaEnergyConsumption, ExcessiveEnergyConsumption, Other, Unknown. | O | FFS |
| Trends of the traffic load | Predictions related to the trends of traffic load (e.g., PRB utilization rate, RRC connection number, etc.) which could be used as references for making decision on energy saving | M | FFS |
| Current Energy Saving State of the cells | The information of the current energy saving state of the cells. The allowed value is one of the enumerated values: notEnergySavingState, EnergySavingState. | O | type: enumeration multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| Recommended ML model information | Describes the information of the optimal ML models which can be used to provide more accurate predictions of the energy saving related information to assist the energy saving decision making. The ML model information may include the ML model file address. | O | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| Energy saving recommendations | For ES on NR cells. It may contain a set of - recommended NR Cell (ES-Cell) to enter energySaving state. - recommended candidate cells with precedence for taking over the traffic of the ES-Cell. - a time period, during which energy saving is or not allowed. - predictions related to the trends of traffic load which could be used as references for making decision on energy saving of NR cells. For ES on UPFs. It contains a set of - recommended UPF (ES-UPF) to conduct energy saving; - recommended candidate UPFs with precedence for taking over the traffic of the ES-UPF. - a time period, during which energy saving is or not allowed. - predictions related to the trends of traffic load which could be used as references for making decision on energy saving of UPFs. | FFS | type: FFS multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

| Name | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |

End of modifications