3GPP TSG-SA5 Meeting #138-e 	S5-214194

- e-meeting, 23 - 31 August 2021

- Source:	Huawei

- Title:	Discussion on the observations of the issues caused by the updated NetworkSliceSubnet inheritence relationship

- Document for:	Discussion and Endorsement

Agenda Item:	6.3

# 1	Decision/action requested

It proposes to discuss and endorse

# 2	References

[1]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[2]	3GPP TS 28.622: "Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

# 3	Rationale

In 3GPP SA5#137emeeting, the agreed CR S5-213468 and S5-213429 introduced to change the NetworkSliceSubnet inherit from the TOP (Before change, it inherited from SubNetwork).

Following are the observations for the issues needs to be addressed after the inheritance relationship updated.

Observation#1: The inherited attribute “priorityLabel” is missing for the NetworkSliceSubnet to satisfy the use case Network slice subnet management with assigned priority in clause 5.1.23 in TS 28.531.

Before the inheritance relationship update, NetworkSliceSubnet IOC inherit the attribute “priorityLabel” in SubNetwork IOC, now the NetworkSliceSubnet IOC is changed to inherit from Top IOC so the attribute “priorityLabel” for NetworkSliceSubnet IOC is missing.

It proposes to add attribute “priorityLabel” in NetworkSliceSubnet IOC in TS 28.541[1].

Observation#2: Several control capabilities (e.g. PM control, threshould monitoring control) for network slice subnet are missing.

Before the inheritance relationship update, NetworkSliceSubnet IOC inherit from SubNetwork IOC acting as the role for ManagedEnetity in several Control NRM fragement (PM control NRM fragment), which means NetworkSliceSubnet IOC can contain several control IOCs (e.g. PerfMeticJob IOC, ThresholdMonitor IOC) to support corresponding control capability for the network slice subnet. Now the NetworkSliceSubnet IOC is changed to inherit from Top IOC instead of SubNetwork, which will lead the NetworkSliceSubnbet can not contain these control IOCs. Following are some analysis on the necessary for usage of these control IOCs for NetworkSliceSubnet.

The NtfSubscriptionCtrl IOC name contained by NetworkSliceSubnet can be used to subscribe the notifications (e.g. notifyMOIAttributeValueChanges) for NetworkSliceSubnet IOC.

The PerfMeticJob IOC name contained by NetworkSliceSubnet can be used to present a performance metric production job for KPI (e.g. downstream throughput for Single Network Slice Instance) associated to NetworkSliceSubnet IOC.

The ThresholdMonitor IOC name contained by NetworkSliceSubnet can be used to present s a threshold monitor for KPI associated to NetworkSliceSubnet IOC.

It proposes to add NetworkSliceSubnet IOC as one possibility for ManagedEntity for several control NRM fragements (including PM control NRM fragment, Threshold monitoring control NRM fragment, Notification subscription and heartbeat notification control NRM fragment) in TS 28.622.

# 4	Detailed proposal

Proposal#1: It proposes to add attribute “priorityLabel” in NetworkSliceSubnet IOC in TS 28.541[1].

Proposal#2:  It proposes to add NetworkSliceSubnet IOC as one possibility for ManagedEntity for several control NRM fragements (including PM control NRM fragment, Threshold monitoring control NRM fragment, Notification subscription and heartbeat notification control NRM fragment).