3GPP TSG-SA5 Meeting #138-e 	S5-214210

- e-meeting, 23 - 31 August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.536 | CR | 0030 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add concrete closed loop policy configuration for radio coverage optimization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | S5 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei |  |  |  |  |  |  |  |  |  |
| Work item code: | eCOSLA |  |  |  |  |  | Date: |  |  | 2021-08-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To support the use case for radio network coverage optimization provided in TS 28.535, the coverage optimization policy is added here. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update stage2 information model definition for AssuranceClosedControlLoop to add coverage optimization policy. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.2.2.1, 4.1.2.2.2, 4.1.2.3.X(new),4.1.2.3.Y(new),4.1.2.3.Z(new),4.1.2.3.A(new), 4.1.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR … CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

#### 4.1.2.2.1	Relationships

This clause depicts the set of classes that encapsulates the information relevant for this MnS. This clause provides an overview of the relationships between relevant classes in UML.

Figure 4.1.2.2.1.1: Assurance management NRM fragment

Editor’s NOTE: Whether the AssuranceGoal and AssurancePolicy can be set for one ACCL is FFS.

4.1.2.2.2	Inheritance

Figure 4.1.2.2.1.1: Assurance management inheritance relationships

3rd Change

##### 4.1.2.3.X AssurancePolicy <<IOC>>

4.1.2.3.X.1	Definition

This IOC represents the properties of an assurance policy for a closed control loop. An AssurancePolicy can name-contain multiple instances of AssurancePolicyContent (including coverageOptimizationPolicyContent which represents the policy to control the closed loop).

Editor’s NOTE: This will be revisited if more AssurancePolicyContent are introduced.

4.1.2.3.X.2	Attributes

The AssurancePolicy includes attributes inherited from Top <<IOC>> (defined in TS 28.622[5]).

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |

Editor’s NOTE: The relation of AssurancePolicy and Policy IOC in TS 28.556 needs to be considered.

4.1.2.3.X.3	Attribute constraints

NA

4.1.2.3.X.4	Notifications

The common notifications defined in sub clause 4.1.2.5 are valid for this IOC, without exceptions or additions.

##### 4.1.2.3.Y CoverageOptimizationPolicyContent <<IOC>>

4.1.2.3.Y.1	Definition

This IOC represents the attributes of radio coverage policy content. It consists of condition and the corresponding actions applied for ACCL for radio coverage optimization.

4.1.2.3.Y.2	Attributes

The CoverageOptimizationPolicyContent includes attributes inherited from Top <<IOC>> (defined in TS 28.622[5]) and the following attributes:

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| covOptPolicyContentCondition | M | T | T | F | T |
| covOptPolicyContentAction | M | T | T | F | T |

4.1.2.3.Y.3	Attribute constraints

NA

4.1.2.3.Y.4	Notifications

The common notifications defined in sub clause 4.1.2.5 are valid for this dataType, without exceptions or additions.

##### 4.1.2.3.Z CovOptPolicyContentAction <<dataType>>

4.1.2.3.Z.1	Definition

This dataType represents the parameters to be adjusted to optimize radio coverage.

4.1.2.3.Z.2	Attributes

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| downlinkTransmitPowerInfo | O | T | T | F | T |
| antennaTiltInfo | O | T | T | F | T |
| antennaAzimuthInfo | O | T | T | F | T |
| digitalTiltInfo | O | T | T | F | T |
| digitalAzimuthInfo | O | T | T | F | T |
| coverageShapeList | O | T | T | F | T |

4.1.2.3.Z.3	Attribute constraints

NA

4.1.2.3.Z.4	Notifications

The common notifications defined in subclause 4.1.2.5 are valid for this dataType, without exceptions or additions.

##### 4.1.2.3.A RangeInfo <<dataType>>

4.1.2.3.A.1	Definition

This dataType represents the adjustment range for parameters.

4.1.2.3.A.2	Attributes

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| maxValue | M | T | T | F | T |
| minValue | M | T | T | F | T |

4.1.2.3.A.3	Attribute constraints

NA

4.1.2.3.A.4	Notifications

The common notifications defined in sub clause 4.1.2.5 are valid for this dataType, without exceptions or additions.

3rd Change

#### 4.1.2.4	Attribute definitions

##### 4.1.2.4.1	Attribute properties

The following table defines the properties of attributes that are specified in the present document.

Table 4.1.2.4.1.1

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| controlLoopLifeCyclePhase | It indicates the lifecycle phase of the AssuranceClosedControlLoop instance. AllowedValues: Preparation, Commissioning, Operation and Decommissioning. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: NULL isNullable: False |
| assuranceTargetName | The name of the attribute which is part of AssuranceTarget. The assuranceTargetName shall be equal to the name of an attribute in the relevant ServiceProfile or SliceProfile. The relevant ServiceProfile or SliceProfile is identified by the attribute serviceProfileId or sliceProfileId in the AssuranceGoal. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| assuranceTargetValue | The value of the attribute which is part of AssuranceTarget | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| assuranceTargetList | This is an attribute containing a list of AssuranceTarget(s) that are part of an AssuranceGoal | type: AssuranceTarget multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| observationTime | It indicates the time duration over which an AssuranceGoal is observed. The observation time is expressed in seconds. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| assuranceGoalStatusObserved | It holds the status of the observed goal fulfilment to the assuranceGoal. The value is FULFILLED only if all the constituent assuranceTargetStatusObserved are FULFILLED. allowedValues: "FULFILLED", “NOT_FULFILLED | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| assuranceGoalStatusPredicted | It holds the status of the predicted future goal fulfilment to the assuranceGoal . The value is FULFILLED only if all the constituent assuranceTargetStatusPredicted are FULFILLED. allowedValues: "FULFILLED", “NOT_FULFILLED" | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| assuranceTargetStatusObserved | It holds the status of the observed target fulfilment to the assuranceGoal. allowedValues: "FULFILLED", “NOT_FULFILLED | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| assuranceTargetStatusPredicted | It holds the status of the predicted future target fulfilment to the assuranceGoal allowedValues: "FULFILLED", “NOT_FULFILLED" | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| networkSliceRef | It holds the reference to the NetworkSlice instance subject to assurance requirements | type: Dn multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| networkSliceSubnetRef | It holds the reference to the NetworkSliceSubnet instance subject to assurance requirements | type: Dn multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| operationalState | It indicates the operational state of the AssuranceClosedControlLoop instance. It describes whether the resource is installed and partially or fully operable (Enabled) or the resource is not installed or not operable (Disabled). Allowed values; Enabled/Disabled allowedValues: "ENABLED", "DISABLED". The meaning of these values is as defined in 3GPP TS 28.625 [14] and ITU-T X.731 [15]. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: Disabled allowedValues: Enabled, Disabled isNullable: False |
| administrativeState | It indicates the administrative state of the AssuranceClosedControlLoop instance. It describes the permission to use or the prohibition against using the AssuranceClosedControlLoop instance. The administrative state is set by the MnS consumer. Allowed values; Locked/Unlocked allowedValues: "LOCKED", "UNLOCKED". The meaning of these values is as defined in 3GPP TS 28.625 [14] and ITU-T X.731 [15]. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: Locked allowedValues: Locked, Unlocked isNullable: False |
| covOptPolicyContentCondition | It indicates the condition which will trigger the actions to optimize radio coverage. For example, allowedValues for the condition can be “weak coverage”, “pilot pollution”, “overshoot coverage”. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| covOptPolicyContentAction | It indicates the parameters which can be adjusted to optimize radio coverage. allowedValues: N/A | type: CovOptPolicyContentAction multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| downlinkTransmitPowerInfo | It indicates adjustment range (including maximum value, minimum value) of downlinkTransmitPower to optimize radio coverage. allowedValues: N/A | type: RangeInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| antennaTiltInfo | It indicates adjustment range (including maximum value, minimum value) of antennaTilt to optimize radio coverage. allowedValues: N/A | type: RangeInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| antennaAzimuthInfo | It indicates adjustment range (including maximum value, minimum value) of antennaAzimuth to optimize radio coverage. allowedValues: N/A | type: RangeInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| digitalTiltInfo | It indicates adjustment range (including maximum value, minimum value) of digitalTilt to optimize radio coverage. allowedValues: N/A For the detailed definition of digitalTilt see clause 6.3.23 in TS 28.541[6]. | type: RangeInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| digitalAzimuthInfo | It indicates adjustment range (including maximum value, minimum value) of digitalAzimuth to optimize radio coverage. allowedValues: N/A For the detailed definition of digitalAzimuth see clause 6.3.23 in TS 28.541[6]. | type: RangeInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| coverageShapeList | It indicates the coverageShape which can be selected to optimize radio coverage. allowedValues: 0 : 65535 For the detailed definition of coverageShape see clause 6.3.23 in TS 28.541[6]. | type: Integer multiplicity: 1..* isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| maxValue | It indicates the maximum value of the parameter. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| minValue | It indicates the minimum value of the parameter. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NOTE 1: Void NOTE 2: Void |  |  |

End of changes