3GPP TSG-SA5 Meeting #138-e 	S5-214214

- e-meeting, 23 - 31 August 2021

- Source:	Huawei, China Telecom, China Mobile，China Unicom

- Title:	Add concrete RadioServiceExpectation

- Document for:	Approval

Agenda Item:	6.4.10

# 1	Decision/action requested

The group is asked to discuss and approval.

# 2	References

[1]	3GPP draft TS 28.312: “Management and orchestration; Intent driven management services for mobile networks v0.5.0”.

# 3	Rationale

This contribution proposes to add a concrete IntentExpectation for radio service.

# 4	Detailed proposal

It proposes to make the following changes to TS 28.312[1].

1st Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.531: "Management and orchestration; Provisioning".

[3]	3GPP TS 28.532: " Management and orchestration; Generic management services"

[X]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

2nd Change

##### 6.2.1.2.X	RadioServiceExpectation <<IOC>>

###### 6.2.1.2.X.1	Definition

This IOC represents the properties of an IntentExpectation for radio service provisioning and optimization.

Editor’s Note: Whether the RadioNetworkExpectation <<IOC>> needs to be stripped into smaller Expectation is FFS.

###### 6.2.1.2.X.2	Attribute

Editor’s Note: Whether the RadioNetworkExpectation <<IOC>> can be inhertited from Top_ <<IOC>> is FFS.

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| expectedAreas | M | T | T | F | T |
| rANSLSTargets | M | T | T | F | T |

###### 6.2.1.2.X.3	Attribute constraints

None

3rd Change

#### 6.2.1.4	Attribute definition

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| userLabel | A user-friendly (and user assignable) name of the intent. allowedValues: Not Applicable | type: String multiplicity: 1 isOrdered: F isUnique: F defaultValue: None isNullable: False |
| intentExpectation | It indicates the expectations including requirements, goals and constraints given to a 3GPP system. The intentExpectation includes the intentAreas which describes a list of coverage areas for the intent to be applied. Editor’s Note: the detailed intentExpectation is for further discussion | type: FFS multiplicity: * isOrdered: F isUnique: F defaultValue: None isNullable: False |
| intentFulfilStatus | It describes the status of the intent fulfilment result, which is configured by MnS producer and can be read by MnS consumer. allowedValues: "FULFILLED", “NOT_FULFILLED Editor’s Note: whether other allowed values should be supported is FFS, and the name for the attribute intentFulfilStatus is FFS. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| RadioServiceExpectation.expectedAreas | It describes a list of coverage area for the expected radio service. | type: CoverageArea multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| rANSLSTargets | It describes the SLSs for the radio service to be deployed. In case MnS consumer’s intentExpectation is for a radio network slice subnet, all SLSs defined in RANSliceSubnetProfile can be used as RANSLSTargets. In case MnS consumer’s intentExpectation is for a radio service (have no knowledge of network slicing), the SLS defined in RANSliceSubnetProfile can be used as RANSLSTargets except for the following Slice Specific attributes: - sliceSimultaneousUse - resourceSharingLevel For the detailed definition of RANSliceSubnetProfile see clause 6.3.23 in TS 28.541. | type: RANSliceSubnetProfile multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

End of changes