3GPP TSG-SA5 Meeting #138e 	S5-214243

- Electronic meeting, online, 23 - 31 August 2021

- Source:	Huawei

- Title:	pCR TR 28.813 Key Issue #7: Solution for area based energy saving

- Document for:	Approval

Agenda Item:	6.5.1

# 1	Decision/action requested

This contribution is proposed to be approved.

# 2	References

[1]	3GPP TS 28.813: "Study on new aspects of Energy Efficiency (EE) for 5G"

# 3	Rationale

In TR28.813 Key Issue #7: Area based energy saving studies whether and how to support an area based ES which means there are multiple capacity booster cells in the geographical area to consider ES as a whole. In this contribution an example for area based energy saving is introduced.

# 4	Detailed proposal

It is proposed to apply the following changes as below in TR 28.813 [1].

Start of the change

- 4.7	Key Issue #7: Area based energy saving

- 4.7.1	Description

- 4.7.2	Potential solutions

- 4.7.2.1	Potential solution #Y-1: Centralized energy saving for RAN domain area

- 4.7.2.2	Potential solution #2: An example solution for area based energy saving

- 4.7.2.2.1	Introduction

The potential solution focused on the example for area based energy saving to improve the energy efficiency of the network.

- 4.7.2.2.2	Description

Because of the complexity of the environments and the conditions of the wireless network, the energy efficiency of the different networks may behave differently. The traffic model, user distribution, service type and the climate zones may lead to the variation of the energy efficiency values. For each scenario the energy efficiency KPI value may be interpreted differently. Therefore, the reference energy efficiency value for different scenarios or typical scenarios need to be defined. The energy efficiency of the same or similar scenarios can be compared and the network with lower energy efficiency is intent to be improved with some policies in order to decrease power consumption and lower the operators’ operational expense.

One of the effective way is to define energy efficiency management groups according to areas. For a certain area the communication conditions such as temperature, humidity are similar. According to the network charictoristics such as the traffic model, user distribution, service type of the cells, the cells in the area are divided into several energy efficiency management groups. One group includes several capacity booster cells and candidate cells. In a certain energy efficiency management group the cells are considered to have the same temperature, same humidity, with the similar traffic model, user distribution and service type. So the EE KPI values of the cells in the energy efficiency management group is considered to be comparable.

In an energy efficiency management group the centralized energy saving process is performed：

- Monitoring: The management system collects the information of energy saving, which may include the performance information, such as DV and EC. And then calculate the energy efficiency values of all of the cells in the energy efficiency management group;

- Analysis: The management system analyzes the energy efficiency of all of the cells in the energy efficiency management group, and sort all of the cells according to the EE KPI values. Tell out the lower EE KPI cells by setting a shreshold and distinguishing the cells whose EE KPI value is lower than the threshold. For the lower EE KPI cells do the root cause analysis according to traffic load, energy consumption, service types, and the equipment of the network. And give the recommendations for improving the energy efficiency.

- Decision: According to the recommadations, the management system performs the solution evaluation and determination, and decides the most efficient energy saving solution for the the lower EE KPI cells, including the detailed energy saving optimization methods for the network.

- Execution: The management system executes the energy saving policies.

- Evaluation: The management system evaluates whether the EE KPI of the lower EE KPI cells has been optimized, and the new reference EE KPI value of the energy efficiency management group will be updated.

End of the change