3GPP TSG-SA5 Meeting #138-e 	S5-214268

e-meeting, 23 - 31 August 2021

- Source:	Huawei, China Mobile, CATT, China Unicom

- Title:	New SID on Deterministic Service Assurance Management

- Document for:	Approval

Agenda Item:	6.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: 	Study on Deterministic Service Assurance Management

## Acronym: FS_DSAM

## Unique identifier: 	{A number to be provided by MCC at the plenary}

Potential target Release: {Rel-18}.

Note that this field above indicates the proposed Release at the time of submission of the WID to TSG approval. It can later be changed without a need to revise the WID. The updated target Release is indicated in the Work Plan.

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No |  |  |  |  |  |
| Don't know | X |  |  |  | X |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a Study Item.

|  | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
| X | Study Item |

### 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 870030 | Enhancements of Management Data Analytics Service | FS_DSAM may interact with eMDAS services. |
| 910027 | Enhanced Closed loop SLS Assurance | FS_DSAM may interact with eCOSLA services. |
| 880027 | Autonomous network levels | FS_DSAM may interact with ANL services. |
| 880028 | Management data collection control and discovery | FS_DSAM may interact with MADCOL services. |
| 850034 | Management Aspects of 5G Service-Level Agreement | FS_DSAM may use services provided by MA5SLA. |
| 870024 | Enhancement on Management Aspects of 5G Service-Level Agreement | FS_DSAM may use services provided by EMA5SLA. |

## 3	Justification

5G network should satisfy diversified SLA requirements to support different vertical applications. Service assurance is more stringent for some vertical applications, e.g. video monitoring for production environment in a factory, remote control etc. Service performance anomaly such as jitter or unexpected packet loss may result in service interruption or severe consequences. Stable and deterministic service experience should be assured for these services.

An attribute “deterministicComm” is included in serviceProfile as one of the SLS requirements for network slice. This attribute specifies the properties of the deterministic communication for periodic user traffic, see clause 4.3 of TS 22.104. How to achieve deterministic service assurance and the impacts on network provisioning and operation should be investigated.

Further study is needed on concrete scenarios and service assurance solutions, e.g. providing stable and deterministic service experience for some typical vertical applications. There are some features to support deterministic communications from 5G network perspective, e.g. URLLC, Industry IoT, 5GS Integration with TSN etc. Coordination is important between Cross Management Domain and different Management Domains to support E2E service assurance.

Deterministic service assurance should consider scope, conditions and constraints, e.g. scope of service requirements, network deployment, mobility level, connection numbers and service areas etc. Potential risks should be predicted and prepared in advance.

## 4	Objective

This study item aims to study the following aspects in support of E2E deterministic service assurance from management aspects:

-	Investigate the scenarios, general workflow, related management services and procedures;

-	Study network capability requirements to support deterministic service assurance considering resource efficiency, e.g. to satisfy required reliability and service availability with appropriate reserved resource and redundancy degree;

-	Collaboration between Cross Management Domain and different Management Domains to support E2E deterministic service assurance;

-	Potential requirements, conditions, constraints and solutions for deterministic service assurance for specific services (e.g. video monitoring, remote control etc.):

-	In provisioning phase, solutions from management aspect to prepare, predict and verify deterministic service assurance, e.g. network planning based on service model and SLS requirements, service assurance verification and prediction according to planning results before provisioning;

-	In operation phase, solutions from management aspect to assure deterministic service experience, e.g. network optimization solutions to improve service and network performance, service assurance verification and prediction according to optimization solutions before and after applying optimization solutions;

-	Potential enhancements related to performance management and fault management;

This study aims to provide higher confidence level for deterministic service assurance.

The study may need cooperation from SA2 and RAN3 on network aspects.

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| TR | 28.xyz | Deterministic Service Assurance Management | TSG#95 | TSG#96 | <Zhang>, <Jian>, <Huawei>, <justincn.zhang@huawei.com> |

| Impacted existing TS/TR |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| Null |  |  |  |

## 6	Work item Rapporteur(s)

<Zhang>, <Jian>, <Huawei>, <justincn.zhang@huawei.com>

## 7	Work item leadership

SA WG5

## 8	Aspects that involve other WGs

SA2, RAN3 for network aspects

## 9	Supporting Individual Members

Supporting IM name

Huawei

China Mobile

CATT

China Unicom