3GPP TSG-SA5 Meeting #138-e 	S5-214282

- e-meeting, 23 - 31 August 2021

- Source:	Huawei

- Title:	PCR TS 28.104 add mobility performance analysis use case

- Document for:	Approval

Agenda Item:	6.4.18

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

[1]	SP-210132 Enhancements of Management Data Analytics Service

[2]	3GPP TS 28.104-000 “Management and orchestration; Management Data Analytics”

# 3	Rationale

According to the approved WI [1], the objectives include defining the use case of the mobility management related issues analysis. In 5G system,

This contribution propose to add mobility performance issue analysis capability of MDAS in [2].

# 4	Detailed proposal

It is proposed to apply following changes in [2].

1st modified section

# 7	MDA Scenarios

## 7.X	Mobility Performance analysis (UC-RMO_MDA)

### 7.X.1	Use cases

The mobility performance related problems may be resulted from too-early/too-late/ping-pong handovers due to inappropriate handover parameters. MDAS can be used to analyse service experience and network performance during handover period in different mobility scenarios. It may also be able to provide the recommendations of optimal handover parameters to MDAS consumer.

### 7.X.2	Requirements

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-RMO_MDA-CON-1 | The MDAS producer should have a capability allowing the authorized consumer to request mobility performance analysis. | (UC-RMO_MDA) mobility performance issue analysis |
| REQ-RMO_MDA-CON-2 | The MDAS producer should have a capability to provide the analytics report for mobility performance analysis. | (UC-RMO_MDA) mobility performance issue analysis |

End of modified section