3GPP TSG-SA5 Meeting #138-e 	S5-214321

- e-meeting, 23 - 31 August 2021															revision of S5-21xabc

- Source:	Huawei Technologies

- Title:	Clarification on the solution 1

- Document for:	Approval

Agenda Item:	7.5.5

# 1	Discussion

This pCR is to clarify the solution1 for new NetworkSliceConvergedCharging service.

# 2	References

[1]		3GPP TR 32.847 "Study on Charging Aspects for Network Slicing Phase 2"

# 3	Rationale

This pCR is to clarify the solution with the CCS described in the solution #3.1 and solution#3.2.

For the key issue #3, the "Tenant CCS" and "UE CCS" are introduced. The information about the max number of the Registrated UEs/max number of the PDU session mentioned in the solution #1 for the key issue #1 is related to the Tenant CCS. In order to keep the unified understanding in the TR 32.847, propose to clarify the CCS used in the solution 1.

The initial NS quota is reqused by the NSACF, CHF may grant the quota with the max number of the Registrated UEs/max number of the PDU session or may grant the quota based on the actural requirement. The NSACF counts the number of the Registrated UEs/ number of the PDU session and can request the NS quota again until the quota of max number of the Registrated UEs/max number of the PDU session or more than the max. Propose to modify the initial NS quota description in the solution 1.

# 4	Detailed proposal

The following changes are proposed to be incorporated into TR 32.847 [1].

First change

- 6.1.2.2	Flow description

The figure 6.1.2.2-1 describes the high-level charging procedure for Network slice converged charging based on NS_quota with setting of initial value by CHF which may provided to the NSACF during slice provisioning procedures.

Figure 6.1.2.2-1: NetworkSlice quota management - registration

The steps are with the following additions:

0ch. CHF gets per S-NSSAI the "max Nb of Reg UEs" obtained during NSI creation and sets the appropriate quota as initial value for NS_quota of Nb of Reg UEs.

0. The NSACF counts the Nb of Reg UEs and check against the initial value for NS_quota (e.g.max Nb of Reg UEs).

1. NS admission control for UEs registrations per 3GPP TS 23.502 [11].

Xch-a: At least one S-NSSAI has reached the initial value for NS_quota of  Nb of Reg UEs.

Xch-b: the NSACF invokes the Nchf_NSConvergedCharging service for requesting more quota (i.e. NS_quota (max Nb of Reg UEs) of the exhausted S-NSSAI.

xch-c. The S-NSSAI for which the initial NS_quota is reached will be either granted with more NS_quota or the request for more quota is indicated as rejected, based on CCS capabilities and CHF update per S-NSSAI the value for "max Nb of Reg UEs" obtained during NSI update.

xch-d. CHF provides response to NSACF with appropriate setting of for S-NSSAI(s) based on the status of step xch-c.

2. NS admission control for UEs registrations per 3GPP TS 23.502 [11].

Editor’s note: Futher adjustments will be added once the corresponding implementation of NSACF has been approved in SA2.

End of change