3GPP TSG-SA5 Meeting #138-e 	S5-214333rev2

- e-meeting, 23 - 31 August 2021

- Source:	Huawei

- Title:	pCR 28.816 Add potential solution for 5G data connectivity domain charging

- Document for:	Approval

Agenda Item:	7.5.2

# 1	Decision/action requested

For approval.

# 2	References

[1]	3GPP TS 28.816

# 3	Rationale

This contribution is to add potential solution for 5G data connectivity domain charging.

# 4	Detailed proposal

(For pseudo CR, include the complete clause(s) or subclause(s) of the latest draft TS/TR to be modified, with clear clause and sub-clause headings included and all modifications shown with revision marks, unambiguously showing where the changes shall be made or inserted in the draft TS/TR. It is not sufficient to just state, for example, “add the following text to the draft TS/TR…”.)

First modification

### 5.4.1.2		Potential solutions

### 5.4.1.2.0	General

The charging enhancement in 5G data connectivity domain are based on the charging aspect specified in TS 32.255 [6]. The description of 5G data connectivity domain charging architecture can be seen in clause 4.2 of this document.

The potential solutions that enhance the charging aspect in this clause may support the charging of following 5GS CIoT optimizations:

- Control Plane 5GS CIoT optimization

- Rate control of user data

- The access service of 5GS CIoT devices

- 5GS CIoT optimization information transfer during EPS interterworking

- The handover of 5GS CIoT devices in home-routed roaming scenario.

### 5.4.1.2.1		Charging enhancements of PDU session charging

The subclause below describes charging enhancement of PDU session charging for 5GS CIoT optimization in non-roaming scenarios.

The charging enhancement for 5GS CIoT optimization during PDU session establishment is based on the message flow described in figure 4.3.2.2.1.1 of TS 23.502 [4] and figure 5.2.2.2.2-1 of TS 32.255 [6] description.

The PDU session establishment charging is updated based on the message flow in figure 5.2.2.2.2-1 of TS 32.255 [6] as following.

It is assumed that a number of 5GS CIoT optimizations are supported by mobile network in this solution.

Step 1-7. The PDU session establishment as described. Some 5GS CIoT optimization information are included during PDU session establishment. The AMF sends Control Plane CIoT 5GS Optimisation to the SMF if determines. The AMF sends small data rate control to the SMF if it has stored in the AMF for the PDU session. The RAT type (NB-IoT or LTE-M) can be included in service request during PDU session establishment.

Step 8-9. The steps are described in clause 5.2.2.2.2 of TS 32.255 [6].

Step 9ch-a. The step is described is in clause 5.2.2.2.2 of TS 32.255 [6]. The possible charging information related to 5GS CIoT optimizations when supports are as following:

- the indication of Control Plane CIoT 5GS Optimisation,

- small data rate control indication,

- control plane only,

- RAT type (NB-IoT or LTE-M).

9ch-b. The CHF opens CDR for this PDU session.

9ch-c. The CHF acknowledges by sending Charging Data Response [Initial] to the SMF.

Step 10-16. The PDU session establishment or modification as described in clause 5.2.2.2.2 of TS 32.255 [6].

16ch-a. The step is described is in clause 5.2.2.2.2 of TS 32.255 [6]. The possible charging information related to 5GS CIoT optimizations when supports are as following:

- the indication of Control Plane CIoT 5GS Optimisation,

- small data rate control indication,

- control plane only,

- RAT type (NB-IoT or LTE-M).

16ch-b. The CHF updates CDR for this PDU session.

16ch-c. The CHF acknowledges by sending Charging Data Response [Update] to the SMF.

Step 17-19.	The steps of PDU session modification are described in clause 5.2.2.2.2 of TS 32.255 [6].

End of modification