- 3GPP TSG SA WG5 Meeting #138e	S5-214403

- Online, , 23 aug 2021- 31 Aug 2021

- Source:	Samsung

- Title:	pCR 28.817 solution for external consumer accessing MnS

- Document for:	Approval

Agenda Item:	6.5.5

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

None

# 3	Rationale

This contribution provides the solution for the use case – MnS is accessed by an external consumer of the 3GPP management system.

# 4	Detailed proposal

1st modified section

## 7.x Exposure governance when an external consumer of 3GPP Management System is accessing the MnS.

Serval type of consumer can access management capabilities provided by the management domain. First category of the consumers are the one which are inside the operator domain but not part of management domain e.g network function in 5GC and RAN domain. Second category of the consumers are the one which are outside the operator domain e.g application function deployed at the public cloud managed and provided by a 3rd party or external service provider. Last category of the consumers are the one which are inside the management domain e.g EMS, NMS. As per the SBMA (service based management architecture) it is possible for ANY network entity to consume MnS produced by ANY management entity in the OAM domain. However, the visibility to management capabilities should be restricted for the consumer which are not part of OAM domain.

In this solution an ExposureGovernanceControl IOC is proposed which will control the exposure of the available management capabilities towards an external consumer. An external consumer may not be provided with the full set of management capabilities. Instead, they will only be authorized to consume a apart of management capabilities as per the contract with the PLMN operator and local polices in place with in operator domain.

The following figure shows the procedure of exposure governance for an external consumer.

Note: Step X, X.1, X.2, Y and Z are not specified as part of this document.

Note: MCEGF can be any management function exposing Generic Provisioning MnS (as defined in 3GPP TS 28.532) to perform basic CRUD operation on the ExposureGovernanceControl IOC.

Note: The entity Tenant is representing the external vertical customer.

- Tenant will register with the operator BSS as consumer of the management capabilities produced by the operators 3GPP Management System. This will be an offline process dependent on the operator policy.

- X.1 A Tenant ID will be generated and provided to the consumer.

- Based on the registration and operator policy the MnS Consumer, acting on behalf of tenant, will create authorization for the particular tenant in the 3GPP Management system. For this MnS Consumer will send createMOI request to MCEGF (behaving as MnS producer) in order to create an instance of ExposureGovernanceControl IOC.

- The instance of ExposureGovernanceControl IOC will be created dictating the detailed authorization for the tenant, identified by the tenant-ID provide in the request. The following table shows the hierarchical attributes which can be used to form the the ExposureGovernanceControl IOC:

| Attributes |  |  |  |  |  | Support | Cardinality | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| consumerExposure |  |  |  |  |  | M | 1…* | The authorized capabilities for a particular consumer |
|  | consumer ID |  |  |  |  | M | 1 | The identification of the consumer provided at the time of registration with BSS |
|  | consumerType |  |  |  |  | O | 1 | The type of consumer (OAM-Internal, OAM-External{gNB, CU-CP, NWDAF}, NOP-External{ASP, ECSP} ) |
|  |  | authorizedMnS |  |  |  | M | 1…* | List of management services and its capabilities the consumer is authorized to access. |
|  |  |  | MnS Type, Label, Version |  |  | O | 0…1 | Type of MnS (PROVISIONING, FAULT_SUPERVISION, PERFORMANCE_ASSURANCE), Label: human readable description, Version: version of the MnS |
|  |  |  | allowedComponentA |  |  | M | 1…* | The list of operation of the MnS, consumer is authorized to access. This will be the {MnSRoot} in case of OpenAPI implementations |
|  |  |  | allowedComponentB |  |  | M | 1…* | The list of IOC the consumer can access. |
|  |  |  |  | uri |  | M | 1 | The URI of the NRM fragment (IOC) |
|  |  |  |  | permission |  | M | 1…N | The list of attributes permission present in the IOC |
|  |  |  |  |  | attName | M | 1 | Attribute name present in the IOC |
|  |  |  |  |  | attPermission | M | 1 | The type of permission (Readonly, Writeable) |
|  |  |  | allowedComponentC |  |  | O | 0…1 | The list of performance measurement and the KPI, the consumer is authorized to collect. |
|  |  |  |  | allowedMeasurement |  | M | 1..* | List of authorized measurements |
|  |  |  |  |  | managedEntity | M | 1 | Managed entity producing the measurement |
|  |  |  |  |  | perfMeasurement | M | 1 | Measurement the consumer is authorized to collect. |
|  |  |  |  | allowedKPI |  | M | 1..* | List of authorized KPI |
|  |  |  |  | allowedAlarmInfo |  | O | 1…* | List of alarm info the consumer is authorized to receive |
|  |  |  |  |  | managedEntityAlarmInfo | M | 1 | Alarm info |
|  |  |  | allowedNotifications |  |  | M | 1…* | The url of the notification which consumer is authorized to subscribe for. |

- The successful response of createMOI is sent to MnS consumer.

- Later, when tenant tries to access any management service provided by the 3GPP Management system, it will send an appropriate request with the tenant-ID assigned to it. Here, MnS consumer will send allocateNSI request to NSI provisioning MnS producer.

- The producer will request the exposure governance from MCEGF. MCEGF is a management function responsible for creating the exposure governance regulation per consumer. This logic will be consulted by every MnS producer before it accept the request from the tenant. getMOIAttribtes request will be used for this purpose.

- The MCEGF will send the required information.

- The NSI provisioning MnS producer will check the exposure governance regulation.

- If the tenant is authorised, the NSI provisioning MnS producer will allocate the NSI as per the procedures defined in 3GPP TS 28.531.

- If the tenant is not-authorized to allocate an NSI, the un-successful response for allocateNSI will be sent to the MnS consumer.

End of modified section