3GPP TSG-SA5 Meeting #138-e 	S5-214428

- e-meeting, 23 - 31 August 2021

- Source:	Ericsson

- Title:	Adding flow to session based solution for key issue 5

- Document for:	Approval

Agenda Item:	7.5.5

# 1	Decision/action requested

Include the proposed changes in TR 28.847.

# 2	References

[1]		3GPP TR 32.847: "Study on Charging Aspects for Network Slicing Phase 2"

# 3	Rationale

Adding an session based charging solution for the key issue 5 charging for number of simultaneous UEs per network slice.

# 4	Detailed proposal

First change

#### 6.5.3.4	Charging scenarios

Determination of passing a threshold for a network slice using Session based charging (SCUR).

Editor’s Note: The NF that will hold the CTF is ffs.

Figure 6.5.2.3.1: SCUR – UE addition to network slice

1)	UE to be added: A UE is to be added to the network slice.

2)	Trigger evaluated: the NF (CTF) determines that the addition of the UE would cause the threshold to be passed.

3)	Charging Data Request [Initial]: the NF (CTF) starts the charging session by sending a request to the CHF for the passing of the threshold.

4)	Account, Rating Control: the CHF may use the information to rate and make deductions from customer's account balance.

5)	 Create CDR: the CHF creates a CDR related to the service.

6)	Charging Data Response [Initial]: the CHF informs the NF (CTF) on the result of the request, the CHF may provide new thresholds.

7)	 UE added: The UE is added to the network slice.

8)	UE to be added or removed: a UE is to be added to or removed from the network slice.

9)	Trigger evaluated: the NF (CTF) determines that the change in number of UE would cause a CHF provided threshold to be passed.

10)	Charging Data Request [Update]: the NF (CTF) updates the charging session by sending a request to the CHF for the passing of the threshold.

11)	Account, Rating Control: the CHF may use the information to rate and make deductions from customer's account balance.

12)	 Update CDR: the CHF updates the CDR related to the service.

13)	Charging Data Response [Update]: the CHF informs the NF (CTF) on the result of the request.

14)	 UE added: if the trigger was addition of UE then the UE is added to the network slice.

15)	UE removed: a UE is to be removed from the network slice.

16)	Trigger evaluated: the NF (CTF) determines that the removal of UE would cause the termination trigger to be passed.

17)	Charging Data Request [Termination]: the NF (CTF) terminates the charging session by sending a request to the CHF for the passing of the threshold.

18)	Account, Rating Control: the CHF may use the information to rate and make deductions from customer's account balance.

19)	 Close CDR: the CHF closes the CDR related to the service.

20)	Charging Data Response [Termination]: the CHF informs the NF (CTF) on the result of the request.

End of changes