3GPP TSG-SA5 Meeting #138-e 	S5-214435

- e-meeting, 23 - 31 August 2021

- Source:	Ericsson

- Title:	Use case for home MNO charging subscriber for 5G data connectivity

- Document for:	Approval

Agenda Item:	7.5.7

# 1	Decision/action requested

Include the proposed changes in TR 28.827.

# 2	References

[1]		3GPP TR 28.827: "Study on 5G charging for additional roaming scenarios and actors"

# 3	Rationale

Use case for charging in visited MNO for wholesale charging towards home MNO.

# 4	Detailed proposal

First change

## 7.x	Convey charging from visited MNO to home MNO

### 7.x.1	Home MNO does retail charging towards subscribers

#### 7.x.1.1	Use cases

##### 7.x.1.1.2	Use case #2a: Use case for home MNO charging subscriber for 5G data connectivity at roaming using local breakout

This use case focuses on home MNO and subscriber business roles.

A subscriber has a UE and a subscription with the home MNO which allows usage of 5G data connectivity while roaming. The subscriber is roaming in a visited network and is allowed to use local breakout.

The home MNO charging of the subscriber could be based on data volume that the home MNO UE has used in the visited MNO’s network considering of the following aspects:

-	Visited network

-	RAT type;

-	S-NSSAI;

-	DNN;

-	QoS information.

### 7.x.3	Potential charging requirements

REQ-CH_CVTOH-01: The charging mechanism in visited MNO should support conveying charging information for the 5G data connectivity usage to the home MNO for each UE.

REQ-CH_ CVTOH-02: The charging mechanism in visited MNO should support collecting charging information related to 5G data connectivity usage for each UE.

### 7.x.4	Key issues

The following key issues are identified:

-	Key Issue #2a: Collection of charging mechanism in visited MNO about the 5G data connectivity usage.

-	Key Issue #2b: Conveying the charging information to the home MNO

- 	Key Issue #2c: Finding the correct CHF

-	Key Issue #2d: Service based interface to use between visited CHF and home CHF

-	Key Issue #2e: Trigger handling between visited CHF and home CHF

### 7.x.4	Possible solutions

#### 7.x.4.1	Potential charging architectures

The possible solutions for charging for number of UEs and 5G data connectivity provided to the home MNO’s by the visited MNO.

Figure 7.x.4.1-1: Roaming 5G data connectivity scenario in service-based interface representation

The visited CHF does converged charging for interconnect, while the home CHF does converged charging for the subscriber.

#### 7.x.4.2	Potential charging flows

Figure 7.x.4.2-1 shows a scenario for session based charging (SCUR) with CHF to CHF communication.

Figure 7.x.4.2-1: SCUR - Session based charging with CHF to CHF communication

1)	Request for service delivery: A request for session establishment is received in the NF (CTF). The service is configured to be authorized by thev CHF to start.

2)	Units determination: the NF (CTF) determines the number of units depending on the service requested by the UE in "Decentralized Units determination" scenario.

3)	Charging data request [Initial, Quota requested]: The NF (CTF) sends the request to the vCHF for the service to be granted authorization to start, and to reserve the number of units if determined in item 2.

4)	Determine hCHF: the vCHF determines that it need to interact with hCHF and which hCHF it should contact.

5)	Charging home request [Initial, Quota requested]: The vCHF sends the request to the hCHF for the service to be granted authorization to start, and to reserve the number of units if determined in item 2.

6)	Account, Rating, Reservation control: the hCHF rates the requests either based on the number of units requested or on internal unit determination, checks if corresponding funds can be reserved on the user's account balance. If the account has sufficient funds, the hCHF performs the corresponding reservations.

7)	Open CDR: based on policies, the hCHF opens a CDR related to the service.

8)	Charging home response [Initial, Quota granted]: The hCHF grants authorization to vCHF  for the service to start, with the reserved number of units.

9)	Open CDR: based on policies, the vCHF opens a CDR related to the service.

10)	Charging data response [Initial, Quota granted]: The vCHF grants authorization to NF (CTF) for the service to start, with the reserved number of units.

11)	Granted units supervision: the NF (CTF) monitors the consumption of the granted units.

12)	Start of service delivery: the NF (CTF) starts to deliver the content/service based on the reserved number of units.

13)	Quota management trigger: A trigger associated to quota management is met. Units determination is performed when applicable.

14)	Charging data request [Update, Unit used, Quota requested]: the NF (CTF) sends the request to the vCHF, for more units to be granted for the service to continue and reporting the used units.

15)	Charging home request [Update, Unit used, Quota requested]: the vCHF sends the request to the hCHF, for more units to be granted for the service to continue and reporting the used units.

16)	Account, Rating, Reservation control: The hCHF performs the process related to the reported usage and the requested reservation, involving rating entity and user's account balance.

17)	 Update CDR: based on policies, the hCHF updates the CDR with charging data related to the service.

18)	Charging home response [Update, Quota granted]: The hCHF grants quota to vCHF for the service to continue, with the reserved number of units.

19)	 Update CDR: based on policies, the vCHF updates the CDR with charging data related to the service.

20)	Charging data response [Update, Quota granted]: The vCHF grants quota to NF (CTF) for the service to continue, with the reserved number of units.

21)	Granted units supervision: the NF (CTF) monitors the consumption of the granted units.

22)	Service delivery ongoing: the NF (CTF) continues the service delivery based on the granted quota.

23)	Service released: the service is terminated or released.

24)	Charging data request [Termination, Unit used]: the NF (CTF) sends the request to the vCHF, for charging data related to the service termination with the final consumed units.

25)	Charging home request [Termination, Unit used]: the vCHF sends the request to the hCHF, for charging data related to the service termination with the final consumed units.

26)	Account, Rating control: The hCHF performs the service termination process involving rating entity and user's account balance.

27)	 Close CDR: based on policies, the hCHF closes the CDR with charging data related to the service termination and the last reported units.

28)	Charging home response [Termination]: The hCHF informs the vCHF on the result of the request.

29)	 Close CDR: based on policies, the vCHF closes the CDR with charging data related to the service termination and the last reported units.

30)	Charging data response [Termination]: The vCHF informs the NF (CTF) on the result of the request.

End of changes