3GPP TSG SA WG5 Meeting 138-e		S5-214469

- electronic meeting, online, 23 - 31 August 2021 					                     revision for S5-21xxxx

- Source:	Nokia, Nokia Shanghai Bell

- Title:	possible solution for network slice protection on N6 interface

- Document for:	Approval

Agenda Item:	6.5.3

# 1	Decision/action requested

The group is asked to discuss and approve the proposal..

# 2	References

[1]	3GPP TR 28.811: "Management and orchestration; Network Slice Management Enhancement"

[2]	GSMA NG.116: "Generic Network Slice Template"

# 3	Rationale

This pCR is to add possible solution for network slice protection on N6 interface according to potential requirements defined in clause 6.5 of this study report.

# 4	Detailed proposal

Start of 1st Change

## 7.x	Possible solutions for network slice protection on N6 interface

It is proposed to enhance the 5G NRM to support network slice protection on N6 interface :

- Add  an optional new attribute for N6 interface protection requirement in ServiceProfile and CNSliceSubnetProfile for CN. The attribute includes required security functions and corresponding rules of each function. E.g. the dataType of the attribute could be defined as following:

- N6Protection <<dataType>>:

| Attribute Name | Support Qualifier | isWritable | multiplex | Description |
| --- | --- | --- | --- | --- |
| servAttrCom | CM | T | 1 | Only valid when the attribute is in ServiceProfile |
| secFuncs | M | T | 1..* | List of security control functions/features required by the slice/slice subnet consumer. E.g. Fireware, NAT, antimalware, parental control, DDoS protection function, etc. |

Each security function could be defined as:

SecFunc <<dataType>>:

| Attribute Name | Support Qualifier | isWritable | multiplex | Description |
| --- | --- | --- | --- | --- |
| secFunId | M | F | 1 | Identifier a security function |
| secFunType | M | F | 1 | Type of the security function |
| secRules | O | T | 0..* | security rules could be configured on each function. If it's absent, the default rules could be applied |

- Enhance PCC rules associated to PCFFunction or SMFFunction IOC to support routing traffic to security function on N6 interface. e.g. extend PredefinedPccRuleSet IOC (add an optional sNSSAIList attribute in the IOC definition) to support per slice (or list of slices) PCC rule list, or extend PccRule dataType (add an optional sNSSAIList attribute in the dataType definition) to support per slice (or list of slices) PCC rule which including traffic steering rules, or extend TrafficControlData dataType directly (add an optional sNSSAIList attribute in the dataType definition) to support per slice (or list of slices) traffic steering policies/rules.

- Example workflow to deploy a network slice for a NSC with N6 protection requirement

Note: how to deploy and configure security function is out of scope of 3GPP

End of Change