| 3GPP TS 25.538 V0.2.0 (2021-07) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Management and orchestration; Edge Computing Management; (Release 17) |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	6

4	Examples for styles	7

4.1	Heading styles	7

Annex <A> (normative):	7

Annex <B> (informative):	8

Annex <X> (informative): Change history	9

For definitive guidance on drafting 3GPP TSs and TRs, see 3GPP TS 21.801 supplemented by the 3GPP web page http://www.3gpp.org/specifications-groups/delegates-corner/writing-a-new-spec.

Ensure all blue guidance text is removed before submitting the TS/TR to the TSG for approval.

# Foreword

This clause is mandatory; do not alter the text in any way other than to choose between "Specification" and "Report".

This Technical Specification|Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In drafting the TS/TR, pay particular attention to the use of modal auxiliary verbs! TRs shall not contain any normative provisions.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

This clause is optional. If it exists, it shall be the second unnumbered clause.

# 1	Scope

The present document specifies the management aspects of edge computing including concepts, use cases, requirements and procedural flows that covers lifecycle management, provisioning, performance assurance and fault supervision for edge computing.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Edge Computing: A concept, as described in 3GPP TS 23.501 [4], that enables operator and 3rd party services to be hosted close to the UE's access point of attachment, to achieve an efficient service delivery through the reduced end-to-end latency and load on the transport network.

Edge Computing Service Provider: A mobile network operator offering Edge Computing service.

Edge Data Network: A local Data Network that supports the architecture for enabling edge applications.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ASP	Application Service Provider

DN	Data Network

DNAI	Data Network Access Identifier

DNN	Data Network Name

EAS	Edge Application Server

ECS	Edge Configuration Server

ECSP	Edge Computing Service Provider

EDN	Edge Data Network

FQDN	Fully Qualified Domain Name

GSM	Global System for Mobile Communications

GSMA	GSM Association

# 4	Concepts  and Overview

## 4.1	Heading styles

# 5	Edge Computing Management Capabilities

## 5.1	Lifecycle Management

### 5.1.1	Description

The lifecycle management of the edge components is to be enabled by the 3GPP Management System. The lifecycle management includes instantiation, termination and scaling of the edge components.

### 5.1.2	EAS Deployment

The goal of this use case is to enable ASP to deploy the EAS in the EDN, by requesting the provisioning MnS producer with the deployment requirements (e.g., the topological or geographical service areas, software image information, ,QoS, affinity/anti-affinity with other EAS etc) to deploy the EAS. The provisioning MnS producer returns a response indicating the operation is in progress to prevent the consumer from waiting, as the deployment in the edge cloud may take a while. Since, there can be multiple Edge Data Network (EDN) present/serving a particular edge location. This makes it critical for application service provider to have their EAS deployed at appropriate EDN(s) to provide high performance services for the UE. Therefore, provisioning MnS producer analyses the deployment requirements to determine where (i.e on which EDN) and how many EAS VNF instance(s) should be instantiated, and requests the NFVO in ETSI NFV MANO to instantiate the EAS VNF instance(s). The provisioning MnS producer sends a notification to ASP indicating the result of instantiation (e.g., success, failure, or partially failure) when a notification is received from NFVO indicating the result of instantiation operation.

### 5.1.2 		EAS Termination

The goal of this use case is to enable ASP to trminate the EAS in the EDN, by requesting the provisioning MnS producer to terminate the EAS VNF instance. The provisioning MnS producer requests the NFVO in ETSI NFV MANO to terminate the EAS VNF instances. The provisioning MnS producer sends a notification to ASP indicating the termination is in progress when a notification is received from NFVO indicating the start of termination operation. The provisioning MnS producer sends another notification to ASP indicating the result of termination (e.g., success, failure, or partially failure) when a notification is received from NFVO indicating the result of termination operation.

### 5.1.3 	Requirements

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-EAS-INST-FUN-1 | Generic provisioning MnS producer should have a capability allowing an authorized consumer to request the deployment of EAS based on the given deployment requirements. | EAS Deployment |
| REQ-EAS-INST-FUN-2 | Generic Provisioning MnS Producershould have the capability to deploy EAS at a suitable EDN which can support the EAS requirements e.g serving location, required latency, affinity/anti-affinity with other EAS, service continuity. | EAS Deployment |
| REQ-EAS-INST-FUN-3 | Generic provisioning MnS producer should have a capability to inform the authorized consumer about the progress of instantiation as the response to the deployment request. | EAS Deployment |
| REQ-EAS-INST-FUN-4 | Generic provisioning MnS producer should have a capability to notify the authorized consumer the result (e.g., success, failure, or partially failure) of instantiation operation. | EAS Deployment |
| REQ-EAS-TERM-FUN-1 | Generic provisioning MnS producer should have a capability allowing an authorized consumer to request the termination of the EAS VNF instance. | EAS Termination |
| REQ-EAS-TERM-FUN-2 | Generic provisioning MnS producer should have a capability to inform the authorized consumer about the progress of termination as the response to the termination request. | EAS Termination |
| REQ-EAS-TERM-FUN-3 | Generic provisioning MnS producer should have a capability to notify the authorized consumer the result (e.g., success, failure, partially failure) of termination operation. | EAS Termination |

# 6	Edge NRM

Editors Note: This section will contain the edge related NRM or just the reference to it in 28.541.

## 6.1		Information Model definitions for Edge NRM

### 6.1.1	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |
| TS 28.622 [30], IOC, Top | Top |
| TS 28.622 [30], IOC, SubNetwork | SubNetwork |
| TS 28.622 [30], IOC, ManagedFunction | ManagedFunction |

## 6.2	Class diagram

### 6.2.1	Relationships

Figure 6.2.1-2 Edge NRM containment/naming relationship

Figure 6.2.1-3 Transport view of EES NRM

Figure 6.2.1-4 Transport view of ECS NRM

### 6.2.2	Inheritance

Figure 6.2.2-1 Edge Inheritance Relationship

Editor's NOTE 4: Whether EASProfile is dataType or IOC is FFS.

# 7	Procedural Flows

Editors Note: This section will contain the procedures for different edge computing management capabilities e.g provisioning/LCM, performance assurance etc.

# 8. 		Management Service for Edge Computing

## 8.1	Provisioning

A, B and C

## 8.1	Performance Assurance

A, B and C

## 8.2	Fault Supervision

A, B and C

# Annex <A> (normative):

# Annex <B> (informative):

# Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 31-Sep-2021 | #138e | S5-214625 |  |  |  | Skeleton | 0.1.0 |
| 31-Sep-2021 | #138e | S5-214625 |  |  |  | Initial TS Structure | 0.2.0 |
| 31-Sep-2021 | #138e | S5-214626 |  |  |  | Edge NRM | 0.2.0 |
| 31-Sep-2021 | #138e | S5-214629 |  |  |  | Terms Scope Reference Abbreviations | 0.2.0 |
| 31-Sep-2021 | #138e | S5-214626 |  |  |  | Edge NRM | 0.2.0 |