3GPP TSG-SA5 Meeting #138-e 	S5-214031

# e-meeting, 23 - 31 August 2021

# Source:	3GPP CT4

# Title:	Reply LS on Clarification on the API design principles

# Document for:	Information, Discussion

Agenda Item:  6.1

3GPP TSG-CT WG4 Meeting #104-e 		C4-213451

# E-Meeting, 19th – 28th May 2021

Title:	Reply LS on Clarification on the API design principles

Response to:	Clarification on the API design principles

Release:	Rel-17

Work Item:	SBIProtoc17

Source:	CT4

To:	CT3

Cc:	CT1, SA4, SA5

Contact person: 	Varini Gupta

varini.gupta@samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachment: 	None

# 1	Overall description

CT4 would like to thank CT3 for the LS on Clarification on the API design principles.

CT4 would like to provide the following responses:

Question 1) Which HTTP method (a standard HTTP GET or a custom HTTP POST) should be used by the NF service consumer to retrieve information from the service producer in the scenario where the NF service producer does not own the information requested by the consumer and the service producer needs to further fetch the information from other services?

[CT4 Answer] Use of HTTP GET or custom operation based on HTTP POST mainly depends on whether the request is safe/idempotent or unsafe/non-idempotent. It does not necessarily depend on whether the retrieved information is “owned” by the service producer or not.

Question 2) If a custom operation is used, then what is the criteria to decide whether associated resource is needed or not?

[CT4 Answer] A custom operation without associated resource is not expected to modify any resource in the server.

Refer Annex C.4 of 3GPP TS 29.501:

When the custom operation is not associated with any resource but with the service, it acts as an executable function with input parameters and returns the result of the executed function in the response body, not modifying any resource.

CT4 would like to highlight that according to guidelines in 29.501, it is highly recommended to use REST-style service operations where possible. It is good to associate a resource with an operation wherever possible; this is also useful when additional operations may be defined on the parent resource in future. This also should be evaluated on a case-by-case basis.

Question 3) whether a resource can be associated only with a custom operation?

[CT4 Answer] There is nothing that prevents a resource to be associated only with custom operations. Such design choice should be evaluated on a case-by-case basis.

# 2	Actions

To CT3

ACTION: 	CT4 kindly requests CT3 to take the above information into account

# 3	Dates of next TSG SA WG2 meetings

CT WG4 Meeting #105-e		Aug 17-27, 2021