3GPP TSG-SA5 Meeting #138-e 	S5-214059

# e-meeting, 23 - 31 August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0307 | rev | - | Current version: | 17.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add PRB Usage for MIMO scenario |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom, Huawei, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | ePM_KPI_5G |  |  |  |  |  | Date: |  |  | 2021-08-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - The PRB usage for MIMO (which is already defined in TS 38.314) is not defined in TS 28.552; - In the existing scheme of PRB usage for MIMO in TS 38.314, the constant alpha in calculation formula of PRB usage cannot reflect the actual spatial transmission capacity of a cell, and there is no clear calculation method of spatial transmission capacity. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the new PRB usage for MIMO in TS 28.552，which takes into account the resource occupancy in both frequency domain and space domain, and the spatial layer resources are considered in the numerator and a dynamic factor representing spatial capability is introduced in the denominator of the formula, which can more accurately reflect the resource utilization of wireless network in MIMO scenarios |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.2.x(new), 5.1.1.2.y(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

5.1.1.2.x	PDSCH PRB Usage for MIMO with dynamic factor

a)	This measurement provides the total usage (in percentage) of physical resource blocks (PRBs) for MIMO with dynamic factor in the downlink.

b)	SI

c)	This measurement is obtained as:

MET=∀i∀j{M1ij(T)*Lij(T)}NT*PT*LM(T)*100,

where MET denotes total PDSCH PRB usage per cell which is percentage of PRBs used, averaged during time period 𝑇 with integer value range: 0-100; M1ijT denotes a count of PDSCH PRBs used for traffic transmission for UE 𝑖 on single MIMO layer per cell at sampling occasion 𝑗; Lij(T) denotes the number of MIMO layers scheduled for UE 𝑖 at sampling occasion 𝑗; N(T) denotes total number of sampling occasions taken during time period T; P(T) denotes total number of PDSCH PRBs available for 1 sampling occasion on single MIMO layer per cell; LM(T) denotes the time-domain average maximum scheduling layer number of PDSCH in time period T; T denotes the time period during which measurement is performed; i denotes a UE that is scheduled during time period T;  and j denotes sampling occasion during time period T, a sampling occasion is 1 symbol.

Among them, the time-domain average maximum scheduling layer number of PDSCH in time period T (LM(T)) is obtained as:

LMT=jLmaxjTKT，KT=j,LmaxjT≠01

Where Lmaxj(T) denotes the maximum number of scheduling layer of PDSCH at sampling occasion j; K(T) denotes the number of sampling occasions at which Lmaxj(T)is not 0; T denotes Time Period during which the measurement is performed; and j denotes the sampling occasion during time period T, a sampling occasion is 1 symbol.

d)	A single integer value from 0 to 100.

e)	RRU.PrbTotDlMimo, which indicates the PDSCH PRB Usage for MIMO with dynamic factor

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for monitoring the load of the radio physical layer under MIMO scenario.

5.1.1.2.y	PUSCH PRB Usage for MIMO with dynamic factor

a)	This measurement provides the total usage (in percentage) of physical resource blocks (PRBs) for MIMO with dynamic factor in the uplink.

b)	SI

c)	This measurement is obtained as:

MET=∀i∀j{M1ij(T)*Lij(T)}NT*PT*LM(T)*100,

where MET denotes total PUSCH PRB usage per cell which is percentage of PRBs used, averaged during time period 𝑇 with integer value range: 0-100; M1ijT denotes a count of PUSCH PRBs used for traffic transmission for UE 𝑖 on single MIMO layer per cell at sampling occasion 𝑗; Lij(T) denotes the number of MIMO layers scheduled for UE 𝑖 at sampling occasion 𝑗; N(T) denotes total number of sampling occasions taken during time period T; P(T) denotes total number of PUSCH PRBs available for 1 sampling occasion on single MIMO layer per cell; LM(T) denotes the time-domain average maximum scheduling layer number of PUSCH in time period T; T denotes the time period during which measurement is performed; i denotes a UE that is scheduled during time period T;  and j denotes sampling occasion during time period T, a sampling occasion is 1 symbol.

Among them, the time-domain average maximum scheduling layer number of PUSCH in time period T (LM(T)) is obtained as:

LMT=jLmaxjTKT，KT=j,LmaxjT≠01

Where Lmaxj(T) denotes the maximum number of scheduling layer of PUSCH at sampling occasion j; K(T) denotes the number of sampling occasions at which Lmaxj(T)is not 0; T denotes Time Period during which the measurement is performed; and j denotes the sampling occasion during time period T, a sampling occasion is 1 symbol.

d)	A single integer value from 0 to 100.

e)	RRU.PrbTotUlMimo, which indicates the PUSCH PRB Usage for MIMO with dynamic factor

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for monitoring the load of the radio physical layer under MIMO scenario.

2nd Change

A.X	Monitoring of PRB Usage for MIMO in NG-RAN

The PRB Usage for MIMO with dynamic factor measurement could provide operators the load information of radio network in MIMO scenario taking spatial resource into consideration. In the early stage of network development, the measurements with a dynamic spatial factor can reflect the actual frequency and space resource utilization of a cell after MU-MIMO is activated.  In the late stage of network development, the measurements can help operators be aware of whether a cell has experienced high load.

End of Change