3GPP TSG-SA5 Meeting #138-e 	S5-214067

# e-meeting, 23 - 31 August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.299 | CR | 0824 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of PSCell ID in SGW CDR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDCE5-CH, TEI17 |  |  |  |  |  | Date: |  |  | 2021-06-29 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Based on the SA5 reply LS (S5-197481) to ETSI TC LI, SA2 has been requested (in S5‑202022 SA5 was in copy) to make the PSCELL ID available at the SGW in EPC scenario supporting 5G New Radio via Dual Connectivity. The objective was to capture this information in SGW CDR to comply with Retained Data regulations. TS 23.401 has been updated for the MME to pass PSCell ID to SGW in different procedures, which can be captured in SGW CDRs |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce PSCell information in PS-information for use in Rf |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Retained data regulations cannot be fulfilled in EPC scenario supporting 5G New Radio via Dual Connectivity |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.0, 7.2.158, 7.2.X(New), 7.2.Y(New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

### 7.2.0	General

For the purpose of offline charging additional AVPs are used in ACR / ACA and for online charging additional AVPs are used in CCR / CCA. All 3GPP specific AVPs mentioned are relevant for both offline and online charging unless specifically excluded. The information is summarized in the following table 7.2.0.1 along with the AVP flag rules.

The 3GPP Charging Application uses the value 10415 (3GPP) as Vendor-Id.

Detailed descriptions of AVPs that are used specifically for 3GPP charging are provided in the subclauses below the table. However, for AVPs that are just borrowed from other applications only the reference (e.g. TS 29.229 [204]), is provided in the following table and the detailed description is not repeated.

Where 3GPP RADIUS VSAs are re-used, they shall be translated to Diameter AVPs as described in RFC 4005 [407] with the exception that the 'M' flag shall be set.

# Table 7.2.0.1: 3GPP specific AVPs

| AVP Name |  |  | AVP Code |  |  | Used in |  |  |  |  |  |  |  |  |  |  |  |  | Value Type |  |  | AVP Flag rules |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | ACR |  | ACA |  |  |  | CCR |  |  |  | CCA |  |  |  |  |  | Must |  |  | May |  |  | Should not |  |  | Must not |  |
| 3GPP-Charging-Characteristics |  |  | 13 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | refer [207] |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3GPP-Charging-Id |  |  | 2 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | refer [207] |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3GPP-GGSN-MCC-MNC |  |  | 9 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | refer [207] |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3GPP-IMSI |  |  | 1 |  |  | - |  | - |  |  |  | X |  |  |  | - |  |  | refer [207] |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3GPP-IMSI-MCC-MNC |  |  | 8 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | refer [207] |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3GPP-MS-TimeZone |  |  | 23 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | refer [207] |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3GPP-NSAPI |  |  | 10 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | refer [207] |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3GPP-OC-Specific-Reduction |  |  | 1320 |  |  | - |  | - |  |  |  | - |  |  |  | X |  |  | Grouped |  |  |  |  |  |  |  |  |  |  |  | M |  |
| 3GPP-OC-Rating-Group |  |  | 1321 |  |  | - |  | - |  |  |  | - |  |  |  | X |  |  | Unsigned32 |  |  |  |  |  |  |  |  |  |  |  | M |  |
| 3GPP-OC-Request-Type |  |  | 1322 |  |  | - |  | - |  |  |  | - |  |  |  | X |  |  | Enumerated |  |  |  |  |  |  |  |  |  |  |  | M |  |
| 3GPP-PDP-Type |  |  | 3 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | refer [207] |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3GPP-PS-Data-Off-Status |  |  | 4406 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  |  |  |  |  |  |
| 3GPP-RAT-Type |  |  | 21 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | refer [207] |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3GPP-Selection-Mode |  |  | 12 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | refer [207] |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3GPP-Session-Stop-Indicator |  |  | 11 |  |  | - |  | - |  |  |  | X |  |  |  | - |  |  | refer [207] |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3GPP-SGSN-MCC-MNC |  |  | 18 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | refer [207] |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3GPP-User-Location-Info |  |  | 22 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | refer [207] |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Access-Availability-Change-Reason |  |  | 2833 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | refer [215] |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Access-Network-Charging-Identifier-Value |  |  | 503 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | refer [214] |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Access-Network-Info-Change |  |  | 4401 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Access-Network-Information |  |  | 1263 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | OctetString |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Access-Transfer-Information |  |  | 2709 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Access-Transfer-Type |  |  | 2710 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Account-Expiration |  |  | 2309 |  |  | - |  | - |  |  |  | - |  |  |  | X |  |  | Time |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Accumulated-Cost |  |  | 2052 |  |  | - |  | - |  |  |  | - |  |  |  | X |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Accuracy |  |  | 3137 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | refer [245] |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Adaptations |  |  | 1217 |  |  | - |  | - |  |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| ADC-Rule-Base-Name |  |  | 1095 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | refer [215] |  |  |  |  |  | - |  |  | - |  |  |  |  |
| Additional-Content-Information |  |  | 1207 |  |  | - |  | - |  |  |  | X |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Additional-Exception-Reports |  |  | 3936 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Additional-Type-Information |  |  | 1205 |  |  | - |  | - |  |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Address-Data |  |  | 897 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Address-Domain |  |  | 898 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Addressee-Type |  |  | 1208 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Address-Type |  |  | 899 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| AF-Charging-Identifier |  |  | 505 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | refer [214] |  |  |  |  |  |  |  |  |  |  |  |  |  |
| AF-Correlation-Information |  |  | 1276 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Allocation-Retention-Priority |  |  | 1034 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | refer [215] |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Alternate-Charged-Party-Address |  |  | 1280 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | UTF8String |  |  | V.M |  |  | - |  |  | - |  |  |  |  |
| Announcement-Identifier |  |  | 3905 |  |  | - |  | - |  |  |  | - |  |  |  | X |  |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Announcement-Information |  |  | 3904 |  |  | - |  | - |  |  |  | - |  |  |  | X |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Announcement-Order |  |  | 3906 |  |  | - |  | - |  |  |  | - |  |  |  | X |  |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Announcing-PLMN-ID |  |  | 4408 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Announcing-UE-HPLMN-Identifier |  |  | 3426 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V.M |  |  | - |  |  | - |  |  |  |  |
| Announcing-UE-VPLMN-Identifier |  |  | 3427 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V.M |  |  | - |  |  | - |  |  |  |  |
| AoC-Cost-Information |  |  | 2053 |  |  | - |  | - |  |  |  | - |  |  |  | X |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| AoC-Format |  |  | 2310 |  |  | - |  | - |  |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| AoC-Information |  |  | 2054 |  |  | - |  | - |  |  |  | X |  |  |  | X |  |  | Grouped |  |  | V.M |  |  | - |  |  | - |  |  |  |  |
| AoC-Request-Type |  |  | 2055 |  |  | - |  | - |  |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V.M |  |  | - |  |  | - |  |  |  |  |
| AoC-Service |  |  | 2311 |  |  | - |  | - |  |  |  | X |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| AoC-Service-Obligatory-Type |  |  | 2312 |  |  | - |  | - |  |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| AoC-Service-Type |  |  | 2313 |  |  | - |  | - |  |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| AoC-Subscription-Information |  |  | 2314 |  |  | - |  | - |  |  |  | X |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| API-Content |  |  | 1309 |  |  | X |  | - |  |  |  |  | X |  |  | - |  |  | UTF8String |  |  | V |  |  | - |  |  | - |  |  | M |  |
| API-Direction |  |  | 1310 |  |  | X |  | - |  |  |  |  | X |  |  | - |  |  | Enumerated |  |  | V |  |  | - |  |  | - |  |  | M |  |
| API-Identifier |  |  | 1311 |  |  | X |  | - |  |  |  |  | X |  |  | - |  |  | OctetString |  |  | V |  |  | - |  |  | - |  |  | M |  |
| API-Invocation-Timestamp |  |  | 1312 |  |  | X |  | - |  |  |  |  | X |  |  | - |  |  | Time |  |  | V |  |  | - |  |  | - |  |  | M |  |
| API-Network-Service-Node |  |  | 1315 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V |  |  | - |  |  | - |  |  | M |  |
| API-Result-Code |  |  | 1313 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Unsigned32 |  |  | V |  |  | - |  |  | - |  |  | M |  |
| API-Size |  |  | 1314 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Unsigned64 |  |  | V |  |  | - |  |  | - |  |  | M |  |
| APN-Rate-Control |  |  | 3933 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| APN-Rate-Control-Downlink |  |  | 3934 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| APN-Rate-Control-Uplink |  |  | 3935 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Applic-ID |  |  | 1218 |  |  | - |  | - |  |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Application-Port-Identifer |  |  | 3010 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | refer[231] |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Application-Provided-Called-Party-Address |  |  | 837 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Application-Server |  |  | 836 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Application-Server-ID |  |  | 2101 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | refer[223] |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Application-Service-Provider-Identity |  |  | 532 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | refer[214] |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Application-Server-Information |  |  | 850 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Application-Service-Type |  |  | 2102 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | refer[223] |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Application-Session-ID |  |  | 2103 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | refer[223] |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Application-Specific-Data |  |  | 3458 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | OctetString |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Associated-Party-Address |  |  | 2035 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Associated-URI |  |  | 856 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Authorised-QoS |  |  | 849 |  |  | - |  | - |  |  |  | - |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Aux-Applic-Info |  |  | 1219 |  |  | - |  | - |  |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Base-Time-Interval |  |  | 1265 |  |  | - |  | - |  |  |  | - |  |  |  | X |  |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Basic-Service-Code |  |  | 3411 |  |  | - |  | - |  |  |  | X |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Bearer-Capability |  |  | 3412 |  |  | - |  | - |  |  |  | X |  |  |  | - |  |  | OctetString |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Bearer-Service |  |  | 854 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | OctetString |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| BSSID |  |  | 2716 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Called-Asserted-Identity |  |  | 1250 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Called-Identity |  |  | 3916 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Called-Identity-Change |  |  | 3917 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Called-Party-Address |  |  | 832 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Calling-Party-Address |  |  | 831 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Callee-Information |  |  | 565 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | refer [214] |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Carrier-Select-Routing-Information |  |  | 2023 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Cause-Code |  |  | 861 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Integer32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Cellular-Network-Information |  |  | 3924 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | OctetString |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| CG-Address |  |  | 846 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Address |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Change-Condition |  |  | 2037 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | Integer32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Change-Time |  |  | 2038 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Time |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Charge-Reason-Code |  |  | 2118 |  |  | - |  | - |  |  |  | X |  |  |  | X |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Charged-Party |  |  | 857 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Charging-Characteristics-Selection-Mode |  |  | 2066 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Charging-Per-IP-CAN-Session-Indicator |  |  | 4400 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Charging-Rule-Base-Name |  |  | 1004 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | refer [215] |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Civic-Address-Information |  |  | 1305 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V |  |  | - |  |  | - |  |  | M |  |
| Class-Identifier |  |  | 1214 |  |  | - |  | - |  |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Client-Address |  |  | 2018 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Address |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| CN-IP-Multicast-Distribution |  |  | 921 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | refer [207] |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CN-Operator-Selection-Entity |  |  | 3421 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Communication-Failure-Information |  |  | 4300 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | refer [244] |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Content-Class |  |  | 1220 |  |  | - |  | - |  |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Content-Disposition |  |  | 828 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Content-ID |  |  | 2116 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | refer[223] |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Content-Provider-ID |  |  | 2117 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | refer[223] |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Content-Length |  |  | 827 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Content-Size |  |  | 1206 |  |  | - |  | - |  |  |  | X |  |  |  | - |  |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Content-Type |  |  | 826 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Coverage-Info |  |  | 3459 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Coverage-Status |  |  | 3428 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| CP-CIoT-EPS-Optimisation-Indicator |  |  | 3930 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| CPDT-Information |  |  | 3927 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| CSG-Access-Mode |  |  | 2317 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| CSG-Id |  |  | 1437 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | refer[219] |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CSG-Membership-Indication |  |  | 2318 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| CUG-Information |  |  | 2304 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | OctetString |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Current-Tariff |  |  | 2056 |  |  | - |  | - |  |  |  | X |  |  |  | X |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Data-Coding-Scheme |  |  | 2001 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Integer32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| DCD-Information |  |  | 2115 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | refer[223] |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Deferred-Location-Event-Type |  |  | 1230 |  |  | - |  | - |  |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Delivery-Report-Requested |  |  | 1216 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Delivery-Status |  |  | 2104 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | refer[223] |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Destination-Interface |  |  | 2002 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Diagnostics |  |  | 2039 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Integer32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Discoveree-UE-HPLMN-Identifier |  |  | 4402 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V.M |  |  | - |  |  | - |  |  |  |  |
| Discoveree-UE-VPLMN-Identifier |  |  | 4403 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V.M |  |  | - |  |  | - |  |  |  |  |
| Discoverer-UE-HPLMN-Identifier |  |  | 4404 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V.M |  |  | - |  |  | - |  |  |  |  |
| Discoverer-UE-VPLMN-Identifier |  |  | 4405 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V.M |  |  | - |  |  | - |  |  |  |  |
| Domain-Name |  |  | 1200 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| DRM-Content |  |  | 1221 |  |  | - |  | - |  |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Dynamic-Address-Flag |  |  | 2051 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Dynamic-Address-Flag-Extension |  |  | 2068 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Early-Media-Description |  |  | 1272 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Enhanced-Diagnostics |  |  | 3901 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Envelope |  |  | 1266 |  |  | - |  | - |  |  |  | X |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Envelope-End-Time |  |  | 1267 |  |  | - |  | - |  |  |  | X |  |  |  | - |  |  | Time |  |  | V,M |  |  |  |  |  |  |  |  |  |  |
| Envelope-Reporting |  |  | 1268 |  |  | - |  | - |  |  |  | - |  |  |  | X |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Envelope-Start-Time |  |  | 1269 |  |  | - |  | - |  |  |  | X |  |  |  | - |  |  | Time |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| EPDG-Address |  |  | 3425 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | Address |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| EPS-Location-Information |  |  | 1496 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | refer [219] |  |  |  |  |  | - |  |  | - |  |  |  |  |
| Event |  |  | 825 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  |  |  |  |  |  |  |  |  |
| Event-Charging-TimeStamp |  |  | 1258 |  |  | - |  | - |  |  |  | X |  |  |  | - |  |  | Time |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Event-Type |  |  | 823 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Grouped |  |  | V,M |  |  |  |  |  |  |  |  |  |  |
| Expires |  |  | 888 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Unsigned32 |  |  | V,M |  |  |  |  |  |  |  |  |  |  |
| Exposure-Function-API-Information |  |  | 1316 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Grouped |  |  | V |  |  | - |  |  | - |  |  | M |  |
| External-Identifier |  |  | 3111 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | refer [245] |  |  |  |  |  | - |  |  | - |  |  |  |  |
| Feature-List |  |  | 630 |  |  | X |  | X |  |  |  | X |  |  |  | X |  |  | refer [204] |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Feature-List-ID |  |  | 629 |  |  | X |  | X |  |  |  | X |  |  |  | X |  |  | refer [204] |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| FE-Identifier-List |  |  | 4413 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| File-Repair-Supported |  |  | 1224 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Fixed-User-Location-Info |  |  | 2825 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | refer [215] |  |  |  |  |  | - |  |  | - |  |  |  |  |
| Flows |  |  | 510 |  |  | - |  | - |  |  |  | X |  |  |  | - |  |  | refer [214] |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Forwarding-Pending |  |  | 3415 |  |  | - |  | - |  |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  |  |  |  |  |  |  |  |  |
| From-Address |  |  | 2708 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| GGSN-Address |  |  | 847 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Address |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Guaranteed-Bitrate-UL |  |  | 1026 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | refer[215] |  |  |  |  |  | - |  |  | - |  |  |  |  |
| IM-Information |  |  | 2110 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | refer[223] |  |  |  |  |  | - |  |  | - |  |  |  |  |
| IMS-Application-Reference-Identifier |  |  | 2601 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| IMS-Charging-Identifier |  |  | 841 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| IMS-Communication-Service-Identifier |  |  | 1281 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| IMS-Emergency-Indicator |  |  | 2322 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| IMS-Information |  |  | 876 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| IMS-Visited-Network-Identifier |  |  | 2713 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| IMSI-Unauthenticated-Flag |  |  | 2308 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Incoming-Trunk-Group-Id |  |  | 852 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Incremental-Cost |  |  | 2062 |  |  | - |  | - |  |  |  | - |  |  |  | X |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Initial-IMS-Charging-Identifier |  |  | 2321 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Instance-Id |  |  | 3402 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Interface-Id |  |  | 2003 |  |  | - |  | - |  |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Interface-Port |  |  | 2004 |  |  | - |  | - |  |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Interface-Text |  |  | 2005 |  |  | - |  | - |  |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Interface-Type |  |  | 2006 |  |  | - |  | - |  |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Inter-Operator-Identifier |  |  | 838 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Inter-UE-Transfer |  |  | 3902 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| IP-Realm-Default-Indication |  |  | 2603 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| ISUP-Cause |  |  | 3416 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| ISUP-Cause-Diagnostics |  |  | 3422 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | OctetString |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| ISUP-Cause-Location |  |  | 3423 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| ISUP-Cause-Value |  |  | 3424 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| ISUP-Location-Number |  |  | 3414 |  |  | - |  | - |  |  |  | X |  |  |  | - |  |  | OctetString |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Language |  |  | 3914 |  |  | - |  | - |  |  |  | - |  |  |  | X |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Layer-2-Group-ID |  |  | 3429 |  |  | - |  | - |  |  |  | X |  |  |  | - |  |  | OctetString |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| LCS-APN |  |  | 1231 |  |  | - |  | - |  |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| LCS-Client-Dialed-By-MS |  |  | 1233 |  |  | - |  | - |  |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| LCS-Client-External-ID |  |  | 1234 |  |  | - |  | - |  |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| LCS-Client-Id |  |  | 1232 |  |  | - |  | - |  |  |  | X |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| LCS-Client-Name |  |  | 1235 |  |  | - |  | - |  |  |  | X |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| LCS-Client-Type |  |  | 1241 |  |  | - |  | - |  |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| LCS-Data-Coding-Scheme |  |  | 1236 |  |  | - |  | - |  |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| LCS-Format-Indicator |  |  | 1237 |  |  | - |  | - |  |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| LCS-Information |  |  | 878 |  |  | - |  | - |  |  |  | X |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| LCS-Name-String |  |  | 1238 |  |  | - |  | - |  |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| LCS-Requestor-ID |  |  | 1239 |  |  | - |  | - |  |  |  | X |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| LCS-Requestor-ID-String |  |  | 1240 |  |  | - |  | - |  |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Local-GW-Inserted-Indication |  |  | 2604 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Local-Sequence-Number |  |  | 2063 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Location-Estimate |  |  | 1242 |  |  | - |  | - |  |  |  | X |  |  |  | - |  |  | OctetString |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Location-Estimate-Type |  |  | 1243 |  |  | - |  | - |  |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Location-Info |  |  | 3460 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | Grouped |  |  | V,M |  |  |  |  |  |  |  |  |  |  |
| Location-Type |  |  | 1244 |  |  | - |  | - |  |  |  | X |  |  |  | - |  |  | Grouped |  |  | V,M |  |  |  |  |  |  |  |  |  |  |
| Low-Balance-Indication |  |  | 2020 |  |  | - |  | - |  |  |  | - |  |  |  | X |  |  | Enumerated |  |  | V,M |  |  |  |  |  |  |  |  |  |  |
| Low-Priority-Indicator |  |  | 2602 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  |  |  |  |  |  |  |  |  |
| Mandatory-Capability |  |  | 604 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | refer [204] |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Max-Requested-Bandwidth-DL |  |  | 515 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | refer [214] |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Max-Requested-Bandwidth-UL |  |  | 516 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | refer [214] |  |  |  |  |  | - |  |  | - |  |  |  |  |
| Maximum-Detection-Time |  |  | 3131 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | refer [245] |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Maximum-Number-of-Reports |  |  | 3128 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | refer [245] |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MBMS-2G-3G-Indicator |  |  | 907 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | refer [207] |  |  |  |  |  | - |  |  | - |  |  |  |  |
| MBMS-Charged-Party |  |  | 2323 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| MBMS-Data-Transfer-Start |  |  | 929 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | refer [207] |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MBMS-Data-Transfer-Stop |  |  | 930 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | refer [207] |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MBMS-GW-Address |  |  | 2307 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | Address |  |  | V,M |  |  |  |  |  |  |  |  |  |  |
| MBMS-Information |  |  | 880 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| MBMS-Service-Area |  |  | 903 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | refer [207] |  |  |  |  |  | - |  |  | - |  |  |  |  |
| MBMS-Service-Type |  |  | 906 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | refer [207] |  |  |  |  |  | - |  |  | - |  |  |  |  |
| MBMS-Session-Identity |  |  | 908 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | refer [207] |  |  |  |  |  | - |  |  | - |  |  |  |  |
| MBMS-User-Service-Type |  |  | 1225 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Media-Initiator-Flag |  |  | 882 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Media-Initiator-Party |  |  | 1288 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Message-Body |  |  | 889 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Message-Class |  |  | 1213 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Message-ID |  |  | 1210 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Message-Size |  |  | 1212 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Unsigned32 |  |  | V,M |  |  |  |  |  |  |  |  |  |  |
| Message-Type |  |  | 1211 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  |  |  |  |  |  |  |  |  |
| MMBox-Storage-Requested |  |  | 1248 |  |  | - |  | - |  |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  |  |  |  |  |  |  |  |  |
| MM-Content-Type |  |  | 1203 |  |  | - |  | - |  |  |  | X |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| MME-Name |  |  | 2402 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | refer [229] |  |  |  |  |  | - |  |  | - |  |  |  |  |
| MME-Number-for-MT-SMS |  |  | 1645 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | refer [230] |  |  |  |  |  | - |  |  | - |  |  |  |  |
| MME-Realm |  |  | 2408 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | refer [229] |  |  |  |  |  | - |  |  | - |  |  |  |  |
| MMS-Information |  |  | 877 |  |  | - |  | - |  |  |  | X |  |  |  | - |  |  | Grouped |  |  | V,M |  |  |  |  |  |  |  |  |  |  |
| MMTel-Information |  |  | 2030 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Grouped |  |  | V,M |  |  |  |  |  |  |  |  |  |  |
| MMTel-SService-Type |  |  | 2031 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Monitored-PLMN-Identifier |  |  | 3430 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Monitoring-Duration |  |  | 3130 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | refer [245] |  |  |  |  |  | - |  |  | - |  |  |  |  |
| Monitoring-Event-Config-Status |  |  | 3142 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | refer [245] |  |  |  |  |  | - |  |  | - |  |  |  |  |
| Monitoring-Event-Configuration-Activity |  |  | 3919 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | Integer32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Monitoring-Event-Functionality |  |  | 3922 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | Integer32 |  |  | V,M |  |  |  |  |  |  |  |  |  |  |
| Monitoring-Event-Information |  |  | 3921 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Monitoring-Event-Report-Data |  |  | 3920 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Monitoring-Event-Report-Number |  |  | 3923 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Monitoring-Type |  |  | 3127 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | refer [245] |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Monitoring-UE-HPLMN-Identifier |  |  | 3431 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Monitoring-UE-Identifier |  |  | 3432 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  |  |  |  |  |  |  |  |  |
| Monitoring-UE-VPLMN-Identifier |  |  | 3433 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| MONTE-Location-Type |  |  | 3136 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | refer [245] |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MSC-Address |  |  | 3417 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | OctetString |  |  | V,M |  |  |  |  |  |  |  |  |  |  |
| MSISDN |  |  | 701 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | refer [221] |  |  |  |  |  | - |  |  | - |  |  |  |  |
| MTC-IWF-Address |  |  | 3406 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Address |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| NBIFOM-Mode |  |  | 2830 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | refer [215] |  |  |  |  |  | - |  |  | - |  |  |  |  |
| NBIFOM-Support |  |  | 2831 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | refer [215] |  |  |  |  |  | - |  |  | - |  |  |  |  |
| Neighbour-Node-Address |  |  | 2705 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | Address |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Network-Call-Reference-Number |  |  | 3418 |  |  | - |  | - |  |  |  | X |  |  |  | - |  |  | OctetString |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Next-Tariff |  |  | 2057 |  |  | - |  | - |  |  |  | X |  |  |  | X |  |  | Grouped |  |  | V/M |  |  | - |  |  | - |  |  |  |  |
| NIDD-Submission |  |  | 3928 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | Grouped |  |  | V/M |  |  | - |  |  | - |  |  |  |  |
| NNI-Information |  |  | 2703 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| NNI-Type |  |  | 2704 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Node-Functionality |  |  | 862 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  |  |  |  |  |  |  |  |  |
| Node-Id |  |  | 2064 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  |  |  |  |  |  |  |  |  |
| Nr-Cgi |  |  | 1325 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | UTF8String |  |  | V, M |  |  | - |  |  | - |  |  |  |  |
| Number-Of-Diversions |  |  | 2034 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Number-Of-Messages-Sent |  |  | 2019 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Number-Of-Messages-Successfully-Exploded |  |  | 2111 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | refer[223] |  |  |  |  |  | - |  |  | - |  |  |  |  |
| Number-Of-Messages-Successfully-Sent |  |  | 2112 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | refer[223] |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Number-Of-Participants |  |  | 885 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Unsigned32 |  |  | V,M |  |  |  |  |  |  |  |  |  |  |
| Number-Of-Received-Talk-Bursts |  |  | 1282 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Number-Of-Talk-Bursts |  |  | 1283 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Number-Of-UE-Per-Location-Configuration |  |  | 4306 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | refer [244] |  |  |  |  |  | - |  |  | - |  |  |  |  |
| Number-Of-UE-Per-Location-Report |  |  | 4307 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | refer [244] |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Number-Portability-Routing-Information |  |  | 2024 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  |  |  |  |  |  |  |  |  |
| Offline-Charging |  |  | 1278 |  |  | - |  | - |  |  |  | - |  |  |  | X |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Online-Charging-Flag |  |  | 2303 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Optional-Capability |  |  | 605 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | refer [204] |  |  |  |  |  | - |  |  | - |  |  |  |  |
| Origin-App-Layer-User-Id |  |  | 3600 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | refer [238] |  |  |  |  |  | - |  |  | - |  |  |  |  |
| Originating-IOI |  |  | 839 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Originator |  |  | 864 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Originator-Address |  |  | 886 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Originator-Interface |  |  | 2009 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Originator-Received-Address |  |  | 2027 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Originator-SCCP-Address |  |  | 2008 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Address |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Outgoing-Session-Id |  |  | 2320 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Outgoing-Trunk-Group-Id |  |  | 853 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Participant-Access-Priority |  |  | 1259 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Participant-Action-Type |  |  | 2049 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Participant-Group |  |  | 1260 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Participants-Involved |  |  | 887 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| PC3-Control-Protocol-Cause |  |  | 3434 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Integer32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| PC3-EPC-Control-Protocol-Cause |  |  | 3435 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Integer32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| PC5-Radio-Technology |  |  | 1300 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V |  |  | - |  |  | - |  |  | M |  |
| PDN-Connection-Charging-ID |  |  | 2050 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| PDP-Address |  |  | 1227 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Address |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| PDP-Address-Prefix-Length |  |  | 2606 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| PDP-Context-Type |  |  | 1247 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Play-Alternative |  |  | 3913 |  |  | - |  | - |  |  |  | - |  |  |  | X |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| PoC-Change-Condition |  |  | 1261 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| PoC-Change-Time |  |  | 1262 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | Time |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| PoC-Controlling-Address |  |  | 858 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| PoC-Event-Type |  |  | 2025 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| PoC-Group-Name |  |  | 859 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| PoC-Information |  |  | 879 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| PoC-Server-Role |  |  | 883 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| PoC-Session-Id |  |  | 1229 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| PoC-Session-Initiation-type |  |  | 1277 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| PoC-Session-Type |  |  | 884 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| PoC-User-Role |  |  | 1252 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| PoC-User-Role-IDs |  |  | 1253 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  |  |  |  |  |  |  |  |  |
| PoC-User-Role-Info-Units |  |  | 1254 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  |  |  |  |  |  |  |  |  |
| Positioning-Data |  |  | 1245 |  |  | - |  | - |  |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  |  |  |  |  |  |  |  |  |
| Preferred-AoC-Currency |  |  | 2315 |  |  | - |  | - |  |  |  | X |  |  |  | - |  |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Presence-Reporting-Area-Identifier |  |  | 2821 |  |  | X |  | - |  |  |  | X |  |  |  | X |  |  | refer [215] |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Presence-Reporting-Area-Information |  |  | 2822 |  |  | X |  | - |  |  |  | X |  |  |  | X |  |  | refer [215] |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Presence-Reporting-Area-Elements-List |  |  | 2820 |  |  | X |  | - |  |  |  | X |  |  |  | X |  |  | refer [215] |  |  |  |  |  | - |  |  | - |  |  |  |  |
| Presence-Reporting-Area-Node |  |  | 2855 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | refer [215] |  |  |  |  |  | - |  |  | - |  |  |  |  |
| Presence-Reporting-Area-Status |  |  |  | 2823 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | refer [215] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| Priority |  |  |  | 1209 |  |  | - |  |  | - |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| Priority-Indication |  |  |  | 3006 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | refer [231] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| Priority-Level |  |  |  | 1046 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | refer [215] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| Privacy-Indicator |  |  |  | 3915 |  |  | - |  |  | - |  |  | - |  |  |  | X |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| ProSe-3rd-Party-Application-ID |  |  |  | 3440 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| ProSe-App-Id |  |  |  | 3811 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | refer [239] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ProSe-Direct-Communication-Reception-Data-Container |  |  |  | 3461 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| ProSe-Direct-Communication- Transmission-Data-Container |  |  |  | 3441 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| ProSe-Direct-Discovery-Model |  |  |  | 3442 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| ProSe-Event-Type |  |  |  | 3443 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| ProSe-Function-ID |  |  |  | 3602 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | refer [238] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| ProSe-Function-IP-Address |  |  |  | 3444 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | Address |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| ProSe-Function-PLMN-Identifier |  |  |  | 3457 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| ProSe-Functionality |  |  |  | 3445 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| ProSe-Group-IP-Multicast-Address |  |  |  | 3446 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | Address |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| ProSe-Information |  |  |  | 3447 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| ProSe-Range-Class |  |  |  | 3448 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| ProSe-Reason-For-Cancellation |  |  |  | 3449 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| ProSe-Request-Timestamp |  |  |  | 3450 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | Time |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| ProSe-Role-Of-UE |  |  |  | 3451 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| ProSe-Source-IP-Address |  |  |  | 3452 |  |  | - |  |  | - |  |  | X |  |  |  | - |  |  | Address |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| ProSe-Target-Layer-2-ID |  |  |  | 4410 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | OctetString |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| ProSe-UE-ID |  |  |  | 3453 |  |  | - |  |  | - |  |  | X |  |  |  | - |  |  | OctetString |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| ProSe-UE-to-Network-Relay-UE-ID |  |  |  | 4409 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | OctetString |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| ProSe-Validity-Timer |  |  |  | 3815 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | refer [239] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| Proximity-Alert-Indication |  |  |  | 3454 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Proximity-Alert-Timestamp |  |  |  | 3455 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | Time |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Proximity-Cancellation-Timestamp |  |  |  | 3456 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | Time |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| PS-Append-Free-Format-Data |  |  |  | 867 |  |  | X |  |  | - |  |  | - |  |  |  | X |  |  | Enumerated |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| PS-Free-Format-Data |  |  |  | 866 |  |  | X |  |  | - |  |  | - |  |  |  | X |  |  | OctetString |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| PS-Furnish-Charging-Information |  |  |  | 865 |  |  | X |  |  | - |  |  | - |  |  |  | X |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| PS-Information |  |  |  | 874 |  |  | X |  |  | - |  |  | X |  |  |  | X |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| PSCell-Info |  |  |  | 1324 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| QoS-Information |  |  |  | 1016 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | refer [215] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| QoS-Class-Identifier |  |  |  | 1028 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | refer [215] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| Quota-Consumption-Time |  |  |  | 881 |  |  | - |  |  | - |  |  | - |  |  |  | X |  |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Quota-Holding-Time |  |  |  | 871 |  |  | - |  |  | - |  |  | - |  |  |  | X |  |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Quota-Indicator |  |  |  | 3912 |  |  | - |  |  | - |  |  | - |  |  |  | X |  |  | Enumerated |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| Radio-Frequency |  |  |  | 3462 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | OctetString |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| Radio-Parameter-Set-Info |  |  |  | 3463 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | Grouped |  |  | V.M |  |  | - |  |  | - |  |  |  |  |  |
| Radio-Parameter-Set-Values |  |  |  | 3464 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | OctetString |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Radio-Resources-Indicator |  |  |  | 3465 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | Integer32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| RAI |  |  |  | 909 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | refer [207] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| RAN-End-Time |  |  |  | 1301 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | Time |  |  | V |  |  | - |  |  | - |  |  | M |  |  |
| RAN-NAS-Release-Cause |  |  |  | 2819 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | refer [215] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAN-Secondary-RAT-Usage-Report |  |  |  | 1302 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | Grouped |  |  | V |  |  | - |  |  | - |  |  | M |  |  |
| RAN-Start-Time |  |  |  | 1303 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | Time |  |  | V |  |  | - |  |  | - |  |  | M |  |  |
| Rate-Control-Max-Message-Size |  |  |  | 3937 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Rate-Control-Max-Rate |  |  |  | 3938 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Rate-Control-Time-Unit |  |  |  | 3939 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Rate-Element |  |  |  | 2058 |  |  | - |  |  | - |  |  | X |  |  |  | X |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Reachability-Information |  |  |  | 3140 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | refer [245] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| Read-Reply-Report-Requested |  |  |  | 1222 |  |  | - |  |  | - |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Real-Time-Tariff-Information |  |  |  | 2305 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  |  |  |  |  |  |  |
| Reason-Header |  |  |  | 3401 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  |  |  |  |  |  |  |
| Received-Talk-Burst-Time |  |  |  | 1284 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Received-Talk-Burst-Volume |  |  |  | 1285 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | Unsigned32 |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| Recipient-Address |  |  |  | 1201 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Recipient-Info |  |  |  | 2026 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Recipient-Received-Address |  |  |  | 2028 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Recipient-SCCP-Address |  |  |  | 2010 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | Address |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Reference-Number |  |  |  | 3007 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | refer [231] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| Refund-Information |  |  |  | 2022 |  |  | - |  |  | - |  |  | X |  |  |  | X |  |  | OctetString |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Related-Change-Condition-Information |  |  |  | 3925 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Related-Trigger |  |  |  | 3926 |  |  | - |  |  | - |  |  | X |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Related-IMS-Charging-Identifier |  |  |  | 2711 |  |  | X |  |  | - |  |  | X |  |  |  |  |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Related-IMS-Charging-Identifier-Node |  |  |  | 2712 |  |  | X |  |  | - |  |  | X |  |  |  |  |  |  | Address |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Relationship-Mode |  |  |  | 2706 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Relay-IP-address |  |  |  | 4411 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | Address |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Remaining-Balance |  |  |  | 2021 |  |  | - |  |  | - |  |  | - |  |  |  | X |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Reply-Applic-ID |  |  |  | 1223 |  |  | - |  |  | - |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| Reply-Path-Requested |  |  |  | 2011 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Reporting-Reason |  |  |  | 872 |  |  | - |  |  | - |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| Requested-Party-Address |  |  |  | 1251 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Requested-PLMN-Identifier |  |  |  | 3436 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Requesting-EPUID |  |  |  | 3816 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | refer [239] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| Requestor-PLMN-Identifier |  |  |  | 3437 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Required-MBMS-Bearer-Capabilities |  |  |  | 901 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | refer [207] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Role-Of-Node |  |  |  | 829 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Role-Of-ProSe-Function |  |  |  | 3438 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| Route-Header-Received |  |  |  | 3403 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| Route-Header-Transmitted |  |  |  | 3404 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| RRC-Cause-Counter |  |  |  | 4318 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | refer [244] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| Scale-Factor |  |  |  | 2059 |  |  | - |  |  | - |  |  | X |  |  |  | X |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| SCEF-Address |  |  |  | 1317 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | Address |  |  | V |  |  | - |  |  | - |  |  | M |  |  |
| SCEF-ID |  |  |  | 3125 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | refer [245] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| SCEF-Reference-ID |  |  |  | 3124 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | refer [245] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| SCS-Address |  |  |  | 3941 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | Address |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| SCS-AS-Address |  |  |  | 3940 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| SCS-Realm |  |  |  | 3942 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | DiameterIdentity |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| SDP-Answer-Timestamp |  |  |  | 1275 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | Time |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| SDP-Media-Component |  |  |  | 843 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| SDP-Media-Description |  |  |  | 845 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| SDP-Media-Name |  |  |  | 844 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| SDP-Offer-Timestamp |  |  |  | 1274 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | Time |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| SDP-Session-Description |  |  |  | 842 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| SDP-TimeStamps |  |  |  | 1273 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | Grouped |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| SDP-Type |  |  |  | 2036 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Secondary-RAT-Type |  |  |  | 1304 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | OctetString |  |  | V |  |  | - |  |  | - |  |  | M |  |  |
| Serving-Node |  |  |  | 2401 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | refer [231] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| Serving-Node-Identity |  |  |  | 3929 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | DiameterIdentity |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Served-Party-IP-Address |  |  |  | 848 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | Address |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Server-Capabilities |  |  |  | 603 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | refer [204] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| Server-Name |  |  |  | 602 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | refer [204] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| Service-Data-Container |  |  |  | 2040 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Service-Generic-Information |  |  |  | 1256 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | refer [223] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Service-Id |  |  |  | 855 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Service-Information |  |  |  | 873 |  |  | X |  |  | - |  |  | X |  |  |  | X |  |  | Grouped |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| Service-Mode |  |  |  | 2032 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Service-Specific-Data |  |  |  | 863 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Service-Specific-Info |  |  |  | 1249 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Service-Specific-Type |  |  |  | 1257 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Serving-Node-Type |  |  |  | 2047 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Serving-PLMN-Rate-Control |  |  |  | 4310 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | refer [244] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| Session-Direction |  |  |  | 2707 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Session-Priority |  |  |  | 650 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | refer [204] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| SGi-PtP-Tunnelling-Method |  |  |  | 3931 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| SGSN-Address |  |  |  | 1228 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | Address |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| SGW-Address |  |  |  | 2067 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | Address |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| SGW-Change |  |  |  | 2065 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| SIP-Method |  |  |  | 824 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| SIP-Request-Timestamp |  |  |  | 834 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | Time |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| SIP-Request-Timestamp-Fraction |  |  |  | 2301 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| SIP-Response-Timestamp |  |  |  | 835 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | Time |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| SIP-Response-Timestamp-Fraction |  |  |  | 2302 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| SM-Device-Trigger-Indicator |  |  |  | 3407 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| SM-Device-Trigger-Information |  |  |  | 3405 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| SM-Discharge-Time |  |  |  | 2012 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | Time |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| SM-Message-Type |  |  |  | 2007 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| SM-Protocol-ID |  |  |  | 2013 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | OctetString |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| SM-Sequence-Number |  |  |  | 3408 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | Unsigned32 |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| SM-Service-Type |  |  |  | 2029 |  |  | - |  |  | - |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| SM-Status |  |  |  | 2014 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | OctetString |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| SM-User-Data-Header |  |  |  | 2015 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | OctetString |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| SMS-Information |  |  |  | 2000 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| SMS-Node |  |  |  | 2016 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| SMS-Result |  |  |  | 3409 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| SMSC-Address |  |  |  | 2017 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | Address |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Sponsor-Identity |  |  |  | 531 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | refer [214] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| SSID |  |  |  | 1524 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | refer [237] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| Start-of-Charging |  |  |  | 3419 |  |  | - |  |  | - |  |  | X |  |  |  | - |  |  | Time |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Start-Time |  |  |  | 2041 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | Time |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Status- AS-Code |  |  |  | 2702 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Stop-Time |  |  |  | 2042 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | Time |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| Submission-Time |  |  |  | 1202 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | Time |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Subscriber-Role |  |  |  | 2033 |  |  | - |  |  | - |  |  | - |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Supported-Features |  |  |  | 628 |  |  | X |  |  | X |  |  | X |  |  |  | X |  |  | refer [204] |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Supplementary-Service |  |  |  | 2048 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | Grouped |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| TAD-Identifier |  |  |  | 2717 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Talk-Burst-Exchange |  |  |  | 1255 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | Grouped |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| Talk-Burst-Time |  |  |  | 1286 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Talk-Burst-Volume |  |  |  | 1287 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Target-App-Layer-User-Id |  |  |  | 3601 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | refer[238] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| Target-IP-Address |  |  |  | 4412 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | Address |  |  | VM |  |  | - |  |  | - |  |  | - |  |  |
| Tariff-Information |  |  |  | 2060 |  |  | X |  |  | - |  |  | X |  |  |  | X |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Tariff-XML |  |  |  | 2306 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| TDF-IP-Address |  |  |  | 1091 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | refer [215] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| Teleservice |  |  |  | 3413 |  |  | - |  |  | - |  |  | X |  |  |  | - |  |  | OctetString |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Terminal-Information |  |  |  | 1401 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | refer [219] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| Terminating-IOI |  |  |  | 840 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Time-First-Reception |  |  |  | 3466 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | Time |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Time-First-Transmission |  |  |  | 3467 |  |  | X |  | - |  |  |  | - |  |  |  | - |  |  | Time |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Time-First-Usage |  |  |  | 2043 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | Time |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| Time-Indicator |  |  |  | 3911 |  |  | - |  |  | - |  |  | - |  |  |  | X |  |  | Unsigned32 |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| Time-Last-Usage |  |  |  | 2044 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | Time |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Time-Quota-Mechanism |  |  |  | 1270 |  |  | - |  |  | - |  |  | - |  |  |  | X |  |  | Grouped |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| Time-Quota-Threshold |  |  |  | 868 |  |  | - |  |  | - |  |  | - |  |  |  | X |  |  | Unsigned32 |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| Time-Quota-Type |  |  |  | 1271 |  |  | - |  |  | - |  |  | - |  |  |  | X |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Time-Stamps |  |  |  | 833 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Time-Usage |  |  |  | 2045 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Time-Window |  |  |  | 3818 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | refer [239] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| TLTRI |  |  |  | 1318 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | Unsigned32 |  |  | V |  |  | - |  |  | - |  |  | M |  |  |
| TMGI |  |  |  | 900 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | refer [207] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| Token-Text |  |  |  | 1215 |  |  | - |  |  | - |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Total-Number-Of-Messages-Exploded |  |  |  | 2113 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | refer [223] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| Total-Number-Of-Messages-Sent |  |  |  | 2114 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | refer [223] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| Traffic-Data-Volumes |  |  |  | 2046 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Traffic-Steering-Policy-Identifier-DL |  |  |  | 2836 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | refer [215] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| Traffic-Steering-Policy-Identifier-UL |  |  |  | 2837 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | refer [215] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| Transcoder-Inserted-Indication |  |  |  | 2605 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Transit-IOI-List |  |  |  | 2701 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| Transmitter-Info |  |  |  | 3468 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | Grouped |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| Trigger |  |  |  | 1264 |  |  | - |  |  | - |  |  | X |  |  |  | X |  |  | Grouped |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| Trigger-Type |  |  |  | 870 |  |  | - |  |  | - |  |  | X |  |  |  | X |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Trunk-Group-Id |  |  |  | 851 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| TWAG-Address |  |  |  | 3903 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | Address |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| TWAN-User-Location-Info |  |  |  | 2714 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Type-Number |  |  |  | 1204 |  |  | - |  |  | - |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| UDP-Source-Port |  |  |  | 2806 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | refer [215] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UE-Local-IP-Address |  |  |  | 2805 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | refer [215] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UE-Reachability-Configuration |  |  |  | 3129 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | refer [245] |  |  |  |  |  | - |  |  | - |  |  |  |  |  |
| UNI-PDU-CP-Only-Flag |  |  |  | 3932 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Unit-Cost |  |  |  | 2061 |  |  | - |  |  | - |  |  | X |  |  |  | X |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Unit-Quota-Threshold |  |  |  | 1226 |  |  | - |  |  | - |  |  | - |  |  |  | X |  |  | Unsigned32 |  |  | V,M |  |  |  |  |  |  |  |  |  |  |  |
| Unused-Quota-Timer |  |  |  | 4407 |  |  | - |  |  | - |  |  | X |  |  |  | X |  |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |  |
| Usage-Information-Report-Sequence-Number |  |  |  | 3439 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | Integer32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| User-CSG-Information |  |  |  | 2319 |  |  | X |  |  | - |  |  | X |  |  |  | X |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| User-Data |  |  |  | 606 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | refer [204] |  |  |  |  |  | - |  |  | - |  |  |  |  |
| User-Location-Info-Time |  |  |  | 2812 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | refer [215] |  |  |  |  |  | - |  |  | - |  |  |  |  |
| User-Participating-Type |  |  |  | 1279 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | Enumerated |  |  | V,M |  |  |  |  |  |  |  |  |  |  |
| User–Session-Id |  |  |  | 830 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| UWAN-User-Location-Info |  |  |  | 3918 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | Grouped |  |  | V,M |  |  |  |  |  |  |  |  |  |  |
| VAS-Id |  |  |  | 1102 |  |  | - |  |  | - |  |  | X |  |  |  | - |  |  | refer [213] |  |  |  |  |  | - |  |  | - |  |  |  |  |
| Variable-Part |  |  |  | 3907 |  |  | - |  |  | - |  |  | - |  |  |  | X |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Variable-Part-Order |  |  |  | 3908 |  |  | - |  |  | - |  |  | - |  |  |  | X |  |  | Unsigned32 |  |  | V,M |  |  |  |  |  |  |  |  |  |  |
| Variable-Part-Type |  |  |  | 3909 |  |  | - |  |  | - |  |  | - |  |  |  | X |  |  | Unsigned32 |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Variable-Part-Value |  |  |  | 3910 |  |  | - |  |  | - |  |  | - |  |  |  | X |  |  | UTF8String |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| VASP-Id |  |  |  | 1101 |  |  | - |  |  | - |  |  | X |  |  |  | - |  |  | refer [213] |  |  |  |  |  | - |  |  | - |  |  |  |  |
| VCS-Information |  |  |  | 3410 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Visited-PLMN-Id |  |  |  | 1407 |  |  | X |  |  | - |  |  | - |  |  |  | - |  |  | refer [230] |  |  |  |  |  | - |  |  | - |  |  |  |  |
| VLR-Number |  |  |  | 3420 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | OctetString |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| VoLTE-Information |  |  |  | 1323 |  |  | X |  |  | - |  |  | X |  |  |  | - |  |  | Grouped |  |  | V,M |  |  | - |  |  | - |  |  |  |  |
| Volume-Quota-Threshold |  |  |  | 869 |  |  | - |  |  | - |  |  | - |  |  |  | X |  |  | Unsigned32 |  |  | V,M |  |  |  |  |  |  |  |  |  |  |
| WLAN-Link-Layer-Id |  |  |  | 3821 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | refer [239] |  |  |  |  |  | - |  |  | - |  |  |  |  |
| WLAN-Operator-Id |  |  |  | 1306 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | Grouped |  |  | V |  |  | - |  |  | - |  |  | M |  |
| WLAN-Operator-Name |  |  |  | 1307 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V |  |  | - |  |  | - |  |  | M |  |
| WLAN-PLMN-Id |  |  |  | 1308 |  |  | X |  | - |  |  |  | X |  |  |  | - |  |  | UTF8String |  |  | V |  |  | - |  |  | - |  |  | M |  |

NOTE:	Per TS 29.230 [206] the following AVP codes are reserved for the present document:
Before Rel-8: 800 to 899, 1200 to 1299.
Rel-8: 2000 to 2199.
Rel-9: 2300 to 2399. 
Rel-10: 2600 to 2699. 
Rel-11: 2700 to 2799. 
Rel-12: 3400 to 3499. 
Rel-13: 3900 to 3999.
Rel-14: 4400 to 4499.

Rel-15: 1300 to 1399.

Next change

### 7.2.158	PS-Information AVP

The PS-Information AVP (AVP code 874) is of type Grouped. Its purpose is to allow the transmission of additional PS service specific information elements.

It has the following ABNF grammar:

PS-Information :: = 	< AVP Header: 874>

* [ Supported-Features ]

[ 3GPP-Charging-Id ]

[ PDN-Connection-Charging-ID ]
   [ Node-Id ]
   [ 3GPP-PDP-Type ]
* [ PDP-Address ]
   [ PDP-Address-Prefix-Length ]
   [ Dynamic-Address-Flag ]
   [ Dynamic-Address-Flag-Extension ]
   [ QoS-Information ]
 * [ SGSN-Address ]
 * [ GGSN-Address ]

* [ TDF-IP-Address ]

* [ SGW-Address ]

* [ ePDG-Address ]

* [ TWAG-Address ]
   [ CG-Address ]
   [ Serving-Node-Type ]
   [ SGW-Change ]
   [ 3GPP-IMSI-MCC-MNC ]
   [ IMSI-Unauthenticated-Flag ]
   [ 3GPP-GGSN-MCC-MNC ]
   [ 3GPP-NSAPI ]
   [ Called-Station-Id ]
   [ 3GPP-Session-Stop-Indicator ]
   [ 3GPP-Selection-Mode ]
   [ 3GPP-Charging-Characteristics ]
   [ Charging-Characteristics-Selection-Mode ]
   [ 3GPP-SGSN-MCC-MNC ]
   [ 3GPP-MS-TimeZone ]

[ Charging-Rule-Base-Name ] 
   [ ADC-Rule-Base-Name ]

[ 3GPP-User-Location-Info ]

[ User-Location-Info-Time ]

[ User-CSG-Information ]

* [ Presence-Reporting-Area-Information ]

[ 3GPP2-BSID ]

[ TWAN-User-Location-Info ]
	[ UWAN-User-Location-Info ]
   [ 3GPP-RAT-Type ]
   [ PS-Furnish-Charging-Information ]
   [ PDP-Context-Type ] 
   [ Offline-Charging ]

* [ Traffic-Data-Volumes ]

* [ Service-Data-Container ]

[ User-Equipment-Info ]

[ Terminal-Information ]

[ Start-Time ]

[ Stop-Time ]

[ Change-Condition ]

[ Diagnostics ]

[ Low-Priority-Indicator ]

[ NBIFOM-Mode ]

[ NBIFOM-Support ]

[ MME-Number-for-MT-SMS ]

[ MME-Name ]

[ MME-Realm ] 
	[ Logical-Access-ID ]
	[ Physical-Access-ID ]

[ Fixed-User-Location-Info ]

[ CN-Operator-Selection-Entity ]

[ Enhanced-Diagnostics ]

[ SGi-PtP-Tunnelling-Method ]

[ CP-CIoT-EPS-Optimisation-Indicator ]

[ UNI-PDU-CP-Only-Flag ]

[ Serving-PLMN-Rate-Control ]

[ APN-Rate-Control ]

[ Charging-Per-IP-CAN-Session-Indicator ]

[ RRC-Cause-Counter ]

[ 3GPP-PS-Data-Off-Status ]

[ SCS-AS-Address ]

[ Unused-Quota-Timer ]

* [ RAN-Secondary-RAT-Usage-Report ]

[ PSCell-Info ]

Next change

### 7.2.X			Nr-Cgi AVP

The Nr-Cgi AVP (AVP code 1325) is of type UTF8String and includes NR Global Cell Identity, with the format as specified in clause 19.6A of TS 23.003 [224].

Next change

### 7.2.Y			PSCell-Info AVP

The PSCell-Info AVP (AVP code 1324) is of type Grouped and provides primary of Secondary Cell Group (SCG)) cell information.

It has the following ABNF grammar:

PSCell-Info :: = < AVP Header: 1324>

[ Nr-Cgi ]

End of changes