3GPP TSG-SA5 Meeting #138-e 	S5-214075

# e-meeting, 23 - 31 August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.298 | CR | 0873 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Inclusive language review |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2021-07-30 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Review conducted based on LS to SA5 on Use of Inclusive Language in 3GPP specifications (SP-201042/S5‑211029) |  |  |  |  |  |  |  |  |
| Summary of change: |  | IMEI status: - "Greylisted" replaced by "Tracklisted" - "Blacklisted" replaced by "Blocklisted" - "Non-whitelisted" replaced by "Non-Allowlisted" Corresponding ASN.1 change |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Use of Non-inclusive language is non compliant with 3GPP dratfing Rules |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.2.1.34, 5.2.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

##### 5.1.2.1.34	IMEI Status

This field contains the result of the IMEI checking procedure:

-	 Tracklisted;

-	 Blocklisted;

-	 Non-Allowlisted.

Next change

#### 5.2.2.1	CS domain CDRs

This subclause contains the abstract syntax definitions that are specific to the CDR types defined in TS 32.250 [10].

.$CSChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) csChargingDataTypes (1) asn1Module (0) version2 (1)}  DEFINITIONS IMPLICIT TAGS	::=

BEGIN

-- EXPORTS everything

IMPORTS

AE-title,

BCDDirectoryNumber,

CallDuration,

CalledNumber,

CallingNumber,

CauseForTerm,

CellId,

ChargeIndicator,

Diagnostics,

LCSCause,

LCSClientIdentity,

LCSQoSInfo,

LevelOfCAMELService,

LocationAreaAndCell,

LocationAreaCode,

ManagementExtensions,

MCC-MNC,

MessageReference,

MSCAddress,

MscNo,

MSISDN,

NodeAddress,

ObjectInstance,

PositioningData,

RecordingEntity,

RecordType,

SMSResult,

SmsTpDestinationNumber,

SystemType,

TimeStamp

FROM GenericChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) genericChargingDataTypes (0) asn1Module (0) version2 (1)}

BearerServiceCode

FROM MAP-BS-Code {itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-BS-Code (20)  version18 (18) }

-- from TS 29.002 [214]

TeleserviceCode

FROM MAP-TS-Code {itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-TS-Code (19)  version18 (18) }

-- from TS 29.002 [214]

SS-Code

FROM MAP-SS-Code { itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-SS-Code (15)  version18 (18) }

-- from TS 29.002 [214]

MOLR-Type

FROM SS-DataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Access (2) modules (3) ss-DataTypes (2)  version14 (14)}

-- from TS 24.080 [209]

DefaultCallHandling,

DefaultSMS-Handling,

NotificationToMSUser,

ServiceKey

FROM MAP-MS-DataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-MS-DataTypes (11)  version18 (18) }

-- from TS 29.002 [214]

CallReferenceNumber,

NumberOfForwarding

FROM MAP-CH-DataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-CH-DataTypes (13)  version18 (18) }

-- from TS 29.002 [214]

AddressString,

BasicServiceCode,

IMEI,

IMSI,

ISDN-AddressString

FROM MAP-CommonDataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-CommonDataTypes (18)  version18 (18) }

-- from TS 29.002 [214]

Ext-GeographicalInformation,

LCSClientType,

LCS-Priority,

LocationType

FROM MAP-LCS-DataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-LCS-DataTypes (25)  version18 (18) }

-- from TS 29.002 [214]

IMS-Charging-Identifier,

InterOperatorIdentifierList,

TransitIOILists

FROM IMSChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) imsChargingDataTypes (4) asn1Module (0) version2 (1)}

BasicService

FROM Basic-Service-Elements	{itu-t(0) identified-organization (4) etsi (0) 196 basic-service-elements (8) }

-- from "Digital Subscriber Signalling System No. one (DSS1) protocol" ETS 300 196 [310]

DestinationRoutingAddress

FROM CAP-datatypes { itu-t(0) identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) cap-datatypes (52) version8 (7) }

-- from TS 29.078 [217]

;

--

--  CS CALL AND EVENT RECORDS

--

CSRecord	::= CHOICE

--

-- Record values 0..21 are circuit switch specific

--

{

moCallRecord				[0] MOCallRecord,

mtCallRecord				[1] MTCallRecord,

roamingRecord			[2] RoamingRecord,

incGatewayRecord			[3] IncGatewayRecord,

outGatewayRecord			[4] OutGatewayRecord,

transitRecord			[5] TransitCallRecord,

moSMSRecord				[6] MOSMSRecord,

mtSMSRecord				[7] MTSMSRecord,

moSMSIWRecord			[8] MOSMSIWRecord,

mtSMSGWRecord			[9] MTSMSGWRecord,

ssActionRecord			[10] SSActionRecord,

hlrIntRecord				[11] HLRIntRecord,

locUpdateHLRRecord		[12] LocUpdateHLRRecord,

locUpdateVLRRecord		[13] LocUpdateVLRRecord,

commonEquipRecord		[14] CommonEquipRecord,

recTypeExtensions		[15] ManagementExtensions,

termCAMELRecord			[16] TermCAMELRecord,

mtLCSRecord				[17] MTLCSRecord,

moLCSRecord				[18] MOLCSRecord,

niLCSRecord				[19] NILCSRecord,

mSCsRVCCRecord			[20] MSCsRVCCRecord,

mMTRFRecord				[21] MTRFRecord,

iCSRegisterRecord		[22] ICSregisterRecord

}

MOCallRecord	::= SET

{

recordType					[0] RecordType,

servedIMSI					[1] IMSI OPTIONAL,

servedIMEI					[2] IMEI OPTIONAL,

servedMSISDN					[3] MSISDN OPTIONAL,

callingNumber				[4] CallingNumber OPTIONAL,

calledNumber					[5] CalledNumber OPTIONAL,

translatedNumber				[6] TranslatedNumber OPTIONAL,

connectedNumber				[7] ConnectedNumber OPTIONAL,

roamingNumber				[8] RoamingNumber OPTIONAL,

recordingEntity				[9] RecordingEntity,

mscIncomingTKGP				[10] TrunkGroup OPTIONAL,

mscOutgoingTKGP				[11] TrunkGroup OPTIONAL,

location						[12] LocationAreaAndCell OPTIONAL,

changeOfLocation				[13] SEQUENCE OF LocationChange OPTIONAL,

basicService					[14] BasicServiceCode OPTIONAL,

transparencyIndicator		[15] TransparencyInd OPTIONAL,

changeOfService				[16] SEQUENCE OF ChangeOfService OPTIONAL,

supplServicesUsed			[17] SEQUENCE OF SuppServiceUsed OPTIONAL,

aocParameters				[18] AOCParameters OPTIONAL,

changeOfAOCParms				[19] SEQUENCE OF AOCParmChange OPTIONAL,

msClassmark					[20] Classmark OPTIONAL,

changeOfClassmark			[21] ChangeOfClassmark OPTIONAL,

seizureTime					[22] TimeStamp OPTIONAL,

answerTime					[23] TimeStamp OPTIONAL,

releaseTime					[24] TimeStamp OPTIONAL,

callDuration					[25] CallDuration,

dataVolume					[26] DataVolume OPTIONAL,

radioChanRequested			[27] RadioChanRequested OPTIONAL,

radioChanUsed				[28] TrafficChannel OPTIONAL,

changeOfRadioChan			[29] ChangeOfRadioChannel OPTIONAL,

causeForTerm					[30] CauseForTerm,

diagnostics					[31] Diagnostics OPTIONAL,

callReference				[32] CallReferenceNumber,

sequenceNumber				[33] INTEGER OPTIONAL,

additionalChgInfo			[34] AdditionalChgInfo OPTIONAL,

recordExtensions				[35] ManagementExtensions OPTIONAL,

gsm-SCFAddress				[36] Gsm-SCFAddress OPTIONAL,

serviceKey					[37] ServiceKey OPTIONAL,

networkCallReference			[38] NetworkCallReference OPTIONAL,

mSCAddress					[39] MSCAddress OPTIONAL,

cAMELInitCFIndicator			[40] CAMELInitCFIndicator OPTIONAL,

defaultCallHandling			[41] DefaultCallHandling OPTIONAL,

hSCSDChanRequested			[42] NumOfHSCSDChanRequested OPTIONAL,

hSCSDChanAllocated			[43] NumOfHSCSDChanAllocated OPTIONAL,

changeOfHSCSDParms			[44] SEQUENCE OF HSCSDParmsChange OPTIONAL,

fnur							[45] Fnur OPTIONAL,

aiurRequested				[46] AiurRequested OPTIONAL,

chanCodingsAcceptable		[47] SEQUENCE OF ChannelCoding OPTIONAL,

chanCodingUsed				[48] ChannelCoding OPTIONAL,

speechVersionSupported		[49] SpeechVersionIdentifier OPTIONAL,

speechVersionUsed			[50] SpeechVersionIdentifier OPTIONAL,

numberOfDPEncountered		[51] INTEGER OPTIONAL,

levelOfCAMELService			[52] LevelOfCAMELService OPTIONAL,

freeFormatData				[53] FreeFormatData OPTIONAL,

cAMELCallLegInformation		[54] SEQUENCE OF CAMELInformation OPTIONAL,

freeFormatDataAppend			[55] BOOLEAN OPTIONAL,

defaultCallHandling-2		[56] DefaultCallHandling OPTIONAL,

gsm-SCFAddress-2				[57] Gsm-SCFAddress OPTIONAL,

serviceKey-2					[58] ServiceKey OPTIONAL,

freeFormatData-2				[59] FreeFormatData OPTIONAL,

freeFormatDataAppend-2		[60] BOOLEAN OPTIONAL,

systemType					[61] SystemType OPTIONAL,

rateIndication				[62] RateIndication OPTIONAL,

locationRoutNum				[63] LocationRoutingNumber OPTIONAL,

lrnSoInd						[64] LocationRoutingNumberSourceIndicator OPTIONAL,

lrnQuryStatus				[65] LocationRoutingNumberQueryStatus OPTIONAL,

jIPPara						[66] JurisdictionInformationParameter OPTIONAL,

jIPSoInd						[67] JurisdictionInformationParameterSourceIndicator OPTIONAL,

jIPQuryStatus				[68] JurisdictionInformationParameterQueryStatus OPTIONAL,

partialRecordType			[69] PartialRecordType OPTIONAL,

guaranteedBitRate			[70] GuaranteedBitRate OPTIONAL,

maximumBitRate				[71] MaximumBitRate OPTIONAL,

redial						[72] BOOLEAN OPTIONAL,	-- set indicates redial attempt

reasonForServiceChange		[73] ReasonForServiceChange OPTIONAL,

serviceChangeInitiator		[74] BOOLEAN OPTIONAL,

iCSI2ActiveFlag				[75] NULL OPTIONAL,

iMS-Charging-Identifier		[76] IMS-Charging-Identifier OPTIONAL,

privateUserID				[77] GraphicString OPTIONAL

}

MTCallRecord			::= SET

{

recordType				[0] RecordType,

servedIMSI				[1] IMSI,

servedIMEI				[2] IMEI OPTIONAL,

servedMSISDN				[3] CalledNumber OPTIONAL,

callingNumber			[4] CallingNumber OPTIONAL,

connectedNumber			[5] ConnectedNumber OPTIONAL,

recordingEntity			[6] RecordingEntity,

mscIncomingTKGP			[7] TrunkGroup OPTIONAL,

mscOutgoingTKGP			[8] TrunkGroup OPTIONAL,

location					[9] LocationAreaAndCell OPTIONAL,

changeOfLocation			[10] SEQUENCE OF LocationChange OPTIONAL,

basicService				[11] BasicServiceCode OPTIONAL,

transparencyIndicator	[12] TransparencyInd OPTIONAL,

changeOfService			[13] SEQUENCE OF ChangeOfService OPTIONAL,

supplServicesUsed		[14] SEQUENCE OF SuppServiceUsed OPTIONAL,

aocParameters			[15] AOCParameters OPTIONAL,

changeOfAOCParms			[16] SEQUENCE OF AOCParmChange OPTIONAL,

msClassmark				[17] Classmark OPTIONAL,

changeOfClassmark		[18] ChangeOfClassmark OPTIONAL,

seizureTime				[19] TimeStamp OPTIONAL,

answerTime				[20] TimeStamp OPTIONAL,

releaseTime				[21] TimeStamp OPTIONAL,

callDuration				[22] CallDuration,

dataVolume				[23] DataVolume OPTIONAL,

radioChanRequested		[24] RadioChanRequested OPTIONAL,

radioChanUsed			[25] TrafficChannel OPTIONAL,

changeOfRadioChan		[26] ChangeOfRadioChannel OPTIONAL,

causeForTerm				[27] CauseForTerm,

diagnostics				[28] Diagnostics OPTIONAL,

callReference			[29] CallReferenceNumber,

sequenceNumber			[30] INTEGER OPTIONAL,

additionalChgInfo		[31] AdditionalChgInfo OPTIONAL,

recordExtensions			[32] ManagementExtensions OPTIONAL,

networkCallReference		[33] NetworkCallReference OPTIONAL,

mSCAddress				[34] MSCAddress OPTIONAL,

hSCSDChanRequested		[35] NumOfHSCSDChanRequested OPTIONAL,

hSCSDChanAllocated		[36] NumOfHSCSDChanAllocated OPTIONAL,

changeOfHSCSDParms		[37] SEQUENCE OF HSCSDParmsChange OPTIONAL,

fnur						[38] Fnur OPTIONAL,

aiurRequested			[39] AiurRequested OPTIONAL,

chanCodingsAcceptable	[40] SEQUENCE OF ChannelCoding OPTIONAL,

chanCodingUsed			[41] ChannelCoding OPTIONAL,

speechVersionSupported	[42] SpeechVersionIdentifier OPTIONAL,

speechVersionUsed		[43] SpeechVersionIdentifier OPTIONAL,

gsm-SCFAddress			[44] Gsm-SCFAddress OPTIONAL,

serviceKey				[45] ServiceKey OPTIONAL,

systemType				[46] SystemType OPTIONAL,

rateIndication			[47] RateIndication OPTIONAL,

locationRoutNum			[48] LocationRoutingNumber OPTIONAL,

lrnSoInd					[49] LocationRoutingNumberSourceIndicator OPTIONAL,

lrnQuryStatus			[50] LocationRoutingNumberQueryStatus OPTIONAL,

jIPPara					[51] JurisdictionInformationParameter OPTIONAL,

jIPSoInd					[52] JurisdictionInformationParameterSourceIndicator OPTIONAL,

jIPQuryStatus			[53] JurisdictionInformationParameterQueryStatus OPTIONAL,

partialRecordType		[54] PartialRecordType OPTIONAL,

guaranteedBitRate		[55] GuaranteedBitRate OPTIONAL,

maximumBitRate			[56] MaximumBitRate OPTIONAL,

reasonForServiceChange	[57] ReasonForServiceChange OPTIONAL,

serviceChangeInitiator	[58] BOOLEAN OPTIONAL,

iCSI2ActiveFlag			[59] NULL OPTIONAL,

iMS-Charging-Identifier	[60] IMS-Charging-Identifier OPTIONAL,

privateUserID			[61] GraphicString OPTIONAL

}

RoamingRecord			::= SET

{

recordType				[0] RecordType,

servedIMSI				[1] IMSI,

servedMSISDN				[2] MSISDN OPTIONAL,

callingNumber			[3] CallingNumber OPTIONAL,

roamingNumber			[4] RoamingNumber OPTIONAL,

recordingEntity			[5] RecordingEntity,

mscIncomingTKGP			[6] TrunkGroup OPTIONAL,

mscOutgoingTKGP			[7] TrunkGroup OPTIONAL,

basicService				[8] BasicServiceCode OPTIONAL,

transparencyIndicator	[9] TransparencyInd OPTIONAL,

changeOfService			[10] SEQUENCE OF ChangeOfService OPTIONAL,

supplServicesUsed		[11] SEQUENCE OF  SuppServiceUsed OPTIONAL,

seizureTime				[12] TimeStamp OPTIONAL,

answerTime				[13] TimeStamp OPTIONAL,

releaseTime				[14] TimeStamp OPTIONAL,

callDuration				[15] CallDuration,

dataVolume				[16] DataVolume OPTIONAL,

causeForTerm				[17] CauseForTerm,

diagnostics				[18] Diagnostics OPTIONAL,

callReference			[19] CallReferenceNumber,

sequenceNumber			[20] INTEGER OPTIONAL,

recordExtensions			[21] ManagementExtensions OPTIONAL,

networkCallReference		[22] NetworkCallReference OPTIONAL,

mSCAddress				[23] MSCAddress OPTIONAL,

locationRoutNum			[24] LocationRoutingNumber OPTIONAL,

lrnSoInd					[25] LocationRoutingNumberSourceIndicator OPTIONAL,

lrnQuryStatus			[26] LocationRoutingNumberQueryStatus OPTIONAL,

jIPPara					[27] JurisdictionInformationParameter OPTIONAL,

jIPSoInd					[28] JurisdictionInformationParameterSourceIndicator OPTIONAL,

jIPQuryStatus			[29] JurisdictionInformationParameterQueryStatus OPTIONAL,

partialRecordType		[30] PartialRecordType OPTIONAL

}

TermCAMELRecord	::= SET

{

recordtype					[0] RecordType,

servedIMSI					[1] IMSI,

servedMSISDN					[2] MSISDN OPTIONAL,

recordingEntity				[3] RecordingEntity,

interrogationTime			[4] TimeStamp,

destinationRoutingAddress	[5] DestinationRoutingAddress,

gsm-SCFAddress				[6] Gsm-SCFAddress,

serviceKey					[7] ServiceKey,

networkCallReference			[8] NetworkCallReference OPTIONAL,

mSCAddress					[9] MSCAddress OPTIONAL,

defaultCallHandling			[10] DefaultCallHandling OPTIONAL,

recordExtensions				[11] ManagementExtensions OPTIONAL,

calledNumber					[12] CalledNumber,

callingNumber				[13] CallingNumber OPTIONAL,

mscIncomingTKGP				[14] TrunkGroup OPTIONAL,

mscOutgoingTKGP				[15] TrunkGroup OPTIONAL,

seizureTime					[16] TimeStamp OPTIONAL,

answerTime					[17] TimeStamp OPTIONAL,

releaseTime					[18] TimeStamp OPTIONAL,

callDuration					[19] CallDuration,

dataVolume					[20] DataVolume OPTIONAL,

causeForTerm					[21] CauseForTerm,

diagnostics					[22] Diagnostics OPTIONAL,

callReference				[23] CallReferenceNumber,

sequenceNumber				[24] INTEGER OPTIONAL,

numberOfDPEncountered		[25] INTEGER OPTIONAL,

levelOfCAMELService			[26] LevelOfCAMELService OPTIONAL,

freeFormatData				[27] FreeFormatData OPTIONAL,

cAMELCallLegInformation		[28] SEQUENCE OF CAMELInformation OPTIONAL,

freeFormatDataAppend			[29] BOOLEAN OPTIONAL,

defaultCallHandling-2		[30] DefaultCallHandling OPTIONAL,

gsm-SCFAddress-2				[31] Gsm-SCFAddress OPTIONAL,

serviceKey-2					[32] ServiceKey OPTIONAL,

freeFormatData-2				[33] FreeFormatData OPTIONAL,

freeFormatDataAppend-2		[34] BOOLEAN OPTIONAL,

mscServerIndication			[35] BOOLEAN OPTIONAL,

locationRoutNum				[36] LocationRoutingNumber OPTIONAL,

lrnSoInd						[37] LocationRoutingNumberSourceIndicator OPTIONAL,

lrnQuryStatus				[38] LocationRoutingNumberQueryStatus OPTIONAL,

jIPPara						[39] JurisdictionInformationParameter OPTIONAL,

jIPSoInd						[40] JurisdictionInformationParameterSourceIndicator OPTIONAL,

jIPQuryStatus				[41] JurisdictionInformationParameterQueryStatus OPTIONAL,

partialRecordType			[42] PartialRecordType OPTIONAL

}

IncGatewayRecord		::= SET

{

recordType				[0] RecordType,

callingNumber			[1] CallingNumber OPTIONAL,

calledNumber				[2] CalledNumber,

recordingEntity			[3] RecordingEntity,

mscIncomingTKGP			[4] TrunkGroup OPTIONAL,

mscOutgoingTKGP			[5] TrunkGroup OPTIONAL,

seizureTime				[6] TimeStamp OPTIONAL,

answerTime				[7] TimeStamp OPTIONAL,

releaseTime				[8] TimeStamp OPTIONAL,

callDuration				[9] CallDuration,

dataVolume				[10] DataVolume OPTIONAL,

causeForTerm				[11] CauseForTerm,

diagnostics				[12] Diagnostics OPTIONAL,

callReference			[13] CallReferenceNumber,

sequenceNumber			[14] INTEGER OPTIONAL,

recordExtensions			[15] ManagementExtensions OPTIONAL,

locationRoutNum			[16] LocationRoutingNumber OPTIONAL,

lrnSoInd					[17] LocationRoutingNumberSourceIndicator OPTIONAL,

lrnQuryStatus			[18] LocationRoutingNumberQueryStatus OPTIONAL,

jIPPara					[19] JurisdictionInformationParameter OPTIONAL,

jIPSoInd					[20] JurisdictionInformationParameterSourceIndicator OPTIONAL,

jIPQuryStatus			[21] JurisdictionInformationParameterQueryStatus OPTIONAL,

reasonForServiceChange	[22] ReasonForServiceChange OPTIONAL,

serviceChangeInitiator	[23] BOOLEAN OPTIONAL

}

OutGatewayRecord		::= SET

{

recordType				[0] RecordType,

callingNumber			[1] CallingNumber OPTIONAL,

calledNumber				[2] CalledNumber,

recordingEntity			[3] RecordingEntity,

mscIncomingTKGP			[4] TrunkGroup OPTIONAL,

mscOutgoingTKGP			[5] TrunkGroup OPTIONAL,

seizureTime				[6] TimeStamp OPTIONAL,

answerTime				[7] TimeStamp OPTIONAL,

releaseTime				[8] TimeStamp OPTIONAL,

callDuration				[9] CallDuration,

dataVolume				[10] DataVolume OPTIONAL,

causeForTerm				[11] CauseForTerm,

diagnostics				[12] Diagnostics OPTIONAL,

callReference			[13] CallReferenceNumber,

sequenceNumber			[14] INTEGER OPTIONAL,

recordExtensions			[15] ManagementExtensions OPTIONAL,

locationRoutNum			[16] LocationRoutingNumber OPTIONAL,

lrnSoInd					[17] LocationRoutingNumberSourceIndicator OPTIONAL,

lrnQuryStatus			[18] LocationRoutingNumberQueryStatus OPTIONAL,

jIPPara					[19] JurisdictionInformationParameter OPTIONAL,

jIPSoInd					[20] JurisdictionInformationParameterSourceIndicator OPTIONAL,

jIPQuryStatus			[21] JurisdictionInformationParameterQueryStatus OPTIONAL,

reasonForServiceChange	[22] ReasonForServiceChange OPTIONAL,

serviceChangeInitiator	[23] BOOLEAN OPTIONAL
}

TransitCallRecord		::= SET

{

recordType				[0] RecordType,

recordingEntity			[1] RecordingEntity,

mscIncomingTKGP			[2] TrunkGroup OPTIONAL,

mscOutgoingTKGP			[3] TrunkGroup OPTIONAL,

callingNumber			[4] CallingNumber OPTIONAL,

calledNumber				[5] CalledNumber,

isdnBasicService			[6] BasicService OPTIONAL,

seizureTimestamp			[7] TimeStamp OPTIONAL,

answerTimestamp			[8] TimeStamp OPTIONAL,

releaseTimestamp			[9] TimeStamp OPTIONAL,

callDuration				[10] CallDuration,

dataVolume				[11] DataVolume OPTIONAL,

causeForTerm				[12] CauseForTerm,

diagnostics				[13] Diagnostics OPTIONAL,

callReference			[14] CallReferenceNumber,

sequenceNumber			[15] INTEGER OPTIONAL,

recordExtensions			[16] ManagementExtensions OPTIONAL,

locationRoutNum			[17] LocationRoutingNumber OPTIONAL,

lrnSoInd					[18] LocationRoutingNumberSourceIndicator OPTIONAL,

lrnQuryStatus			[19] LocationRoutingNumberQueryStatus OPTIONAL,

jIPPara					[20] JurisdictionInformationParameter OPTIONAL,

jIPSoInd					[21] JurisdictionInformationParameterSourceIndicator OPTIONAL,

jIPQuryStatus			[22] JurisdictionInformationParameterQueryStatus OPTIONAL

}

MOSMSRecord				::= SET

{

recordType				[0] RecordType,

servedIMSI				[1] IMSI,

servedIMEI				[2] IMEI OPTIONAL,

servedMSISDN				[3] MSISDN OPTIONAL,

msClassmark				[4] Classmark,

serviceCentre			[5] AddressString,

recordingEntity			[6] RecordingEntity,

location					[7] LocationAreaAndCell OPTIONAL,

messageReference			[8] MessageReference,

originationTime			[9] TimeStamp,

smsResult				[10] SMSResult OPTIONAL,

recordExtensions			[11] ManagementExtensions OPTIONAL,

destinationNumber		[12] SmsTpDestinationNumber OPTIONAL,

cAMELSMSInformation		[13] CAMELSMSInformation OPTIONAL,

systemType				[14] SystemType OPTIONAL,

locationExtension		[15] LocationCellExtension OPTIONAL

}

MTSMSRecord				::= SET

{

recordType				[0] RecordType,

serviceCentre			[1] AddressString,

servedIMSI				[2] IMSI,

servedIMEI				[3] IMEI OPTIONAL,

servedMSISDN				[4] MSISDN OPTIONAL,

msClassmark				[5] Classmark,

recordingEntity			[6] RecordingEntity,

location					[7] LocationAreaAndCell OPTIONAL,

deliveryTime				[8] TimeStamp,

smsResult				[9] SMSResult OPTIONAL,

recordExtensions			[10] ManagementExtensions OPTIONAL,

systemType				[11] SystemType OPTIONAL,

cAMELSMSInformation		[12] CAMELSMSInformation OPTIONAL,

locationExtension		[13] LocationCellExtension OPTIONAL

}

MOSMSIWRecord			::= SET

{

recordType			[0] RecordType,

serviceCentre		[1] AddressString,

servedIMSI			[2] IMSI,

recordingEntity		[3] RecordingEntity,

eventTime			[4] TimeStamp,

smsResult			[5] SMSResult OPTIONAL,

recordExtensions	[6] ManagementExtensions OPTIONAL

}

MTSMSGWRecord			::= SET

{

recordType			[0] RecordType,

serviceCentre		[1] AddressString,

servedIMSI			[2] IMSI,

servedMSISDN			[3] MSISDN OPTIONAL,

recordingEntity		[4] RecordingEntity,

eventTime			[5] TimeStamp,

smsResult			[6] SMSResult OPTIONAL,

recordExtensions		[7] ManagementExtensions OPTIONAL

}

SSActionRecord			::= SET

{

recordType			[0] RecordType,

servedIMSI			[1] IMSI,

servedIMEI			[2] IMEI OPTIONAL,

servedMSISDN			[3] MSISDN OPTIONAL,

msClassmark			[4] Classmark,

recordingEntity		[5] RecordingEntity,

location				[6] LocationAreaAndCell OPTIONAL,

basicServices		[7] BasicServices OPTIONAL,

supplService			[8] SS-Code OPTIONAL,

ssAction				[9] SSActionType OPTIONAL,

ssActionTime			[10] TimeStamp,

ssParameters			[11] SSParameters OPTIONAL,

ssActionResult		[12] SSActionResult OPTIONAL,

callReference		[13] CallReferenceNumber,

recordExtensions		[14] ManagementExtensions OPTIONAL,

systemType			[15] SystemType OPTIONAL

}

HLRIntRecord			::= SET

{

recordType				[0] RecordType,

servedIMSI				[1] IMSI,

servedMSISDN				[2] MSISDN,

recordingEntity			[3] RecordingEntity,

basicService				[4] BasicServiceCode OPTIONAL,

routingNumber			[5] RoutingNumber,

interrogationTime		[6] TimeStamp,

numberOfForwarding		[7] NumberOfForwarding OPTIONAL,

interrogationResult		[8] HLRIntResult OPTIONAL,

recordExtensions			[9] ManagementExtensions OPTIONAL

}

LocUpdateHLRRecord 		::= SET

{

recordType				[0] RecordType,

servedIMSI				[1] IMSI,

recordingEntity			[2] RecordingEntity,

oldLocation				[3] Visited-Location-info OPTIONAL,

newLocation				[4] Visited-Location-info,

updateTime				[5] TimeStamp,

updateResult				[6] LocUpdResult OPTIONAL,

recordExtensions			[7] ManagementExtensions OPTIONAL

}

LocUpdateVLRRecord 		::= SET

{

recordType				[0] RecordType,

servedIMSI				[1] IMSI,

servedMSISDN				[2] MSISDN OPTIONAL,

recordingEntity			[3] RecordingEntity,

oldLocation				[4] Location-info OPTIONAL,

newLocation				[5] Location-info,

msClassmark				[6] Classmark,

updateTime				[7] TimeStamp,

updateResult				[8] LocUpdResult OPTIONAL,

recordExtensions			[9] ManagementExtensions OPTIONAL,

locationExtension		[10] LocationCellExtension OPTIONAL

}

CommonEquipRecord 		::= SET

{

recordType				[0] RecordType,

equipmentType			[1] EquipmentType,

equipmentId				[2] EquipmentId,

servedIMSI				[3] IMSI,

servedMSISDN				[4] MSISDN OPTIONAL,

recordingEntity			[5] RecordingEntity,

basicService				[6] BasicServiceCode OPTIONAL,

changeOfService			[7] SEQUENCE OF ChangeOfService OPTIONAL,

supplServicesUsed		[8] SEQUENCE OF SuppServiceUsed OPTIONAL,

seizureTime				[9] TimeStamp,

releaseTime				[10] TimeStamp OPTIONAL,

callDuration				[11] CallDuration,

callReference			[12] CallReferenceNumber,

sequenceNumber			[13] INTEGER OPTIONAL,

recordExtensions			[14] ManagementExtensions OPTIONAL,

systemType				[15] SystemType OPTIONAL,

rateIndication			[16] RateIndication OPTIONAL,

fnur						[17] Fnur OPTIONAL

}

--

--  OBSERVED IMEI TICKETS

--

ObservedIMEITicket		::= SET

{

servedIMEI			[0] IMEI,

imeiStatus			[1] IMEIStatus,

servedIMSI			[2] IMSI,

servedMSISDN			[3] MSISDN OPTIONAL,

recordingEntity		[4] RecordingEntity,

eventTime			[5] TimeStamp,

location				[6] LocationAreaAndCell	,

imeiCheckEvent		[7] IMEICheckEvent OPTIONAL,

callReference		[8] CallReferenceNumber OPTIONAL,

recordExtensions		[9] ManagementExtensions OPTIONAL

}

--

--  CS LOCATION SERVICE RECORDS

--

MTLCSRecord				::= SET

{

recordType				[0] RecordType,

recordingEntity			[1] RecordingEntity,

lcsClientType			[2] LCSClientType,

lcsClientIdentity		[3] LCSClientIdentity,

servedIMSI				[4] IMSI OPTIONAL,

servedMSISDN				[5] MSISDN OPTIONAL,

locationType				[6] LocationType,

lcsQos					[7] LCSQoSInfo OPTIONAL,

lcsPriority				[8] LCS-Priority OPTIONAL,

mlc-Number				[9] ISDN-AddressString,

eventTimeStamp			[10] TimeStamp,

measureDuration			[11] CallDuration OPTIONAL,

notificationToMSUser		[12] NotificationToMSUser OPTIONAL,

privacyOverride			[13] NULL OPTIONAL,

location					[14] LocationAreaAndCell OPTIONAL,

locationEstimate			[15] Ext-GeographicalInformation OPTIONAL,

positioningData			[16] PositioningData OPTIONAL,

lcsCause					[17] LCSCause OPTIONAL,

diagnostics				[18] Diagnostics OPTIONAL,

systemType				[19] SystemType OPTIONAL,

recordExtensions			[20] ManagementExtensions OPTIONAL,

causeForTerm				[21] CauseForTerm,

servedIMEI				[22] IMEI OPTIONAL

}

MOLCSRecord				::= SET

{

recordType				[0] RecordType,

recordingEntity			[1] RecordingEntity,

lcsClientType			[2] LCSClientType OPTIONAL,

lcsClientIdentity		[3] LCSClientIdentity OPTIONAL,

servedIMSI				[4] IMSI,

servedMSISDN				[5] MSISDN OPTIONAL,

molr-Type				[6] MOLR-Type,

lcsQos					[7] LCSQoSInfo OPTIONAL,

lcsPriority				[8] LCS-Priority OPTIONAL,

mlc-Number				[9] ISDN-AddressString OPTIONAL,

eventTimeStamp			[10] TimeStamp,

measureDuration			[11] CallDuration OPTIONAL,

location					[12] LocationAreaAndCell OPTIONAL,

locationEstimate			[13] Ext-GeographicalInformation OPTIONAL,

positioningData			[14] PositioningData OPTIONAL,

lcsCause					[15] LCSCause OPTIONAL,

diagnostics				[16] Diagnostics OPTIONAL,

systemType				[17] SystemType OPTIONAL,

recordExtensions			[18] ManagementExtensions OPTIONAL,

causeForTerm				[19] CauseForTerm

}

NILCSRecord				::= SET

{

recordType				[0] RecordType,

recordingEntity			[1] RecordingEntity,

lcsClientType			[2] LCSClientType OPTIONAL,

lcsClientIdentity		[3] LCSClientIdentity OPTIONAL,

servedIMSI				[4] IMSI OPTIONAL,

servedMSISDN				[5] MSISDN OPTIONAL,

servedIMEI				[6] IMEI OPTIONAL,

emsDigits				[7] ISDN-AddressString OPTIONAL,

emsKey					[8] ISDN-AddressString OPTIONAL,

lcsQos					[9] LCSQoSInfo OPTIONAL,

lcsPriority				[10] LCS-Priority OPTIONAL,

mlc-Number				[11] ISDN-AddressString OPTIONAL,

eventTimeStamp			[12] TimeStamp,

measureDuration			[13] CallDuration OPTIONAL,

location					[14] LocationAreaAndCell OPTIONAL,

locationEstimate			[15] Ext-GeographicalInformation OPTIONAL,

positioningData			[16] PositioningData OPTIONAL,

lcsCause					[17] LCSCause OPTIONAL,

diagnostics				[18] Diagnostics OPTIONAL,

systemType				[19] SystemType OPTIONAL,

recordExtensions			[20] ManagementExtensions OPTIONAL,

causeForTerm				[21] CauseForTerm

}

--

--  SRVCC RECORDS

--

MSCsRVCCRecord	::= SET

{

recordType					[0] RecordType,

servedIMSI					[1] IMSI OPTIONAL,

servedIMEI					[2] IMEI OPTIONAL,

servedMSISDN					[3] MSISDN OPTIONAL,

calledNumber					[5] CalledNumber,

recordingEntity				[9] RecordingEntity,

mscOutgoingTKGP				[11] TrunkGroup OPTIONAL,

location						[12] LocationAreaAndCell,

changeOfLocation				[13] SEQUENCE OF LocationChange OPTIONAL,

basicService					[14] BasicServiceCode,

supplServicesUsed			[17] SEQUENCE OF SuppServiceUsed OPTIONAL,

msClassmark					[20] Classmark OPTIONAL,

seizureTime					[22] TimeStamp OPTIONAL,

answerTime					[23] TimeStamp OPTIONAL,

releaseTime					[24] TimeStamp OPTIONAL,

callDuration					[25] CallDuration,

causeForTerm					[30] CauseForTerm,

diagnostics					[31] Diagnostics OPTIONAL,

callReference				[32] CallReferenceNumber,

sequenceNumber				[33] INTEGER OPTIONAL,

recordExtensions				[35] ManagementExtensions OPTIONAL,

partialRecordType			[69] PartialRecordType OPTIONAL,

iMS-Charging-Identifier		[75] IMS-Charging-Identifier OPTIONAL,

iCSI2ActiveFlag				[76] NULL OPTIONAL,

relatedICID					[77] IMS-Charging-Identifier OPTIONAL,

relatedICIDGenerationNode	[78] NodeAddress OPTIONAL

}

--

--  MTRF RECORD

--

MTRFRecord			::= SET

{

recordType				[0] RecordType,

servedIMSI				[1] IMSI,

servedIMEI				[2] IMEI OPTIONAL,

servedMSISDN				[3] CalledNumber OPTIONAL,

callingNumber			[4] CallingNumber OPTIONAL,

roamingNumber			[5] RoamingNumber OPTIONAL,

recordingEntity			[6] RecordingEntity,

mscIncomingTKGP			[7] TrunkGroup OPTIONAL,

mscOutgoingTKGP			[8] TrunkGroup OPTIONAL,

basicService				[9] BasicServiceCode OPTIONAL,

seizureTime				[10] TimeStamp OPTIONAL,

answerTime				[11] TimeStamp OPTIONAL,

releaseTime				[12] TimeStamp OPTIONAL,

callDuration				[13] CallDuration,

causeForTerm				[14] CauseForTerm,

diagnostics				[15] Diagnostics OPTIONAL,

callReference			[16] CallReferenceNumber,

sequenceNumber			[17] INTEGER OPTIONAL,

recordExtensions			[18] ManagementExtensions OPTIONAL,

partialRecordType		[19] PartialRecordType OPTIONAL

}

--

--  ICS RECORD

--

ICSregisterRecord ::= SET

{

recordType					[0] RecordType,

servedIMSI					[1] IMSI,

servedMSISDN					[2] MSISDN,

privateUserID				[3] GraphicString OPTIONAL,

recordingEntity				[4] RecordingEntity,

newLocation					[5] Location-info,

locationExtension			[6] LocationCellExtension OPTIONAL,

updateTime					[7] TimeStamp OPTIONAL,

iMS-Charging-Identifier		[8] IMS-Charging-Identifier OPTIONAL,

interOperatorIdentifiers		[9] InterOperatorIdentifierList OPTIONAL,

transit-IOI-Lists			[10] TransitIOILists OPTIONAL,

updateResult					[11] LocUpdResult OPTIONAL,

recordExtensions				[12] ManagementExtensions OPTIONAL

}

--

--  NP Fields

--

LocationRoutingNumber	::= OCTET STRING (SIZE (5))

--

--  The format is selected to meet the existing standards for the wireline in Telcordia

--   Belcore GR-1100-CORE, BAF Module 720.

--

LocationRoutingNumberSourceIndicator	::=	INTEGER

{

lRN-NP-Database				(1),

switchingSystemData			(2),

incomingsignaling			(3),

unknown						(9)

}

LocationRoutingNumberQueryStatus 		::=	INTEGER

{

successfulQuery				(1),

noQueryResponseMsg			(2),

queryProtocolErr				(4),

queryResponseDataErr			(5),

queryRejected				(6),

queryNotPerformed			(9),

queryUnsuccessful			(99)

}

JurisdictionInformationParameter 	::= OCTET STRING (SIZE (5))

--

-- /* JIP Parameter */

--

JurisdictionInformationParameterSourceIndicator 	::=	INTEGER

--

-- Identical to LocationRoutingNumberSourceIndicator

--

{

lRN-NP-Database				(1),

switchingSystemData			(2),

incomingsignaling			(3),

unknown						(9)

}

JurisdictionInformationParameterQueryStatus 	::=	INTEGER

{

successfulQuery				(1),

noQueryResponseMsg			(2),

queryProtocolErr				(4),

queryResponseDataErr			(5),

queryRejected				(6),

queryNotPerformed			(9),

queryUnsuccessful			(99)

}

--

--  CS DATA TYPES

--

AdditionalChgInfo		::= SEQUENCE

{

chargeIndicator		[0] ChargeIndicator OPTIONAL,

chargeParameters	[1] OCTET STRING OPTIONAL

}

AiurRequested			::= ENUMERATED

--

-- See Bearer Capability TS 24.008 [208]

-- (note that value "4" is intentionally missing

-- because it is not used in TS 24.008 [208])

--

{

aiur09600BitsPerSecond		(1),

aiur14400BitsPerSecond		(2),

aiur19200BitsPerSecond		(3),

aiur28800BitsPerSecond		(5),

aiur38400BitsPerSecond		(6),

aiur43200BitsPerSecond		(7),

aiur57600BitsPerSecond		(8),

aiur38400BitsPerSecond1		(9),

aiur38400BitsPerSecond2		(10),

aiur38400BitsPerSecond3		(11),

aiur38400BitsPerSecond4		(12)

}

AOCParameters			::= SEQUENCE

--

-- See TS 22.024 [104].

--

{

e1					[1] EParameter OPTIONAL,

e2					[2] EParameter OPTIONAL,

e3					[3] EParameter OPTIONAL,

e4					[4] EParameter OPTIONAL,

e5					[5] EParameter OPTIONAL,

e6					[6] EParameter OPTIONAL,

e7					[7] EParameter OPTIONAL

}

AOCParmChange			::= SEQUENCE

{

changeTime			[0] TimeStamp,

newParameters		[1] AOCParameters

}

BasicServices			::= SET OF BasicServiceCode

CallingPartyCategory	::= Category

CallType				::= INTEGER

{

mobileOriginated	(0),

mobileTerminated	(1)

}

CallTypes 				::= SET OF CallType

CAMELDestinationNumber	::= DestinationRoutingAddress

CAMELInformation		::= SET

{

cAMELDestinationNumber		[1] CAMELDestinationNumber OPTIONAL,

connectedNumber				[2] ConnectedNumber OPTIONAL,

roamingNumber				[3] RoamingNumber OPTIONAL,

mscOutgoingTKGP				[4] TrunkGroup OPTIONAL,

seizureTime					[5] TimeStamp OPTIONAL,

answerTime					[6] TimeStamp OPTIONAL,

releaseTime					[7] TimeStamp OPTIONAL,

callDuration					[8] CallDuration OPTIONAL,

dataVolume					[9] DataVolume OPTIONAL,

cAMELInitCFIndicator			[10] CAMELInitCFIndicator OPTIONAL,

causeForTerm					[11] CauseForTerm OPTIONAL,

cAMELModification			[12] ChangedParameters OPTIONAL,

freeFormatData				[13] FreeFormatData OPTIONAL,

diagnostics					[14] Diagnostics OPTIONAL,

freeFormatDataAppend			[15] BOOLEAN OPTIONAL,

freeFormatData-2				[16] FreeFormatData OPTIONAL,

freeFormatDataAppend-2		[17] BOOLEAN OPTIONAL

}

CAMELInitCFIndicator	::= ENUMERATED

{

noCAMELCallForwarding		(0),

cAMELCallForwarding			(1)

}

CAMELModificationParameters		::= SET

--

-- The list contains only parameters changed due to CAMEL call handling.

--

{

callingPartyNumber			[0] CallingNumber OPTIONAL,

callingPartyCategory			[1] CallingPartyCategory OPTIONAL,

originalCalledPartyNumber	[2] OriginalCalledNumber OPTIONAL,

genericNumbers				[3] GenericNumbers OPTIONAL,

redirectingPartyNumber		[4] RedirectingNumber OPTIONAL,

redirectionCounter			[5] NumberOfForwarding OPTIONAL

}

CAMELSMSInformation		::= SET

{

gsm-SCFAddress						[1] Gsm-SCFAddress OPTIONAL,

serviceKey							[2] ServiceKey OPTIONAL,

defaultSMSHandling			 		[3] DefaultSMS-Handling OPTIONAL,

freeFormatData       				[4] FreeFormatData OPTIONAL,

callingPartyNumber 					[5] CallingNumber OPTIONAL,

destinationSubscriberNumber			[6] SmsTpDestinationNumber OPTIONAL,

cAMELSMSCAddress						[7] AddressString OPTIONAL,

smsReferenceNumber					[8] CallReferenceNumber OPTIONAL

}

Category		::= OCTET STRING (SIZE(1))

--

-- The internal structure is defined in Recommendation Q.763.

--

ChangedParameters		::= SET

{

changeFlags		[0] ChangeFlags,

changeList		[1] CAMELModificationParameters OPTIONAL

}

ChangeFlags				::= BIT STRING

{

callingPartyNumberModified			(0),

callingPartyCategoryModified			(1),

originalCalledPartyNumberModified	(2),

genericNumbersModified				(3),

redirectingPartyNumberModified		(4),

redirectionCounterModified			(5)

}

ChangeOfClassmark 		::= SEQUENCE

{

classmark			[0] Classmark,

changeTime			[1] TimeStamp

}

ChangeOfRadioChannel 	::= SEQUENCE

{

radioChannel		[0] TrafficChannel,

changeTime			[1] TimeStamp,

speechVersionUsed	[2] SpeechVersionIdentifier OPTIONAL

}

ChangeOfService 		::= SEQUENCE

{

basicService		[0] BasicServiceCode,

transparencyInd	[1] TransparencyInd OPTIONAL,

changeTime		[2] TimeStamp,

rateIndication	[3] RateIndication OPTIONAL,

fnur				[4] Fnur OPTIONAL

}

ChannelCoding				::= ENUMERATED

{

tchF4800				(1),

tchF9600				(2),

tchF14400			(3)

}

Classmark				::= OCTET STRING

--

-- See Mobile station classmark 2, Mobile station classmark 3, TS 24.008[208]

--

ConnectedNumber			::= BCDDirectoryNumber

DataVolume				::= INTEGER

--

-- The volume of data transferred in segments of 64 octets.

--

Day						::= INTEGER (1..31)

DayClass				::= ObjectInstance

DayClasses				::= SET OF DayClass

DayDefinition			::= SEQUENCE

{

day				[0] DayOfTheWeek,

dayClass			[1] ObjectInstance

}

DayDefinitions			::= SET OF DayDefinition

DateDefinition			::= SEQUENCE

{

month				[0] Month,

day					[1] Day,

dayClass				[2] ObjectInstance

}

DateDefinitions			::= SET OF DateDefinition

DayOfTheWeek			::= ENUMERATED

{

allDays				(0),

sunday				(1),

monday				(2),

tuesday				(3),

wednesday			(4),

thursday				(5),

friday				(6),

saturday				(7)

}

Destinations			::= SET OF AE-title

EmergencyCallIndEnable	::= BOOLEAN

EmergencyCallIndication	::= SEQUENCE

{

cellId			[0] CellId,

callerId			[1] IMSIorIMEI

}

EParameter ::= INTEGER

EquipmentId				::= INTEGER

EquipmentType			::= INTEGER

{

conferenceBridge	(0)

}

FileType				::= INTEGER

{

callRecords			(1),

traceRecords			(9),

observedIMEITicket	(14)

}

Fnur					::= ENUMERATED

--

-- See Bearer Capability TS 24.008 [208]

--

{

fnurNotApplicable			(0),

fnur9600-BitsPerSecond		(1),

fnur14400BitsPerSecond		(2),

fnur19200BitsPerSecond		(3),

fnur28800BitsPerSecond		(4),

fnur38400BitsPerSecond		(5),

fnur48000BitsPerSecond		(6),

fnur56000BitsPerSecond		(7),

fnur64000BitsPerSecond		(8),

fnur33600BitsPerSecond		(9),

fnur32000BitsPerSecond		(10),

fnur31200BitsPerSecond		(11)

}

ForwardToNumber 		::= AddressString

FreeFormatData			::= OCTET STRING (SIZE(1..160))

--

-- Free formatted data as sent in the FCI message

-- See TS 29.078 [217]

--

GenericNumber			::= BCDDirectoryNumber

GenericNumbers			::= SET OF GenericNumber

Gsm-SCFAddress			::= ISDN-AddressString

--

-- See TS 29.002 [214]

--

GuaranteedBitRate ::= ENUMERATED

{

gbr14400BitsPerSecond (1),		-- BS20 non-transparent

gbr28800BitsPerSecond (2),		-- BS20 non-transparent and transparent,

-- BS30 transparent and multimedia

gbr32000BitsPerSecond (3),		-- BS30 multimedia

gbr33600BitsPerSecond (4),		-- BS30 multimedia

gbr56000BitsPerSecond (5),		-- BS30 transparent and multimedia

gbr57600BitsPerSecond (6),		-- BS20 non-transparent

gbr64000BitsPerSecond (7)		-- BS30 transparent and multimedia

}

HLRIntResult			::= Diagnostics

HSCSDParmsChange		::= SEQUENCE

{

changeTime				[0] TimeStamp,

hSCSDChanAllocated		[1] NumOfHSCSDChanAllocated,

initiatingParty			[2] InitiatingParty OPTIONAL,

aiurRequested			[3] AiurRequested OPTIONAL,

chanCodingUsed			[4] ChannelCoding,

hSCSDChanRequested		[5] NumOfHSCSDChanRequested OPTIONAL

}

IMEICheckEvent			::= INTEGER

{

mobileOriginatedCall	(0),

mobileTerminatedCall	(1),

smsMobileOriginating	(2),

smsMobileTerminating	(3),

ssAction				(4),

locationUpdate		(5)

}

IMEIStatus				::= ENUMERATED

{

trackListedMobileEquipment		(0),

blockListedMobileEquipment		(1),

nonAllowListedMobileEquipment	(2)

}

IMSIorIMEI				::= CHOICE

{

imsi				[0] IMSI,

imei				[1] IMEI

}

InitiatingParty			::= ENUMERATED

{

network				(0),

subscriber			(1)

}

LocationCellExtension	::= BIT STRING (SIZE (12))

LocationChange			::= SEQUENCE

{

location			[0] LocationAreaAndCell,

changeTime		[1] TimeStamp

}

Location-info			::= SEQUENCE

{

mscNumber			[1] MscNo OPTIONAL,

location-area		[2] LocationAreaCode,

cell-identification	[3] CellId OPTIONAL,

mCC-MNC				[4] MCC-MNC OPTIONAL

}

LocUpdResult			::= Diagnostics

MaximumBitRate ::= ENUMERATED

{

mbr14400BitsPerSecond	(1),		-- BS20 non-transparent

mbr28800BitsPerSecond	(2),		-- BS20 non-transparent and transparent,

-- BS30 transparent and multimedia

mbr32000BitsPerSecond	(3),		-- BS30 multimedia

mbr33600BitsPerSecond	(4),		-- BS30 multimedia

mbr56000BitsPerSecond	(5),		-- BS30 transparent and multimedia

mbr57600BitsPerSecond	(6)		-- BS20 non-transparent

}

Month		::= INTEGER (1..12)

MSPowerClasses	::= SET OF RFPowerCapability

NetworkCallReference	::= CallReferenceNumber

--

-- See TS 29.002 [214]

--

NetworkSpecificCode		::= INTEGER

--

-- To be defined by network operator

--

NetworkSpecificServices	::= SET OF NetworkSpecificCode

NumOfHSCSDChanRequested		::= INTEGER

NumOfHSCSDChanAllocated		::= INTEGER

ObservedIMEITicketEnable	::= BOOLEAN

OriginalCalledNumber	::= BCDDirectoryNumber

OriginDestCombinations	::= SET OF OriginDestCombination

OriginDestCombination	::= SEQUENCE

--

-- Note that these values correspond to the contents

-- of the attributes originId and destinationId

-- respectively. At least one of the two must be present.

--

{

origin					[0] INTEGER OPTIONAL,

destination				[1] INTEGER OPTIONAL

}

PartialRecordTimer	::= INTEGER

PartialRecordType	::= ENUMERATED

{

timeLimit					(0),

serviceChange				(1),

locationChange				(2),

classmarkChange				(3),

aocParmChange				(4),

radioChannelChange			(5),

hSCSDParmChange				(6),

changeOfCAMELDestination	(7)

}

PartialRecordTypes		::= SET OF PartialRecordType

RadioChannelsRequested	::= SET OF RadioChanRequested

RadioChanRequested	::= ENUMERATED

--

-- See Bearer Capability TS 24.008 [208]

--

{

halfRateChannel				(0),

fullRateChannel				(1),

dualHalfRatePreferred		(2),

dualFullRatePreferred		(3)

}

RateIndication ::= OCTET STRING(SIZE(1))

ReasonForServiceChange	::= ENUMERATED

{

msubInitiated			(0),

mscInitiated				(1),

callSetupFallBack		(2),

callSetupChangeOrder		(3)

}

RecordClassDestination	::= CHOICE

{

osApplication	[0] AE-title,

fileType			[1] FileType

}

RecordClassDestinations	::= SET OF RecordClassDestination

RecordingMethod	::= ENUMERATED

{

inCallRecord		(0),

inSSRecord		(1)

}

RedirectingNumber	::= BCDDirectoryNumber

RFPowerCapability	::= INTEGER

--

-- This field contains the RF power capability of the Mobile station

-- classmark 1 and 2 of TS 24.008 [208] expressed as an integer.

--

RoamingNumber			::= ISDN-AddressString

--

-- See TS 23.003 [200]

--

RoutingNumber			::= CHOICE

{

roaming				[1] RoamingNumber,

forwarded			[2] ForwardToNumber

}

Service						::= CHOICE

{

teleservice					[1] TeleserviceCode,

bearerService				[2] BearerServiceCode,

supplementaryService			[3] SS-Code,

networkSpecificService		[4] NetworkSpecificCode

}

ServiceDistanceDependencies	::= SET OF ServiceDistanceDependency

ServiceDistanceDependency	::= SEQUENCE

--

-- Note that these values correspond to the contents

-- of the attributes aocServiceId and zoneId

-- respectively.

--

{

aocService			[0] INTEGER,

chargingZone			[1] INTEGER OPTIONAL

}

SimpleIntegerName			::= INTEGER

SimpleStringName			::= GraphicString

SpeechVersionIdentifier		::= OCTET STRING (SIZE(1))

--

--	see GSM 08.08[313]

--

--	000 0001	GSM speech full rate version 1

--	001 0001	GSM speech full rate version 2	used for enhanced full rate

--	010 0001	GSM speech full rate version 3	for future use

--	000 0101	GSM speech half rate version 1

--	001 0101	GSM speech half rate version 2 	for future use

--	010 0101	GSM speech half rate version 3	for future use

--

SSActionResult				::= Diagnostics

SSActionType				::= ENUMERATED

{

registration				(0),

erasure					(1),

activation				(2),

deactivation				(3),

interrogation			(4),

invocation				(5),

passwordRegistration		(6)

}

SSParameters				::= CHOICE

{

forwardedToNumber	[0] ForwardToNumber,

unstructuredData		[1] OCTET STRING

}

SupplServices				::= SET OF SS-Code

SuppServiceUsed				::= SEQUENCE

{

ssCode					[0] SS-Code,

ssTime					[1] TimeStamp OPTIONAL

}

SwitchoverTime				::= SEQUENCE

{

hour					INTEGER (0..23),

minute					INTEGER (0..59),

second					INTEGER (0..59)

}

TariffId					::= INTEGER

TariffPeriod				::= SEQUENCE

--

-- Note that the value of tariffId corresponds to the attribute tariffId.

--

{

switchoverTime			[0] SwitchoverTime,

tariffId				[1] INTEGER

}

TariffPeriods				::= SET OF TariffPeriod

TariffSystemStatus			::= ENUMERATED

{

available				(0),	-- available for modification

checked					(1),	-- "frozen" and checked

standby					(2),	-- "frozen" awaiting activation

active					(3)		-- "frozen" and active

}

TrafficChannel			::=	ENUMERATED

{

fullRate			(0),

halfRate			(1)

}

TranslatedNumber		::= 	BCDDirectoryNumber

TransparencyInd			::=	ENUMERATED

{

transparent			(0),

nonTransparent		(1)

}

TrunkGroup				::=	 CHOICE

{

tkgpNumber		[0] INTEGER,

tkgpName			[1] GraphicString

}

TSChangeover			::=	SEQUENCE

--

-- Note that if the changeover time is not

-- specified then the change is immediate.

--

{

newActiveTS			[0] INTEGER,

newStandbyTS			[1] INTEGER,

changeoverTime		[2] GeneralizedTime OPTIONAL,

authkey				[3] OCTET STRING OPTIONAL,

checksum				[4] OCTET STRING OPTIONAL,

versionNumber		[5] OCTET STRING OPTIONAL

}

TSCheckError			::=	SEQUENCE

{

errorId				[0] TSCheckErrorId,

fail					ANY DEFINED BY errorId OPTIONAL

}

TSCheckErrorId			::=	CHOICE

{

globalForm			[0] OBJECT IDENTIFIER,

localForm			[1] INTEGER

}

TSCheckResult			::=	CHOICE

{

success				[0] NULL,

fail					[1] SET OF TSCheckError

}

TSCopyTariffSystem		::=	SEQUENCE

{

oldTS				[0] INTEGER,

newTS				[1] INTEGER

}

TSNextChange			::=	CHOICE

{

noChangeover			[0] NULL,

tsChangeover			[1] TSChangeover

}

TypeOfSubscribers		::= ENUMERATED

{

home					(0),	-- HPLMN subscribers

visiting				(1),	-- roaming subscribers

all	(2)

}

TypeOfTransaction		::=	ENUMERATED

{

successful			(0),

unsuccessful			(1),

all					(2)

}

Visited-Location-info		::= SEQUENCE

{

mscNumber			[1] MscNo,

vlrNumber			[2] VlrNo

}

VlrNo		::= ISDN-AddressString

--

-- See TS 23.003 [200]

--

.#END

End of changes