# 3GPP TSG-SA5 Meeting #138-e	S5-214161

# e-meeting, 23 - 31 August 2021

# Source:	CATT

# Title:	pCR Updating descriptions on 5G ProSe

# Document for:	Approval

# Agenda Item:	7.5.3

1	Decision/action requested

The group is asked to discuss and agree on the proposal.

2	References

[1]	3GPP TR 32.846: “Study on charging aspects of Proximity-based Services in 5GS”.

3	Rationale

SA2 decided that UE-to-UE Relay is not pursued in Rel-17, this contribution updates some descriptions based on latest TS 23.304.

4	Detailed proposal

1st Modified Section

1	Scope

The present document studies on charging aspects of Proximity-based Services in 5GS based on TS 23.304 [11].

The charging aspects related to the Proximity-based Services in 5GS are covered within the scope:

-	Charging architecture, for supporting charging of ProSe Direct Discovery and Direct Communication related to Public Safety and commercial proximity service.

-	Charging principles, scenarios and potential requirements related to the ProSe service in 5GS;

-	Potential charging solutions for the Proximity-based Services in 5GS.

Next Modified Section

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

5G ProSe Direct Discovery: A procedure employed by a ProSe-enabled UE to discover other ProSe-enabled UEs in its vicinity by using only the capabilities of the two UEs with NR technology. See 3GPP TS 23.304 [11].

5G ProSe Direct Communication: A communication between two or more UEs in proximity that are ProSe-enabled, by means of user plane transmission using NR technology via a path not traversing any network node. See 3GPP TS 23.304 [11].

5G ProSe UE-to-Network Relay: A UE that provides functionality to support connectivity to the network for Remote UE(s). See 3GPP TS 23.304 [11].

5G ProSe UE-to-UE Relay: A UE that provides functionality to support connectivity between other UEs. See 3GPP TS 23.304 [11].

Remote UE: A 5G ProSe-enabled UE that communicates with a DN via a 5G ProSe UE-to-Network Relay. See 3GPP TS 23.304 [11].

Next Modified Section

5.1	High level ProSe architecture

The scope of the present document is Proximity-based Service charging in 5GS, related to architecture reference as follows:

-	Architecture reference model defined in TS 23.501 [7] are used as basis architecture for supporting ProSe in 5GS.

-	Architecture reference models defined in TS 23.304 [11] are used as reference architecture for supporting ProSe in 5GS.

Next Modified Section

5.2.1	General

The converged charging architectures for ProSe in 5GS should support charging for the following services:

-	ProSe Direct Discovery, including ProSe open Direct Discovery and restricted Direct Discovery, and

-	ProSe UE-to-Network Relay Discovery, and

-	ProSe Direct communication, including Broadcast and Groupcast Direct Communication and Unicast Direct Communication, and

-	ProSe Direct Communication via ProSe UE-to-Network Relay.

Details on the interfaces and functions can be found in TS 32.240 [5] for the general architecture components Nchf is described in TS 32.290 [6].

Next Modified Section

6.3.1	General description and assumptions

For NR based PC5, a QoS model similar to that defined in TS 23.501 [7] for Uu reference point is used, i.e. based on 5QIs, a PC5 QoS Flow is associated with a PC5 QoS Rule and the PC5 QoS parameters as defined in TS 23.287 [2] clauses 5.4.2, 5.4.3 and 5.4.4.

The UE may be configured with a set of default PC5 QoS parameters, as defined in TS 23.287 [2] clause 5.1.2.1. For NR based unicast, groupcast and broadcast mode communication over PC5, Per-flow QoS model for PC5 QoS management could be applied. This may need to be charged differently based on the monitored QoS for them respectively.

For Direct communication via ProSe UE-to-Network Relay, PC5 QoS handling to support end-to-end QoS is defined in TS 23.304 [11].

The end-to-end QoS for a relay service handling is supported as shown in figure below.

Figure 6.3.1-1: End-to-End QoS translation for Layer 3 UE-to-Network Relay

In case the QoS Flows setup are initiated by network, PCF or SMF decides the Uu part QoS parameters.

In case that the Remote UE requested dedicated PC5 QoS Flows when establishing the L2 Link over PC5, the Remote UE gets the QoS mapping on the Relay UE via PC5 and decides the PC5 part QoS parameters (i.e. PQI).

Next Modified Section

6.3.3.1	Key issue #3.1: QoS flow Based Charging for ProSe Direct Communication

This key issue is for investigating how to support QoS flow Based Charging for ProSe Direct Communication considering REQ-CH_PROSE_5GS_QBC-01.

This investigation covers the following:

-	determination of which entity/entities in the 5G system are suitable to provide the charging information for QoS flow Based Charging;

-	identification of the triggers for flow Based Charging for ProSe Direct Communication, and Direct Communication via UE-to-Network Relay.

Next Modified Section