3GPP TSG-SA5 Meeting #138-e 	S5-214180

# e-meeting, 23 - 31 August 2021

# Source:	Huawei

# Title:	Add requirements for management of PNI-NPN

# Document for:	Approval

Agenda Item:	6.4.1

1	Decision/action requested

Discuss and approve on the proposal.

2	References

[1]	TS 28.557 Management of non-public networks; Stage 1 and stage 2 v0.5.0

3	Rationale

Based on the concepts and use case of management of PNI-NPN, it is proposed to add corresponding requirements for management of PNI-NPN in draft TS 28.557 [1].

4	Detailed proposal

This document proposes the following changes in TS 28.557 [1].

1st Change

5.2.3	Requirements for management of PNI-NPN

REQ-PNIN-FUN-01 The 3GPP management system shall have the capability to collect NPN UE related data which may include MDT data and trace data.

REQ-PNIN-FUN-02 The 3GPP management system shall have the capability to provide NPN UE related data to authorized NPN service customer according to pre-defined agreements.

REQ-PNIN-FUN-03 The 3GPP management system should have the capability to support assignment and maintenance of CAG ID which identifies the CAG cells.

REQ-PNIN-FUN-04 The 3GPP management system shall have the capability to configure a CAG cell with a CAG list per PLMN Identity.

REQ-PNIN-FUN-05 The 3GPP management system should have the capability to manage the list of UEs that are allowed on the corresponding CAG.

REQ-PNIN-FUN-06 The 3GPP management system should have the capability to configure access policy of CAG cells.

NOTE: The access policy of CAG cells includes such as allowed days/time slots for NPN UEs that are allowed on the corresponding CAG cells.

REQ-PNIN-FUN-0X The 3GPP management system shall have the capability to provision a PNI-NPN using Network Slice as a Service (NSaaS), with the vertical taking the NSaaS customer role.

End of change