# 3GPP TSG-SA5 Meeting #138e	S5-214185

# 23 Aug to 31 Aug 2021, E-meeting

# Source:	China Telecom

# Title:	The measurement of the useful output for the 5GC NF

# Document for:	Approval

# Agenda Item:	6.5.1

1	Decision/action requested

The group is asked to discuss and agree on the proposal.

2	References

[1]	3GPP TR 28.813: "Study on new aspects of Energy Efficiency (EE) for 5G "

[2]	S5-213551: “Discussion paper on Useful Output Measurement Solution”

[x]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements"

3	Rationale

In TS 28.813[1], an Editor’s note was left:

The measurement of the useful output for the 5GC NF is FFS.

In S5-213551[2], the measurements of the useful output for the user plane and control plane of the 5GC were discussed, and the following proposal has been endorsed:

- Useful Output of the UPF may reuse the measurement of Data Volume of UPF.

- The measurement of mean number of the registered subscribers defined in TS 28.552 can be taken as the measurement of the Useful Output of the AMF.

- The measurement of mean number of the registered subscribers measured on AMF may be taken as the measurement of the Useful Output of the control plane of the 5GC.

- The measurement of mean of the number of PDU Sessions defined in TS 28.552 can be taken as the measurement of the Useful Output of the SMF.

- The measurement of mean of the number of PDU Sessions measured in SMF may be taken as the measurement of the Useful Output of the control plane of the 5GC.

This pCR is to define the measurement of the useful output for the 5GC NF based on the S5-213551[2].

4	Detailed proposal

Start of modifications (all new text)

4.2a.2.2	Potential solution #1 for measuring UsefulOutput5GC

4.2a.2.2.1	Introduction

This potential solution focuses on the definition of UsefulOuput5GC appeared in:

For this proposed solution, it can be applied to both User Plane Function (UPF) and Control Plane network functions that constitute the 5GC. And it is assumed that the definition of the useful output for UPF and control plane network function are different.

4.2a.2.2.2	Description

The useful output of a 5GC NF is defined as the capacity of the 5GC NF, and depending on the different type of 5GC NFs, it may be:

- throughput (e.g. bps) for 5GC User Plane Functions

- capacity (e.g. subscribers, sessions) for control plan 5GC NF.

The UsefulOuput5GC is composed of the useful output of all the 5GC NF. Therefore, the UsefulOuput5GC can be divided into UsefulOuput5GC, User Plane Functions, UsefulOutput5GC, subscribers and UsefulOutput5GC, sessions.

NOTE: Since each type of the useful output is one dimension used to describe the UsefulOuput5GC, when one or multiple types of useful output are used for calculation, such as the calculation of resource efficiency ratio, the calculation is made with respect to each type of useful output, i.e. the useful output of the same kind can be added together, while the useful output of different kinds cannot be added together.

In this proposed solution, the following network functions may be considered (see clause 4.2.2 of TS 23.501 [2]):

-	Access and Mobility Management Function (AMF)

-	Session Management Function (SMF)

-	User Plane Function (UPF)

The Useful Output of the UPF can be measured by the Data Volume of UPF defined in TS 28.552[x].

The UsefulOuput5GC, User Plane Functions can be taken as the measurement of the Useful Output of the UPF. Therefore, the UsefulOuput5GC, User Plane Functions is equivalent to the Data Volume of UPF defined in TS 28.552[x].

The Useful Output of the AMF can be measured by the mean number of the registered subscribers defined in TS 28.552[x]. The UsefulOutput5GC, subscribers is taken as the measurement of the Useful Output of the AMF. Therefore, the UsefulOutput5GC, subscribers is equivalent to the mean number of the registered subscribers of the AMF.

The Useful Output of the SMF can be measured by the mean of the number of PDU Sessions defined in TS 28.552 [x].  The UsefulOutput5GC, sessions is taken as the measurement of the Useful Output of the SMF. Therefore, the UsefulOutput5GC, sessions is equivalent to the mean of the number of PDU Sessions of SMF.

End of modifications