3GPP TSG-SA5 Meeting #138-e 	S5-214189

# e-meeting, 23 - 31 August 2021

# Source:	ZTE

# Title:	Add deployment examples of NG-RAN sharing with multiple SSBs

# Document for:	Approval

Agenda Item:	6.5.9

1	Decision/action requested

The group is asked to discuss and approve the proposals.

2	References

[1]	3GPP TS 38.300: “NR; NR and NG-RAN Overall description; Stage-2”

3	Rationale

The RAN groups have defined some features which can be used to support NG-RAN sharing, but there is no description to show how to use these features in the NG-RAN sharing deployment.

This contribution proposes to add the deployment approach example of MOCN NG-RAN sharing with multiple SSBs.

4	Detailed proposal

X. NG-RAN sharing deployment approach examples

X.a Deployment approach example of MOCN NG-RAN sharing with multiple SSBs

The feature multiple SSBs in a carrier is introduced in TS 38.300. The following figure is an example of multiple SSBs in a carrier from TS 38.300, Annex B.2:

The Multiple SSBs in a carrier feature can be used to support NG-RAN network sharing, the following figure is a deployment example of NG-RAN network sharing with Multiple SSBs in a carrier:

In this example, there are two SSBs in a carrier. According to the definition of RAN specification, the system information including multiple PLMNs in SIB1 of cells associated to different CD-SSBs can be different, so cells associated to the different CD-SSBs can be used by different operators.

So, with multiple SSBs in a carrier, one physical gNB-DU can be mapped to different logical gNB-DUs. This provides the possibility for the POP to manage the shared resources individually based on its own policies.