3GPP TSG-SA5 Meeting #138-e 	S5-214226

# e-meeting, 23 - 31 August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.541 | CR | 0540 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add table to introduce SLA relevant KPIs and performance measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | EMA5SLA |  |  |  |  |  | Date: |  |  | 2021-08-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Lack of information of SLA relevant KPIs and performance measurements and some editorial errors exist in the annex L.1 and L.2 text. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New table L.2.2 with information of SLA relevant KPIs and performance measurements and some editorial changes for the existing text. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex L.1, L.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | None |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st modified section

Annex L (normative):
Relation of GSMA GST, ServiceProfile and SliceProfile

L.1	General

This annex describes the relation between GSMA GST [50] and information model ServiceProfile and SliceProfile.

L.2	GSMA GST, ServiceProfile and sliceProfile

The GSMA GST is used as the SLA information for the communication between the NSC (e.g. vertical industry) and the NSP. The SLA requirements can be fulfilled from management aspect and control aspect in a coordinated way. The SLS includes ServiceProfile information model.

As shown in figure L.2.1, the GST parameters [50] are translated and used as input to NRM ServiceProfile.  The ServiceProfile, which is entered by the network slice consumer, defines the wanted service requirements for a wanted service. The ServiceProfile requirements are then translated to a Top SliceProfile (including specific TopSliceSubnetProfile attributes). Based on Top SliceProfile requirements, the corresponding requirements for the dedicated domain specific network slice subnets in the RAN/TN/5GC are defined. For example, a 5GC SliceProfile (including specific CNSliceSubnetProfile attributes) is used to carry 5GC domain requirements, a NG-RAN SliceProfile (including specific RANSliceSubnetProfile attributes) is used to carry NG-RAN domain requirements, and the TN requirements are derived and provide input to TN domain. Some of the information in 5GC SliceProfile and NG-RAN SliceProfile can be translated to configurable parameters of network function for the control plane SLA support purpose.

As shown in Table L.2.1 some of the attributes in 5GC SliceProfile and NG-RAN SliceProfile parameters can be translated to configurable parameters related to network function behaviour to satisfy SLA of the service in the control plane. While other information (e.g delay tolerance, deterministic communication support) in 5GC SliceProfile and NG-RAN SliceProfile are kept at OAM domain and is used to determine the overall behaviour of the network slice.

The following table show the translation of GST attributes.

Table L.2.1: GST translation

| GST Attributes | ServiceProfile Parameter | SliceProfile Parameter |  |  | Configurable Parameter |
| --- | --- | --- | --- | --- | --- |
|  |  | TopSlice SubnetProfile | RANSlice SubnetProfile | CNSliceSubnetProfile |  |
| Maximum number of UEs | maxNumberofUEs | maxNumberofUE | maxNumberofUEs | maxNumberofUEs | attributes in NSACF |
| Maximum number of PDU sessions | maxNumberofConns | maxNumberofPDUSessions | N/A | maxNumberofPDUSessions | TBD |
| Downlink maximum throughput per UE | dLThptPerUE | dLThptPerUE | dLThptPerUEPerSubne | dLThptPerUEPerSubne | TBD |
| Uplink maximum throughput per UE | uLThptPerUE | uLThptPerUE | uLThptPerUEPerSubnet | uLThptPerUEPerSubnet | TBD |

Editor's note:	The list of exact configurable parameters is to be revisited depending on the requirements from SA2 and RAN WGs.

NOTE:	Void.

A number of SLA relevant KPIs and performance measurements are shown in the Table L.2.2 below.

Table L.2.2: SLA relevant KPIs and performance measurements

| GST Attributes | ServiceProfile Parameter | KPIs and performance measurements |
| --- | --- | --- |
| Maximum number of PDU sessions | maxNumberofPDUSessions | See clause 6.4.5 Maximum number of PDU sessions of network slice in TS 28.554 [27] and clause 5.1.1.5 PDU Session Management in TS 28.552 [69]. |
| Downlink maximum throughput per UE | dLThptPerUE | See clause 5.1.1.3.2 Distribution of DL UE throughput in gNB in TS 28.552 [69]. |
| Uplink maximum throughput per UE | uLThptPerUE | See clause 5.1.1.3.4 Distribution of UL UE throughput in gNB in TS 28.552 [69]. |

Figure L.2.1 Relation between GSMA GST, ServiceProfile and SliceProfile

End of modified section