3GPP TSG SA WG5 Meeting 138-e		S5-214302

# electronic meeting, online, 23 - 31 August 2021

# Source:	Nokia

# Title:	Add MDA Reporting

# Document for:	Approval

Agenda Item:	6.4.18

# 1	Decision/action requested

This contribution is for approval.

# 2	References

[1]	TR 28.809 Management and orchestration; Study on enhancement of Management Data Analytics (MDA)

# 3	Rationale

This contribution introduces an MDA Reporting that enables the MDA MnS consumer to receive analytics.

This MDA Reporting can be applicable to all analytics use cases described in [1].

# 4	Detailed proposal

1st Modified Section

## X.Y	MDA Reporting

### X.Y.1	Description

The MDA reporting allows any MDA MnS consumer to receive a MDA report.

### X.Y.2	Use Case

The MDA MnS conusmer can receive a MDA report that contains a name, which corresponds to a use case. The MDA report may contain one or more results that can be numeric, recommendations or root cause analysis.

The MDA reporting follows the requested reporting method, e.g., file-based reporting or streaming. The MDA report indicates the start time and duration of the MDA results or the time until the MDA results are valid. The MDA report can point out the geographic location and/or the target objects, e.g., managed elements, related to the MDA result.

Any MDA report may be provided upon request or when specified filter conditions are met, e.g. threshold crossing.

### X.Y.3	Requirements for Requesting MDA

REQ-MDA_MDA-REP-1 The MDA MnS producer shall provide a MDA report to any authorized MDA MnS consumer based on the requested MDA reporting method.

REQ-MDA_MDA-REP-2 The MDA MnS producer shall provide a MDA report to any authorized MDA MnS consumer, while indicating the type of analytics, the geographic location and/or the target objects related to the MDA result.

REQ-MDA_MDA-REP-3 The MDA MnS producer shall provide a MDA report to any authorized MDA MnS consumer, while indicating the start time and duration related to the MDA results and/or the time until the MDA results are valid.

REQ-MDA_MDA-REP-4 The MDA MnS producer shall provide a MDA report that contains one or more numeric, recommendations or root cause analysis output results to any authorized MDA MnS consumer.

REQ-MDA_MDA-REP-5 The MDA MnS producer shall provide a MDA report to any authorized MDA MnS consumer upon request or when specified filter conditions are met.

End of Modified Sections