3GPP TSG SA WG5 Meeting 138-e		S5-214348

# electronic meeting, online, 23 - 31 August 2021

# Source:	Nokia

# Title:	Add MDA Reporting Control

# Document for:	Approval

Agenda Item:	6.4.18

# 1	Decision/action requested

This contribution is for approval.

# 2	References

[1]	TR 28.809 Management and orchestration; Study on enhancement of Management Data Analytics (MDA)

# 3	Rationale

This contribution introduces an MDA reporting control description, use case and requirements that enables the MDA MnS consumer to request analytics indicating certain reporting control attributes to the MDA MnS producer.

This MDA reporting control can be applicable to all analytics use cases described in [1], which request an MDA report.

# 4	Detailed proposal

1st Modified Section

## X.Y	MDA Reporting Control

### X.Y.1	Description

The MDA reporting control allows any MDA MnS consumer to request and modify the contents of the MDA report.

### X.Y.2	Use Case

The MDA MnS conusmer can request the MDA MnS producer a MDA report by indicating the name of the MDA report, which corresponds to a use case, and its preferences on reporting method, e.g. file-based reporting or streaming, the geographic location and/or the target objects, e.g., managed elements. The MDA MnS conusmer may control these reporting attributes and can have the capability to modify them.

The MDA MnS conusmer can inidcate when to receive a MDA report, i.e., upon request or may specify filter conditions to be met, e.g. threshold crossing, before receiving a MDA report. The MDA MnS conusmer can request a MDA report that contains numeric output results, recommendation options or root case analysis.

The MDA MnS consumer may also be capable to deactivate the MDA report.

The MDA MnS consumer can be informed with an acknowledgment that includes the MDA report name if the request was successful. If the request was not successful the consumer is informed about potential errors indicating the reasons.

### X.Y.3	Requirements for Requesting MDA

REQ-MDA_MDA-REQ-1 The MDA MnS producer shall support the capability allowing any authorized MDA MnS consumer to request an MDA reporting, while indicating its preferences on reporting method, type of analytics, geographic location and/or the target objects.

REQ-MDA_MDA-REQ-2 The MDA MnS producer shall support the capability allowing any authorized MDA MnS consumer to modify MDA reporting, while indicating its preferences on reporting method, type of analytics, geographic location and/or the target objects.

REQ-MDA_MDA-REQ-3 The MDA MnS producer shall support the capability allowing any authorized MDA MnS consumer to specify filter conditions for MDA reporting.

REQ-MDA_MDA-REQ-4 The MDA MnS producer shall support the capability allowing any authorized MDA MnS consumer to specify the type of MDA output result.

REQ-MDA_MDA-REQ-5 The MDA MnS producer shall support the capability allowing any authorized MDA MnS consumer to express that the requested MDA reporting is no longer needed.

REQ-MDA_MDA-REQ-6 The MDA MnS producer shall inform any authorized MDA MnS consumer whether the MDA reporting request was successfully processed and any errors that may have occurred.

End of Modified Sections