3GPP TSG-SA5 Meeting #138-e 	S5-214360

# e-meeting, 23 - 31 August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.541 | CR | 0550 | rev | <Rev#> | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add translated attributes for TN domain |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMA5SLA |  |  |  |  |  | Date: |  |  | 2021-08-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This CR proposes to add the specific attribute derived from the SLA requirements for TN domain. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a table of slice attribute for TN domain slice profile. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete ServiceProfile solution. The TN slice attributes will be missing in this specificaiton even though the TN domain is out of 3GPP scope. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# L.2	GSMA GST, ServiceProfile and sliceProfile

The GSMA GST is used as the SLA information for the communication between the NSC (e.g. vertical industry) and the NSP. The SLA requirements can be fulfilled from management aspect and control aspect in a coordinated way. The SLS includes ServiceProfile information model.

As shown in figure L.2.1, the GST parameters [50] are translated and used as input to NRM ServiceProfile.  The ServiceProfile, which is entered by the network slice consumer, defines the wanted service requirements for a wanted service. The ServiceProfile requirements are then translated to a Top SliceProfile (including specific TopSliceSubnetProfile attributes).  Based on Top SliceProfile requirements, the corresponding requirements for the dedicated domain specific network slice subnets in the RAN/TN/5GC are defined. For example, a 5GC SliceProfile (including specific CNSliceSubnetProfile attributes) is used to carry 5GC domain requirements, a NG-RAN SliceProfile  (including specific RANSliceSubnetProfile attributes) is used to carry NG-RAN domain requirements, and the TN requirements are derived and provide  input to TN domain. Some of the information in 5GC SliceProfile  and NG-RAN SliceProfile can be translated to configurable parameters  of network function for the control plane SLA support purpose. The TN slice profile is defined to ensure the integrity of end-to-end service profile and can be a reference suggestion for IETF and ITU. Whether TNSliceSubnetProfile is an IOC or dataType is outside the scope of the present document. Table L.2.x shows some suggested attributes can be transferred into the TN domain.

Table L.2.x: Attributes for TN domain

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| resourceSharingLevel | O | T | T | F | T |
| availability | O | T | T | F | T |
| dLThptPerUEPerSlicesubnet | O | T | T | F | T |
| uLThptPerUEPerSliceSubnet | O | T | T | F | T |
| maxPktSize | O | T | T | F | T |
| synchronicity | O | T | T | F | T |
| latency | O | T | T | F | T |
| delayTolerance | O | T | T | F | T |
| packetLossRate | O | T | T | F | T |
| jitter | O | T | T | F | T |
| reliability | O | T | T | F | T |

As shown in Table L.2.1 some of the attributes in 5GC SliceProfile and NG-RAN SliceProfile parameters can be translated to configurable parameters related to network function behaviour to satisfy SLA of the service in the control plane. While other information (e.g delay tolerance, determistic communication support) in 5GC SliceProfile and NG-RAN SliceProfile are kept at OAM domain and is used to determine the overall behaviour of the network slice.

The following table show the translation of GST attributes.

Table L.2.1: GST translation

| GST Attributes | ServiceProfile Parameter | SliceProfile Parameter |  |  | Configurable Parameter |
| --- | --- | --- | --- | --- | --- |
|  |  | TopSlice SubnetProfile | RANSlice SubnetProfile | CNSlice SubnetProfile |  |
| Maximum number of UEs | maxNumberofUEs | maxNumberofUE | maxNumberofUEs | maxNumberofUEs | attributes in NSACF |
| Maximum number of PDU sessions | maxNumberofConns | maxNumberofPDUSessions | N/A | maxNumberofPDUSessions | TBD |
| Downlink maximum throughput per UE | dLThptPerUE | dLThptPerUE | dLThptPerUEPerSubne | dLThptPerUEPerSubne | TBD |
| Uplink maximum throughput per UE | uLThptPerUE | uLThptPerUE | uLThptPerUEPerSubnet | uLThptPerUEPerSubnet | TBD |

Editor's note:	The list of exact configurable parameters is to be revisited depending on the requirements from SA2 and RAN WGs.

NOTE:	Void.

Figure L.2.1 Relation between GSMA GST, ServiceProfile and SliceProfile