3GPP TSG-SA5 Meeting #138-e 	S5-214368

# e-meeting 23 - 31 August 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.313 | CR | - | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DAPS HO requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | E_HOO |  |  |  |  |  | Date: |  |  | 2021-08-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Add requirements and use case for the development of management of DAPS HO. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specification level requrements and use case for DAPS HO management added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 6.1.1.X, 6.4.1.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | This is input to the Rel-17 28.313 draft CR for WI E_HOO. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ANR	Automatic Neighbour Relation

DAPS	Dual Active Protocol Stack

NCR	Neighbour Cell Relation

NG-RAN	Next Generation Radio Access Network

Next change

#### 6.1.1.X	DAPS handover management

REQ-DDAPSHO-FUN-1	The producer of NF provisioning MnS should have the capability allowing an authorized consumer to enable or disable DAPS handover from one cell to another cell.

REQ-DDAPSHO-FUN-2	The producer of NF provisioning MnS should have the capability allowing an authorized consumer to configure parameters for DAPS handover.

REQ-DDAPSHO-FUN-3	The producer of NF performance assurance MnS should have the capability to produce measurements related to DAPS handover.

Next change

#### 6.4.1.X	DAPS HO (Dual Active Protocol Stack Handover)

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To configure DAPS HO parameters in cells in order to improve DAPS HO performance. |  |
| Actors and Roles | D-SON management function to support the DAPS HO function. |  |
| Telecom resources | - gNB; - The producer of provisioning MnS. |  |
| Assumptions | N/A |  |
| Pre-conditions | - 5G NR cells are in operation. - DAPS HO is not in operation from the source cell to the target cell. |  |
| Begins when | The D-SON management function intends to enable DAPS HO from the source cell to the target cell. |  |
| Step 1 (M) | The D-SON management function requests the producer of NF provisioning MnS to configure parameters for the management of DAPS HO on the source cell. |  |
| Step 2 (M) | The D-SON management function requests the producer of provisioning MnS to enable DAPS HO from a source cell to a target cell. |  |
| Step 3 (M) | The DAPS HO function detects handover issues (e.g. too late DAPS HO, too early DAPS HO and DAPS HO to a wrong cell) by analysing reports from UEs and network side information, and acts to mitigate the DAPS HO issues by adjusting DAPS HO related parameters. |  |
| Step 4 (M) | The D-SON management function collects DAPS HO related measurements and analyses them to evaluate the DAPS HO performance. |  |
| Step 5 (M) | If the D-SON management function does not find the DAPS HO performance satisfactory, it updates the configuration parameters for the DAPS HO function. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | DAPS HO is in operation from the source cell to the target cell. |  |
| Traceability | REQ-DDAPSHO-FUN-1, REQ-DDAPSHO-FUN-2, REQ-DDAPSHO-FUN-3 |  |

End of changes