3GPP TSG-SA5 Meeting #138-e	S5-214455

# Online, , 23rd Aug 2021 - 31st Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.541 | CR | 0560 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-17 CR 28.541 Edge NRM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung Electronics Benelux BV |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | ECM |  |  |  |  |  | Date: |  |  | 2021-08-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The NRM fragment for edge need to be defined as depicted in 28.814. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The introduction of edge NRM fragment |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Impossible edge management solution |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Y (new clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First modification

Y.		Edge NRM

Y.1		Information Model definitions for Edge NRM

## Y.1	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |
| TS 28.622 [30], IOC, Top | Top |
| TS 28.622 [30], IOC, SubNetwork | SubNetwork |

## Y.2	Class diagram

### Y.2.1	Relationships

### Y.2.2	Inheritance

## Y.3	Class definition

Y.3.1		EDNFunction

Y.3.1.1 		Definition

This IOC represent the properties of a EDN in a 3GPP network. For more information about EDN, see 3GPP TS 23.558.

Y.3.1.2		Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNInfoList | M | T | T | F | T |
| eDNservingLocation | M | T | T | F | T |
| virtualResCapInfo | M | T | T | F | T |
| eESFunctionRef | M | T | T | F | T |
| eASFunctionRef | M | T | T | F | T |

#### Y.3.1.3	Attribute constraints

#### Y.3.1.4	Notifications

The common notifications defined in subclause Y.3 are valid for this IOC, without exceptions or additions.

Y.3.1		EASFunction

Y.3.1.1 		Definition

This IOC represent the properties of a EAS in a 3GPP network. For more information about EAS, see 3GPP TS 23.558.

Y.3.1.2		Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| eASRequirements | M | T | T | F | T |
| pLMNInfoList | M | T | T | F | T |
| eASProfile | M | T | T | F | T |
| eESFunctionRef | M | T | T | F | T |

#### Y.3.1.3	Attribute constraints

#### Y.3.1.4	Notifications

The common notifications defined in subclause Y.3 are valid for this IOC, without exceptions or additions.

Y.3.2		EASRequirements <<datatype>>

Y.3.2.1 		Definition

This datatype represent the deployment requirements of an EAS which need to be considered during EASFunction instantiation.

Y.3.2.2		Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| requiredEASservingLocation | M | T | F | F | T |
| requiredVR | O | T | F | F | T |

#### Y.3.2.3	Attribute constraints

#### Y.3.2.4	Notifications

The common notifications defined in subclause Y.3 are valid for this IOC, without exceptions or additions.

Y.3.3		ServingLocation <<datatype>>

Y.3.3.1 		Definition

This datatype represent the location which is to be served by the node.

Y.3.3.2		Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| geographicalLocation | CM | T | F | F | T |
| topologicalLocation | CM | T | F | F | T |

#### Y.3.3.3	Attribute constraints

| Name | Definition |
| --- | --- |
| geographicalLocation Support Qualifier | Condition: either geographicalLocation or topologicalLocation shall be present. |
| topologicalLocation Support Qualifier | Condition: either geographicalLocation or topologicalLocation shall be present. |

#### Y.3.3.4	Notifications

The common notifications defined in subclause Y.3 are valid for this IOC, without exceptions or additions.

Y.3.4		EASProfile <<datatype>>

Y.3.1.1 		Definition

This IOC represent the EAS profile, see 3GPP TS 23.558.

Y.3.1.2		Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| eASservingLocation | M | T | T | F | T |
| eASProvider | O | T | T | F | T |
| eASType | O | T | T | F | T |
| eASDescription | O | T | T | F | T |

#### Y.3.1.3	Attribute constraints

#### Y.3.1.4	Notifications

The common notifications defined in subclause Y.3 are valid for this IOC, without exceptions or additions.

Y.3.5		GeoLoc <<datatype>>

Y.3.5.1 		Definition

This datatype represent the geographical location.

Y.3.5.2		Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| geoPoint | CM | T | T | F | T |
| civicAddress | CM | T | T | F | T |

#### Y.3.5.3	Attribute constraints

| Name | Definition |
| --- | --- |
| geoPoint Support Qualifier | Condition: either geoPoint or civicAddress shall be present. |
| civicAddress Support Qualifier | Condition: either geoPoint or civicAddress shall be present. |

#### Y.3.5.4	Notifications

The common notifications defined in subclause Y.3 are valid for this IOC, without exceptions or additions.

Y.3.6		GeoP <<datatype>>

Y.3.6.1 		Definition

This datatype represent the geographical location.

Y.3.6.2		Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| lat | M | T | T | F | T |
| long | M | T | T | F | T |

#### Y.3.6.3	Attribute constraints

#### Y.3.6.4	Notifications

The common notifications defined in subclause Y.3 are valid for this IOC, without exceptions or additions.

Y.3.7		TopoLoc <<datatype>>

Y.3.7.1 		Definition

This datatype represent the topological location.

Y.3.7.2		Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| cellID | O | T | T | F | T |
| tAI | O | T | T | F | T |
| pLMNID | O | T | T | F | T |

#### Y.3.7.3	Attribute constraints

#### Y.3.7.4	Notifications

The common notifications defined in subclause Y.3 are valid for this IOC, without exceptions or additions.

## Y.2	Attribute definition

Y.2.1		Attribute Properties

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| EASFunction.pLMNInfoList | It defines which PLMNs that can be served by the EASFunction, and which S-NSSAIs can be supported by the EASFunction for corresponding PLMN in case of network slicing feature is supported. | type: PLMNInfo multiplicity: 1..* isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| eASREquirements | It defines the deployment requirement of an EAS. | type: EASRequirements multiplicity: isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| eASservingLocation | It defines the serving location for an EAS. | type: ServingLocation multiplicity: 1..* isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| eASProvider | It defines the EAS Provider Identifier, see 3GPP TS 23.558. | type: String multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| eASType | It defines the EAS Type, see 3GPP TS 23.558. | type: String multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| eASDescription | It defines the EAS description, see 3GPP TS 23.558. | type: Sting multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| requiredEASservingLocation | It defines the location where the EAS service should be available. | type: ServingLocation multiplicity: 1..* isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| requiredVR | It defines the virtual resources requirements of the EAS. | type: FFS multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| eESFunctionRef | This will define the EES(s) serving this particular EAS. This holds a DN of EESFunction relating to the EASFunction instance. | type: DN multiplicity: 1…* isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| topologicalLocation | This refers to the Topological Service Area, see 3GPP TS 23.558. | type: TopoLoc multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| geographicalLocation | This refers to the Geographical Service Area, see 3GPP TS 23.558. | type: GeoLoc multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| eASProfile | This refers to the EAS profile, see 3GPP TS 23.558 | type: EASProfile multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| geoPoint | This defines the location in terms of latitude and longitude. | type: geoP multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| lat | This defines the single latitude coordinate. | type: Float multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| long | This defines the single longtitude coordinate. | type: Float multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| civicAddress | This defines the location in terms of a civic address | type: String multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| cellID | The list of cell IDs defining the topological service area | type: String multiplicity: 1..* isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| tAI | The list of Tracking Area IDs defining the topological service area | type: String multiplicity: 1..* isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| pLMNID | The list of PLMN IDs defining the topological service area | type: String multiplicity: 1..* isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| eDNservingLocation | This defines the serving location of the EDN. | type: ServingLocation multiplicity: 1..* isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| virtualResCapInfo | The parameter defined the total and available virtual resource capacity available in the EDN. This would contain the virtual compute, storage and network resource capacity. | type: FFS multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| eASFunctionRef | This will define the the EAS(s) available in the EDN. This holds a DN of EASFunction relating to the EDNFunction instance. | type: DN multiplicity: 1…* isOrdered: N/A isUnique: True defaultValue: None isNullable: False |

Y.3 		Common Notifications

### Y.3.1	Alarm notifications

This clause presents a list of notifications, defined in TS 28.532 [35], that an MnS consumer may receive. The notification header attribute objectClass/objectInstance shall capture the DN of an instance of a class defined in the present document.

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyNewAlarm | M | -- |
| notifyClearedAlarm | M | -- |
| notifyAckStateChanged | M | -- |
| notifyAlarmListRebuilt | M | -- |
| notifyChangedAlarm | O | -- |
| notifyCorrelatedNotificationChanged | O | -- |
| notifyChangedAlarmGeneral | O | -- |
| notifyComments | O | -- |
| notifyPotentialFaultyAlarmList | O | -- |

### Y.3.2	Configuration notifications

This clause presents a list of notifications, defined in TS 28.532 [35], that an MnS consumer may receive. The notification header attribute objectClass/objectInstance shall capture the DN of an instance of a class defined in the present document.

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyMOICreation | O | -- |
| notifyMOIDeletion | O | -- |
| notifyMOIAttributeValueChanges | O | -- |
| notifyEvent | O | -- |

### Y.3.3	Threshold Crossing notifications

This clause presents a list of notifications, defined in TS 28.532 [35], that an MnS consumer may receive. The notification header attribute objectClass/objectInstance shall capture the DN of an instance of a class defined in the present document.

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyThresholdCrossing | M |  |

End of first modification