3GPP TSG-SA5 Meeting #138-e	S5-214476

# Online, , 23rd August 2021 – 31th August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.622 | CR | Draft CR | rev | - | Current version: | 16.8.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-17 Input to DraftCR 28.622 Add solution for data discovery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MADCOL |  |  |  |  |  | Date: |  |  | 2021-08-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Solution for discovering management data is defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | DataCatalogue and DataRecord IOCs for data discovery solution are added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2, 4.3.XX, 4.3.XY |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First modification

## 4.2	Class diagrams

### 4.2.1	Relationships

This clause depicts the set of classes (e.g. IOCs) that encapsulates the information relevant for this IRP. This clause provides the overview of the relationships of relevant classes in UML. Subsequent clauses provide more detailed specification of various aspects of these classes.

The following figure shows the containment/naming hierarchy and the associations of the classes defined in the present document. See Annex A of a class diagram that combines this figure with Figure 1 of [2], the class diagram of UIM.

NOTE 1:	ManagedElement may be contained either

-	in a SubNetwork (since SubNetwork inherits from Domain_ and ManagedElement inherits from ManagedElement_ and Domain_ name-contained ManagedElement_ as observed in the figure of Annex A) or

-	in a MeContext instance as observed by the above figure or in the figure of Annex A.

This either-or relation cannot be shown by using an {xor} constraint in the above figure.

ManagedElement may also have no parent instance at all.

NOTE 2:	Void

NOTE 3:	If the configuration contains several instances of SubNetwork, exactly one SubNetwork instance shall directly or indirectly contain all the other SubNetwork instances.

NOTE 4:	The SubNetwork instance not contained in any other instance of SubNetwork is referred to as "the root SubNetwork instance".

NOTE 5:	ManagementNode shall be contained in the root SubNetwork instance.

NOTE 6:	If contained in a SubNetwork instance, IRPAgent shall be contained in the root SubNetwork instance.

NOTE 7:	For a clarification on the choice of containment of the IRPAgent (since it has three possible parents), see the def. of IRPAgent.

NOTE 8:	Cardinality * is identical to multiplicity 0..*.

# Figure 4.2.1-1: Containment/Naming and Association NRM fragment

Each Managed Object is identified with a Distinguished Name (DN) according to 3GPP TS 32.300 [13] that expresses its containment hierarchy. As an example, the DN of a ManagedElement instance could have a format like:

SubNetwork=Sweden,MeContext=MEC-Gbg-1,ManagedElement=RNC-Gbg-1.

NOTE 8:	Void

NOTE 9:	Void

Figure 4.2.1-2: Vendor specific data container NRM fragment

Figure 4.2.1-3: PM control NRM fragment

Figure 4.2.1-4: Threshold monitoring control NRM fragment

Figure 4.2.1-5: Notification subscription and heartbeat notification control NRM fragment

Figure 4.2.1-6: FM control NRM fragment

Figure 4.2.1-7: Trace control NRM fragment

Figure 4.2.1-8: Data Discovery NRM fragment

### 4.2.2	Inheritance

# This clause depicts the inheritance relationships.

# Figure 4.2.2-1: Inheritance Hierarchy NRM fragment

# Figure 4.2.2-2: PM control NRM fragment

# Figure 4.2.2-3: Threshold monitoring control NRM fragment

# Figure 4.2.2-4: Notification subscription and heartbeat notification control NRM fragment

Figure 4.2.2-5: FM control NRM fragment

Figure 4.2.2-6: Trace control NRM fragment

Figure 4.2.2-7: Trace control NRM fragment

Next modified section

### 4.3.XX	DataCatalogue

#### 4.3.XX.1	Definition

This IOC represents a set of managed entities. There may be zero or more instances of a DataRecord.

DataCatalogue can be contained only by SubNetwork IOC.  A SubNetwork IOC can contain only one instance of DataCatalogue.

| Referenced TS | Requirement label | Comment |
| --- | --- | --- |
| TS 28.537 [32] | REQ-MDMS-2 |  |
| TS 28.537 [32] | REQ-MDMED-4 |  |

#### 4.3.XX.2	Attributes

The DataCatalogue IOC includes the attributes inherited from Top IOC (defined in clause 4.3.29)

#### 4.3.XX.3	Attribute constraints

None.

#### 4.3.XX.4	Notifications

Editor's note:

Notifications support are ffs.

### 4.3.XY	DataRecord

#### 4.3.XY.1	Definition

This IOC represents a management data available in a DataCatalogue.

DataRecord can be contained only by DataCatalogue IOC and there may be zero or more instances of a DataRecord IOC contained by DataCatalogue IOC

| Referenced TS | Requirement label | Comment |
| --- | --- | --- |
| TS 28.537 [32] | REQ-MDMS-2 |  |
| TS 28.537 [32] | REQ-MDMED-4 |  |

#### 4.3.XY.2	Attributes

The DataCatalogue IOC includes the attributes inherited from Top IOC (defined in clause 4.3.29) and the following attributes:

| Attribute Name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dataRecordType | M | T | T | F | T |
| dataSpace | M | T | T | F | T |
| fileFormat | CM | T | T | F | T |
| messageFormat | CM | T | T | F | T |
| dataRepository | O | T | T | F | T |

Editor's note:

Other attributes are ffs.

#### 4.3.XY.3	Attribute constraints

| Name | Definition |
| --- | --- |
| fileFormat Support Qualifier | Condition: Either fileFormat or messageFormat must be specified. fileFormat and messageFormat must not be specified by the same DataRecord instance |
| messageFormat Support Qualfied | Condition: Either fileFormat or messageFormat must be specified. fileFormat and messageFormat must not be specified by the same DataRecord instance |

#### 4.3.XY.4	Notifications

Editor's note:

Notifications support are ffs.

End of modifications