3GPP TSG-SA5 Meeting #138-e 	S5-214665

# e-meeting, 23 - 31 August 2021

# Source:	3GPP SA2

# Title:	LS Reply on the offline charging only indication

# Document for:	Information, Discussion

Agenda Item:  7.1

3GPP TSG-WG SA2 Meeting #146E e-meeting  	S2-2106558

Elbonia, August 16 – 27, 2021	(revision of S2-2106150)

# Title:	LS Reply on the offline charging only indication

# Response to:	LS C3-213546 on Clarifications on the offline charging only indication from CT3

# Release:	Rel-17

# Work Item:	en5GPccSer17

Source:	SA2

To:	SA5

Cc:	CT3

Contact Person:

Name:	Haiyang, Sun

Tel. Number:

E-mail Address:	sunhaiyang3 AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

SA2 would like to thank CT3 for the LS on Clarifications on the offline charging only indication in C3-213546. SA2 discussed the questions and agreed the following answers.

Question 1: When the "PDU session with offline charging only" indication is provisioned by the PCF for a PDU session within the SM Policy Decision to indicate that the online charging method shall never be used for any PCC rule of the PDU session, does this also mean that the offline charging method shall be used for all the PCC rules of the PDU session?

SA2 answer: No, the "PDU session with offline charging only" indication does not mean that the offline charging method shall be used for all PCC rules of the PDU session. Indeed, a PCC rule may not be subject to any end user charging, i.e. neither online nor offline charging should be applied to such PCC rule, and this is considered as a possible value for the charging method at PCC rule level.

Question 2: If the answer to Question 1 is yes, then should the PCF be allowed to also provision the offline charging method as a default charging method for the PDU session when it provisions the "PDU session with offline charging only" indication? In other words, does the provisioning by the PCF of the "PDU session with offline charging only" indication mean that the offline charging method does not need to be provisioned by the PCF as the default charging method for the PDU session or as the charging method for any PCC rule of the PDU session?

SA2 answer: The answer to Question 1 is no. However, SA2 would like to provide the following additional clarifications:

-  When the "PDU session with offline charging only" indication is provisioned by the PCF,  PCC Rules may still include the charging method. The PCF cannot set the charging method for a PCC rule to “online” but may provision the charging method for a PCC rule within the PDU session to either “offline” or “neither” (neither “online” nor “offline”).

- The SMF may not support the “PDU session with offline charging only’, here SA2 understanding is that the SMF does not support ‘OfflineChargingOnly’ service offered by CHF. The PCF does not set the "PDU session with offline charging only" indication when the SMF does not support it.

Question 3: Can the offline charging only indication be locally configured in the SMF and apply if no charging method is provided by the PCF (e.g. local policies indicate so)? If the answer is yes, then how to handle that the PCF may not be aware of it and would hence not be able to enforce that for all the PCC rules of the PDU session the online charging method cannot be set?

SA2 answer: Yes, the SMF can have the PDU Session with offline only indication locally configured per DNN,S-NSSAI that will be used if dynamic PCC is not deployed.. Allowing a local configuration of it at the SMF allows the SMF to control the values when the PCF does not control the PDU Session. When dynamic PCC is deployed, the offline charging only indication cannot be locally configured in the SMF, but the PCF can only provide it if the SMF supports it for the PDU Session.

SA2 would like to adopt the corresponding specification once the clarification is consolidated with SA5..

2. Actions:

To SA5 groups.

ACTION: 	SA2 kindly asks SA5 to provide feedback on the above response.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #147E		October 18 – 22, 2021			Elbonia

TSG-SA2 Meeting #148E		November 15 – 19, 2021			Elbonia