| 3GPP TR 32.846 V1.1.0 (2021-08) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Charging management; Study on charging aspects of Proximity-based Services (ProSe) in 5GS (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	7

1	Scope	9

2	References	9

3	Definitions of terms, symbols and abbreviations	9

3.1	Terms	9

3.2	Symbols	10

3.3	Abbreviations	10

4	Concepts and overview	10

5	Architecture considerations	11

5.1	High level ProSe architecture	11

5.2	Potential converged charging architectures for ProSe	11

5.2.1	General	11

5.2.2	ProSe Direct Discovery converged charging architecture	11

5.2.3	ProSe Direct Communication converged charging architecture	11

6	Charging scenarios and key issues	11

6.1	Scenario 1: 5GS charging for ProSe Direct Discovery	11

6.1.1	General description and assumptions	11

6.1.2	Potential charging requirements	12

6.1.3	Key issues	12

6.1.3.1	Key issue #1.1: Converged charging for ProSe Direct Discovery	12

6.1.4	Possible solutions	12

6.1.4.1	Solution #1.1: ProSe Direct Discovery charging with 5G DDNMF	12

6.1.4.1.1	Consideration for ProSe Direct Discovery charging with 5G DDNMF	12

6.1.4.1.2	Architecture Description	13

6.1.4.1.3	Flow Description	14

6.1.4.1.4	Solution evaluation	16

6.1.4.2	Solution #1.2: Charging for ProSe Direct Discovery over PC5 reference point	16

6.1.4.2.1	Consideration for ProSe Direct Discovery charging over PC5 reference point	16

6.1.4.2.2	Architecture Description	17

6.1.4.2.3	Flow Description	18

6.1.4.2.4	Solution evaluation	19

6.1.5	Evaluation	19

6.1.6	Conclusion	19

6.2	Scenario 2: 5GS charging for ProSe Direct Communication	19

6.2.1	General description and assumptions	19

6.2.2	Potential charging requirements	19

6.2.3	Key issues	20

6.2.3.1	Key issue #2.1: Converged charging for ProSe Direct Communication	20

6.2.4	Possible solutions	20

6.2.4.1	Solution #2.1: ProSe unicast mode of Direct Communication event-based charging for Key issues #2.1	20

6.2.4.1.1	Consideration for ProSe unicast mode of Direct Communication charging	20

6.2.4.1.2	Architecture Description	20

6.2.4.1.3	Flow Description	22

6.2.4.1.4	Solution evaluation	24

6.2.4.2	Solution #2.2: ProSe unicast mode of Direct Communication session-based charging for Key issues #2.1	24

6.2.4.2.1	Consideration for ProSe unicast mode of Direct Communication session-based charging	24

6.2.4.2.2	Architecture Description	24

6.2.4.2.3	Flow Description	25

6.2.4.2.4	Solution evaluation	26

6.2.4.3	Solution #2.3: ProSe Broadcast mode of Direct Communication charging for Key issues #2.1	26

6.2.4.3.1	Consideration for Broadcast mode of Direct Communication charging	26

6.2.4.3.2	Architecture Description	27

6.2.4.3.3	Flow Description	27

6.2.4.3.4	Solution evaluation	29

6.2.4.4	Solution #2.4: ProSe Groupcast mode of Direct Communication charging for Key issues #2.1	29

6.2.4.4.1	Consideration for Groupcast mode of Direct Communication charging	29

6.2.4.4.2	Architecture Description	30

6.2.4.4.3	Flow Description	30

6.2.4.4.4	Solution evaluation	30

6.2.4.5	Solution #2.5: ProSe charging for ProSe Direct Communication via UE-to-Network Relay for Key issues #2.2	30

6.2.4.5.1	Consideration for ProSe Direct Communication via UE-to-Network Relay charging	30

6.2.4.5.2	Architecture Description	31

6.2.4.5.3	Flow Description	31

6.2.5	Evaluation	34

6.2.6	Conclusion	34

6.3	Scenario 3: 5GS charging for ProSe Direct Communication based on QoS flow	34

6.3.1	General description and assumptions	34

6.3.2	Potential charging requirements	34

6.3.3	Key issues	34

6.3.3.1	Key issue #3.1: QoS flow Based Charging for ProSe Direct Communication	34

6.3.4	Possible solutions	34

6.3.4.1	Solution #3.1: QoS flow Based Charging for ProSe Direct Communication for Key issues #3.1	34

6.3.4.1.1	Consideration for ProSe QoS flow Based Charging	34

6.3.4.1.2	Architecture Description	35

6.3.4.1.3	Flow Description	35

6.3.5	Evaluation	35

6.3.6	Conclusion	35

7	Conclusions and recommendations	35

Annex A (informative): Change history	36

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document studies on charging aspects of Proximity-based Services in 5GS based on TS 23.304 [11].

The charging aspects related to the Proximity-based Services in 5GS are covered within the scope:

-	Charging architecture, for supporting charging of ProSe Direct Discovery and Direct Communication related to Public Safety and commercial proximity service.

-	Charging principles, scenarios and potential requirements related to the ProSe service in 5GS;

-	Potential charging solutions for the Proximity-based Services in 5GS.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[3]	3GPP TR 23.752: “Study on system enhancement for Proximity based Services (ProSe) in the 5G System (5GS)”.

[4]	3GPP TS 32.277: "Proximity-based Services (ProSe) charging".

[5]	3GPP TS 32.240: "Telecommunication management; Charging management; Charging architecture and principles ".

[6]	3GPP TS 32.290: "Telecommunication management; Charging management; 5G system; Services, operations and procedures of charging using Service Based Interface (SBI)"

[7]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[8]	3GPP TS 23.303: "Proximity-based services (ProSe); Stage 2".

[9]	3GPP TS 22.115: "Service aspects; Charging and billing".

[10]	3GPP TS 28.201: "Charging management; Network slice performance and analytics charging in the 5G System (5GS); Stage 2".

[11]	3GPP TS 23.304: "Proximity based Services (ProSe) in the 5G System (5GS)".

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

5G ProSe Direct Discovery: A procedure employed by a ProSe-enabled UE to discover other ProSe-enabled UEs in its vicinity by using only the capabilities of the two UEs with NR technology. See 3GPP TR TS 23.304 [11].

5G ProSe Direct Communication: A communication between two or more UEs in proximity that are ProSe-enabled, by means of user plane transmission using NR technology via a path not traversing any network node. See 3GPP TR TS 23.304 [11].

5G ProSe UE-to-Network Relay: A UE that provides functionality to support connectivity to the network for Remote UE(s). See 3GPP TS 23.304 [11].

Remote UE: A 5G ProSe-enabled UE that communicates with a DN via a 5G ProSe UE-to-Network Relay. See 3GPP TS 23.304 [11].

3.2	Symbols

Void.

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GS	5G System

CDR	Charging Data Record

CHF	Charging Function

CTF (AMC)	Charging Trigger Function (Accounting Metrics Collection)

CTF (ADF)	Charging Trigger Function (Accounting Data Forwarding)

DDNMF	Direct Discovery Name Management Function

ProSe	Proximity-based Services

PFI	PC5 QoS Flow Identifier

PQI	PC5 5QI

4	Concepts and overview

Proximity-based Services (ProSe) are services that can be provided by the 3GPP system based on UEs being in proximity to each other. Proximity-based Services has been developed in EPS to support both commercial and public safety services. Proximity Services (specifically the direct communication) has been enhanced to support V2X services over LTE.

In 5GS, the 3GPP system enablers for ProSe include the following functions:

-	Direct Discovery;

-	Direct Communication;

-	UE-to-Network Relay;

The present document aims to describe the converged charging for the Proximity-based Services (ProSe) in 5GS, based on the stage 2 description of ProSe in TS 23.303 [8].

As indicated in TS 32.277 [4], the offline and online charging description for the Proximity-based Services was specified based on EPS system, including the support of charging support of ProSe Direct Discovery, one-to-many Direct Communication, and one-to-one Direct Communication including UE-to-Network Relay (for public safety use), and EPC based discovery.

When mapped to 5GS, the study will focus on how to accommodate charging ProSe Direct Discovery and Direct Communication over NR based PC5 for commercial services and public safety based on the 5G service-based architecture.

5	Architecture considerations

5.1	High level ProSe architecture

The scope of the present document is Proximity-based Service charging in 5GS, related to architecture reference as follows:

-	Architecture reference model defined in TS 23.501 [7] are used as basis architecture for supporting ProSe in 5GS.

-	Architecture reference models defined in TS 23.304 [11] are used as reference architecture for supporting ProSe in 5GS.

5.2	Potential converged charging architectures for ProSe

5.2.1	General

The converged charging architectures for ProSe in 5GS should support charging for the following services:

-	ProSe Direct Discovery, including ProSe open Direct Discovery and restricted Direct Discovery, and

-	ProSe UE-to-Network Relay Discovery, and

-	ProSe Direct communication, including Broadcast and Groupcast Direct Communication and Unicast Direct Communication, and

-	ProSe Direct Communication via ProSe UE-to-Network Relay.

Details on the interfaces and functions can be found in TS 32.240 [5] for the general architecture components Nchf is described in TS 32.290 [6].

5.2.2	ProSe Direct Discovery converged charging architecture

Editor's Note:	the charging architecture for ProSe Direct Discovery is FFS along with the related key issues.

5.2.3	ProSe Direct Communication converged charging architecture

Editor's Note:	the charging architecture for ProSe Direct communication is FFS along with the related key issues.

6	Charging scenarios and key issues

6.1	Scenario 1: 5GS charging for ProSe Direct Discovery

6.1.1	General description and assumptions

5G ProSe Direct Discovery is defined as a procedure employed by a ProSe-enabled UE to discover other ProSe-enabled UEs in its vicinity by using only the capabilities of the two UEs with NR technology.

There are two types of 5G ProSe Direct Discovery: open and restricted. Open is the case where there is no explicit permission that is needed from the UE being discovered, whereas restricted discovery only takes place with explicit permission from the UE that is being discovered.

There are two models for 5G ProSe Direct Discovery: Model A and Model B. For discovery procedure over NR based PC5 for commercial services and public safety in 5GS, the definition for Model A and Model B is re-used as defined in clause 5.3.1.2 of TS 23.303 [8].

For dynamic ProSe Direct Discovery, 5G DDNMF in the 5GS is used for ProSe Discovery Code management (including allocation, and resolution). The architecture defined in TS 23.304 [11] will be adopted as the reference architecture, and reuse the PC3 procedures defined in TS 23.303 [8] clause 5.3 for UE and 5G DDNMF interactions. When 5G DDNMF successfully response to different Discovery Request and Discovery Report message, the Charging Data Request [Event] may be generated and forward them to CHF.

For ProSe Direct Discovery over PC5, PC5 communication channel is used to carry the discovery message over PC5 and discovery message over PC5 is differentiated with other PC5 messages by AS layer. When the UE decides that reporting criteria are met, according to the pre-configuration, the UE creates the corresponding usage information report and forward to 5G Network.

6.1.2	Potential charging requirements

The following are potential high-level charging requirements for ProSe services in 5GS, derived from the requirements in TS 22.115 [9], and TS 23.303 [8].

REQ-CH_PROSE_5GS_DD-01: The 5GS should support converged charging and charging information reporting for ProSe Discovery including:

-	ProSe open Direct Discovery Model A;

-	ProSe restricted Direct Discovery Model A and Model B;

-	ProSe open and restricted Direct Discovery for Announce;

-	ProSe open and restricted Direct Discovery for Monitor;

-	ProSe open and restricted Direct Discovery for Match;

-	ProSe Direct Discovery over NR PC5, including UE-to-Network Relay.

Editor's note: These requirements is FFS based on use cases.

REQ-CH_ PROSE _5GS_DD -02: The 5GS should support identifying chargeable events and collecting charging information from UE.

REQ-CH_ PROSE _5GS_DD -03:	The 5GS should support identifying chargeable events and collecting charging information via 5G ProSe service.

6.1.3	Key issues

6.1.3.1	Key issue #1.1: Converged charging for ProSe Direct Discovery

This key issue is for investigating how to support converged charging for ProSe Direct Discovery considering REQ-CH_PROSE_5GS_DD -01, REQ-CH_PROSE_5GS_DD-02 and REQ-CH_PROSE_5GS_DD-03.

This investigation covers the following:

-	determination of which entity/entities in the 5G system are suitable to provide the charging information for ProSe Direct Discovery;

-	identification of the triggers for charging events for Prose Direct Discovery with or without network control.

-	determination of the behaviour with respect to the chargeable events, the matching charging events, and the charging information elements forwarded towards the CHF.

6.1.4	Possible solutions

6.1.4.1	Solution #1.1: ProSe Direct Discovery charging with 5G DDNMF

6.1.4.1.1	Consideration for ProSe Direct Discovery charging with 5G DDNMF

ProSe Direct Discovery is defined as the process that detects and identifies another UE in proximity using NR radio signals. There are two types of ProSe Direct Discovery supported over PC3a reference point: open and restricted, as defined in TS 23.303 [8]. ProSe Direct Discovery can be a standalone service or can be used for subsequent actions e.g. to initiate ProSe Direct Communication.

5G DDNMF is defined to manage the dynamic ProSe Direct Discovery. Functionalities of the 5G DDNMF and the interactions with the UEs are defined in TS 23.304 [11].

The ProSe Direct Discovery with 5G DDNMF procedures could be divided into Discovery Request procedure and Discovery Reporting procedure, both could support Model A, and Model B. The exact signalling procedures are defined in TS 23.303 [8] clause 5.3.3.

The Charging Enablement Function (CEF) is defined in TS 28.201 [10], for ProSe converged charging the CEF could be a consumer of NF service provider for ProSe (e.g.,5G DDNMF) and charging (Nchf) service provider.

The charging information on the use of ProSe Direct Discovery is collected by the 5G DDNMF in HPLMN, VPLMN, and local PLMNs.

The 5GS may collect the following minimum charging information:

Table 6.1.4.1.1-1: Structure of the ProSe Discovery Information

| Information Element | Description |
| --- | --- |
| UE identity | The identity of the ProSe UE |
| Serving PLMN ID | PLMN identity of the serving PLMN which signalled the carrier frequency |
| Announcing PLMN ID | This field holds PLMN identity of PLMN for UE to be monitored in monitor/match report procedure |
| Monitored PLMN ID | This field holds PLMN identity of PLMN for UE requested to be monitored in monitor/match report procedure |
| ProSe Application ID | The identities used for ProSe Direct Discovery, identifying application related information for the ProSe-enabled UE |
| Application ID | A globally unique identifier identifying a specific 3rd party application |
| Direct Discovery Model | Model of the Direct Discovery used by the UE, e.g. Model A, or Model B. |
| ProSe Event Type | This IE holds the event which triggers the charging message delivery, e.g. Announce, Monitor, Match Report |
| ProSe Request Timestamp | The time when ProSe Request is received from UE. |
| PC5 Radio Technology | The PC5 radio technology used by UE for ProSe Direct Discovery |

Editor's note:	Whether other information elements are needed is FFS.

6.1.4.1.2	Architecture Description

A set of trigger conditions are defined for the 5G DDNMF (CTF) or CEF to invoke a Charging Data Request [Event] towards the CHF.

The converged charging architecture is proposed for the event based charging for 5GS ProSe under the alternatives：

-	Charging Trigger Function (CTF) based, as depicted in figure 6.1.4.1.2-1.

-	Charging Enablement Function (CEF) based, depicted in figure 6.1.4.1.2-2.

Figure 6.1.4.1.2-1: The Converged Charging System (CTF)

Figure 6.1.4.1.2-2: The Converged Charging System (CEF)

6.1.4.1.3	Flow Description

6.1.4.1.3.1-a	Message flows for ProSe Direct Discovery Request - CTF

Figure 6.1.4.1.3.1-a-1: Message flow for ProSe Direct Discovery Request - CTF (non-roaming)

The detailed description for the message flow as defined in TS 23.304[11] clause 6.3.1.4 and TS 23.303 [8] clause 5.3.4.

1-2. These steps are the same as the ProSe Direct Discovery will be defined in TS 23. 304 [11]. The Direct Discovery Request could be with command (Announce request, Monitor request, Discoverer request, Discoveree Request).

3. 	The 5G DDNMF responds with a Discovery Response with:

-	(ProSe Application Code, validity timer, PC5_tech) for open discovery.

-	(ProSe Application Code, ProSe Restricted Code/ ProSe Restricted Code Prefix[ProSe Restricted Code Suffix pool], validity timer, Discovery Entry ID, PC5_tech) for restricted discovery.

3a.	 The 5G DDNMF triggers Charging Data Request[Event] to CHF in HPLMN where event represents Announce. The PF-DD-CDR is generated by CHF for Announcing UE.

3b. The CHF creates a CDR for this Announcing UE.

3c. The CHF returns Charging Data Response corresponding to the received Charging Data Request[Event].

NOTE1: 	Roaming/inter-PLMN procedures are similar to that procedures as defined in TS 32.277 [4] clause 5.2.2.1.

6.1.4.1.3.1-b	Message flows for ProSe Direct Discovery Reporting – CTF

The Direct Discovery Reporting procedure can be used by the "monitoring UE" (in Model A) and Discoverer UE (in Model B) to request the 5G DDNMF to resolve a matched ProSe Discovery Code(s), for both open discovery and restricted discovery.

The message flows for ProSe Direct Discovery Reporting could re-use the procedures as defined in TS 32.277 [4] clause 5.2.2.1, with the following modification:

-	the 5G DDNMF takes the role of "ProSe Function" in the procedure;

-	the HSS is replaced by PCF/UDM;

-	corresponding 5GS identifiers replace the EPS identifiers, e.g. use SUPI instead of IMSI, and use GPSI instead of MSISDN;

-	PC5_tech parameter is omitted and the intended PC5 radio technology is NR.

6.1.4.1.3.2-a	Message flows for ProSe Direct Discovery Request - CEF

Figure 6.1.4.1.3.2-a-1: CEF-Message flow for ProSe Direct Discovery Request (non-roaming)

1.	Determination by CEF to subscribe to ProSe Direct Discovery Service.

2.	Subscribe Request: the CEF subscribes to 5G DDNMF.

3-5. These steps are the same as the ProSe Direct Discovery will be defined in TS 23.304[11]. The Direct Discovery Request could be with command (Announce request, Monitor request, Discoverer request, Discoveree Request).

Editor's Note:	This message flow needs to align with future TS 23.304 based on TR 23.752 conclusion.

6.	Notification: DDNMF notifies the CEF that ProSe Direct Discovery message has been processed.

7.	Notification Acknowledge sent by the CEF.

8-a. The CEF sends Charging Data Request [Event] to CHF associated to the event represents Discovery Request.

8-b. The CHF creates a CDR.

8-c. The CHF acknowledges by sending Charging Data Response to the CEF.

Editor's Note:	It is FFS for the use of services proved from 5G DDNMF for charging information.

6.1.4.1.3.2-b	Message flows for ProSe Direct Discovery Reporting – CEF

The message flows could re-use the CEF subscription/notification procedure and Charging Data Request procedure as described in 6.1.4.1.3.2-a.

6.1.4.1.4	Solution evaluation

TBD

6.1.4.2	Solution #1.2: Charging for ProSe Direct Discovery over PC5 reference point

6.1.4.2.1	Consideration for ProSe Direct Discovery charging over PC5 reference point

A PC5 communication channel is used to carry the discovery message over PC5 and the discovery message over PC5 is differentiated from other PC5 messages by AS layer.

Both Model A and Model B discovery as defined in TS 23.303 [8] are supported:

-	Model A uses a single discovery protocol message (Announcement).

-	Model B uses two discovery protocol messages (Solicitation and Response).

5G DDNMF is defined to manage the dynamic ProSe Direct Discovery. Functionalities of the 5G DDNMF and the interactions with the UEs are defined in TS 23.304 [11].

The Charging Enablement Function (CEF) is defined in TS 28.201 [10], for ProSe converged charging the CEF could be a consumer of NF service provider for ProSe (e.g., 5G DDNMF Service) and charging (Nchf) service provider.

The charging information should be collected by UEs when ProSe Direct Discovery over PC5 reference point, both Group Member Discovery and UE-to-Network Relay Discovery are applicable to public safety use and commercial services as defined in TS23.304 [11] clause 6.3.2.

When the UE decides that reporting criteria are met, according to the pre-configuration, the UE creates the corresponding usage information report to NF(CTF) over PC3ch or to ProSe UE-to-Network Relay UE. If the UE is out of coverage of NR and has no connection to the 5G network, the usage information is stored in a secure environment in the UE, it will trigger the reporting when UE comes back to NR coverage, as described in TS 32.277 [4].

Besides the information list in Table 6.1.4.1.1-1, the 5GS may collect the following charging information:

Table 6.1.4.2.1-1: Structure of the ProSe Discovery Information

| Information Element | Description |
| --- | --- |
| User Location Information | The location of the UE |
| UE identity | The identity of the ProSe UE |
| Serving PLMN ID | PLMN identity of the PLMN which signalled the carrier frequency |
| ProSe Relay UE ID | A link layer identifier that uniquely represents the ProSe UE-to-Network Relay UE |
| ProSe Target Layer-2 ID | The identifier of target UE, uniquely represents a specific one-to-one or one-to-many discovery/communication |
| ProSe Group IP multicast address | The IP multicast address to be used for performing ProSe direct discovery/communication. |
| Coverage Info | This IE provides information on the coverage status (i.e., whether the UE is served by NG-RAN or not) and the time when the coverage status changed to its current state. |
| Radio Resources indicator | This IE identifies whether the operator-provided radio resources or the configured radio resources were used for ProSe direct discovery/communication. |
| Usage Data Container | This field holds the container associated to a trigger conditions (e.g. go out of coverage, come back to coverage, etc.) on a specific ProSe communication. |

Editor's note:	The information may also be used for Direct Communication, it is FFS for whether more information elements are needed.

6.1.4.2.2	Architecture Description

Regarding the PC5 usage reporting from the UE, the CTF is divided into two functional blocks as described in Annex D of TS 32.240 [5]. The Accounting Metrics Collection (AMC) function block is in the UE. The AMC sends usage information collected to the Accounting Data Forwarding (ADF) function block. Upon reception of the event, CTF(ADF) or CEF could generate Charging Data Request [Event] and forward them to CHF.

The converged charging architecture is proposed for the event based charging for 5GS ProSe under the alternatives:

-	Charging Trigger Function (CTF) based, as depicted in figure 6.1.4.2.2-1.

-	Charging Enablement Function (CEF) based, depicted in figure 6.1.4.2.2-2.

Figure 6.1.4.2.2-1: The Converged Charging System (CTF)

Figure 6.1.4.2.2-2: The Converged Charging System (CEF)

6.1.4.2.3	Flow Description

6.1.4.2.3.1	Message flows for ProSe Direct Discovery over PC5 charging CTF

Figure 6.1.4.2.3.1-1: Message flow for ProSe Direct Discovery over PC5 charging - CTF (non-roaming)

1. UE-1 sends announcement message with model A or solicitation message with model B. In the latter case, UE 2 sends a response message.

NOTE 1: 	In procedure for UE-to-Network Relay Discovery, the Remote UE and UE-to-Network Relay UE will perform UE-to-Network Relay UE discovery and selection (see TS 23.304 [11] clause 6.2.3.2).

2. When the UE-1 decides that reporting criteria are met, according to the pre-configuration, the UE creates the corresponding usage information report.

3. UE-1 triggers the usage reporting procedure by sending the usage information report to the CTF located in ProSe NF (e.g., 5G-DDNFM).

NOTE 2: 	Both UE-1, UE2 or other UEs can decide that reporting criteria are met and trigger the usage reporting procedure.

4ch-a. The 5G NF (CTF) triggers Charging Data Request[Event] to CHF in HPLMN.

4ch-b. The CHF creates a CDR for this UE.

4ch-c. The CHF returns Charging Data Response.

6.1.4.2.3.2	Message flows with CEF

The message flows with CEF could re-use the CEF subscription/notification procedure and Charging Data Request procedure as described in 6.1.4.1.3.2.

6.1.4.2.4	Solution evaluation

TBD

6.1.5	Evaluation

6.1.6	Conclusion

……

6.2	Scenario 2: 5GS charging for ProSe Direct Communication

6.2.1	General description and assumptions

5G ProSe Direct Communication is defined as a communication between two or more UEs in proximity that are ProSe-enabled, by means of user plane transmission using NR technology via a path not traversing any network node.

5G ProSe Direct Communication over NR based PC5 reference point supports broadcast mode, groupcast mode, and unicast mode. Each communication mode is supported when the UE is served by NR and when the UE is outside of NR coverage.

5G ProSe Direct Communication supports both the case of public safety and commercial service.

5G ProSe Direct Communication supports both event based and session based charging;

To perform ProSe direct communication over PC5 reference point, the UE is configured with the related information as described in TS 23.304 [11] clause 5.1.3. The procedures for service authorization and provisioning to UE may be initiated by the PCF, by the UE, or by the AF, as defined in TS 23.304 [11] clause 6.2, the parameters could be provided/updated by ProSe Application Server or PCF, provisioned in the ME or configured in the UICC.

The 5GS could re-use the Service authorization and provisioning mechanism, and the UE could be configured with information to be included in the usage information report.

Editor's Note:	The support for Direct Communication via UE-to-Network Relay FFS.

6.2.2	Potential charging requirements

The following are potential high-level charging requirements for ProSe services in 5GS, derived from the requirements in TS 22.115 [9], and TS 23.303 [8].

REQ-CH_ PROSE _5GS_DC -01: The 5GS should support converged charging and charging information reporting for ProSe Communication including:

-	ProSe Broadcast modeDirect Communication;

-	ProSe Groupcast mode Direct Communication;

-	ProSe Unicast Direct Communication, including UE-to-Network Relay.

REQ-CH_ PROSE _5GS_DC - 02: The 5GS should support identifying chargeable events and collecting charging information from UE.

REQ-CH_ PROSE _5GS_DC - 03:	The 5GS should support identifying chargeable events and collecting charging information via 5G ProSe service.

REQ-CH_ PROSE _5GS_DC - 04: The 5GS should support charging for PDU Session types of IP, Ethernet and Unstructured.

REQ-CH_ PROSE _5GS_DC - 05: In 5G ProSe UE-to-Network Relay scenario, the 5GS should support charging for both Layer-2 and Layer-3 UE-to-Network Relay.

6.2.3	Key issues

6.2.3.1	Key issue #2.1: Converged charging for ProSe Direct Communication

This key issue is for investigating how to support converged charging for ProSe Direct Communication considering REQ-CH_PROSE_5GS_DC-01.

This investigation covers the following:

-	determination of which entity/entities in the 5G system are suitable to provide the charging information for ProSe Direct Communication;

-	identification of the triggers for charging events or event based charging and session based charging.

-	determination of the behaviour with respect to the chargeable events, the matching charging events, and the charging information elements.

6.2.4	Possible solutions

6.2.4.1	Solution #2.1: ProSe unicast mode of Direct Communication event-based charging for Key issues #2.1

6.2.4.1.1	Consideration for ProSe unicast mode of Direct Communication charging

This reporting is achieved by sending Charging Data Request to the CHF from the ProSe related CTF or CEF.

When the CTF (ACM) in the UE decides that reporting criteria are met and the UE have network connection, the CTF (ACM) sends the collected usage information to the CTF (ADF).

When the CTF (ADF) receives usage information from the CTF (ACM) it triggers a charging data request for the unicast (one-to-one) mode communication over PC3ch.

The CTF(ADF) goes through the reported usage information, and if it contains valid ProSe Direct Communication usage data for the unicast mode communication, then sends the events directly to the CHF or through a CEF.

Editor's note:	It is FFS for where the ADF part of the CTF would be located.

The CHF may then, triggers the generation of the CDR for the unicast mode communication.

The 5GS may collect the following charging information for ProSe Direct communication from following table:

Table 6.2.4.1.1-1: Structure of the ProSe direct communication charging Information

| Information Element | Description |
| --- | --- |
| User Location Information | The location of the UE |
| UE identity | The identity of the ProSe UE |
| Serving PLMN ID | PLMN identity of the PLMN which signalled the carrier frequency |
| ProSe Target Layer-2 ID | The identifier of target UE, uniquely represents a specific one-to-one one-to-many, or relay discovery/communication |
| Coverage Info | This IE provides information on the coverage status (i.e., whether the UE is served by NG-RAN or not) and the time when the coverage status changed to its current state. |
| Radio Resources indicator | This IE identifies whether the operator-provided radio resources or the configured radio resources were used for ProSe direct discovery/communication |
| Usage Data Container | This field holds the container associated to a trigger conditions (e.g. go out of coverage, come back to coverage, etc.) on a specific ProSe communication |

Editor's note:	Whether other information elements are needed is FFS.

6.2.4.1.2	Architecture Description

Trigger conditions are defined for the CTF or CEF to invoke a Charging Data Request [Event] towards the CHF.

The converged charging architecture is proposed for the event based charging for 5GS ProSe under the alternatives：

-	Charging Trigger Function (CTF) based, as depicted in figure 6.2.4.1.2-1.

-	Charging Enablement Function (CEF) based, depicted in figure 6.2.4.1.2-2.

Figure 6.2.4.1.2-1: ProSe converged charging architecture (CTF)

Figure 6.2.4.1.2-2: ProSe converged charging architecture (CEF)

6.2.4.1.3	Flow Description

6.2.4.1.3.1	Message flows with CTF – PEC

Figure 6.2.4.1.3.1-1: Message flow for ProSe  Direct Communication (event based)

Editor's Note:	This message flow needs to align with future TS 23.304 based on TR 23.752 conclusion.

1. UE-1 sends a Direct Communication Request message to UE-2 in order to trigger mutual authentication.

2-3. UE-2 initiates the procedure for mutual authentication. The successful completion of the authentication procedure completes the establishment of the secure layer-2 link over PC5 and then UE performs unicast mode Direct Communication.

4. UE-1 sends a Disconnect Request message to UE-2 in order to release the layer-2 link and deletes all context data associated with.

5. Upon reception of the Disconnect Request message UE-2 responds with a Disconnect Response message and deletes all context data associated with the layer-2 link.

6. When the UE-1 decides that reporting criteria are met, according to the pre-configuration, the UE creates the corresponding usage information report.

NOTE 1: 	Both UE-1 and UE-2 can decide that reporting criteria are met and trigger the usage reporting procedure.

7.	Upon reception of the Disconnect Response from UE-2, the UE-1 triggers the usage reporting procedure. UE-1 sends the usage information report to the CTF(ADF).

8ch-a.	Upon reception of Direct Communication usage information report, the CTF(ADF) triggers the Charging Data Request[Event].The CTF(ADF) sends Charging Data Request [Event] to CHF.

8ch-b. 	The ProSe unicast mode Direct Communication CDR is generated by CHF for the UE-1 and UE-2.

8ch-c. The CHF acknowledges by sending Charging Data Response [Event] to the CTF(ADF).

NOTE 2:	The procedure applies to UE1 to UE2 independently, i.e. each of the UE sends the respective usage information reports to the network using either the under coverage procedure or out of coverage procedure.

6.2.4.1.3.2	Message flows with CEF – PEC

Figure 6.2.4.1.3.2-1: CEF-Message flow for ProSe Direct Communication (event based)

1.	Determination by CEF to subscribe to ProSe Service provider which is able to handle usage information report from the UE.

2.	Subscribe Request: the CEF subscribes to ProSe Service provider.

3.	Subscribe Request: the ProSe Service Producer sends successful subscription response.

4-10. These steps are the same as procedure described in Figure 6.2.4.1.3.1-1.

11.	Notification: ProSe Service provider notifies the CEF that usage information report has been processed.

12.	Notification Acknowledge sent by the CEF.

13ch-a. The CEF sends Charging Data Request [Event] to CHF.

13ch-b. The CHF creates a CDR.

13ch-c The CHF acknowledges by sending Charging Data Response to the CEF.

Editor's Note:	This message flow needs to align with future TS 23.304 based on TR 23.752 conclusion.

Editor's Note:	It is FFS for the use of services proved from ProSe Service provider for charging information.

6.2.4.1.4	Solution evaluation

TBD

6.2.4.2	Solution #2.2: ProSe unicast mode of Direct Communication session-based charging for Key issues #2.1

6.2.4.2.1	Consideration for ProSe unicast mode of Direct Communication session-based charging

The 5G ProSe charging service may be configured to use session based charging in order to reduce the number of CDRs generated.

The CTF(ADF) or CEF generates charging data related to the service delivered that is based on a trigger for usage reporting is met.

The Charging Data Request[Initial] is sent when the ProSe CTF(ADF) receives a Received Direct Communication Usage Report event, and there is no open charging session for the group communication and unicast communication including the UE-Network-relay

The Charging Data Request[Update] is sent when the ProSe CTF(ADF) receives a Received Direct Communication Usage Report event, and there is an open charging session for the group communication and unicast communication including the UE-Network-relay.

The Charging Data Request[Termination] is sent when the ProSe service provider decides one of the following conditions is met:

-	operator configured maximum number of reports, or

-	operator configured maximum time limit.

6.2.4.2.2	Architecture Description

See clause 6.2.4.1.2.

6.2.4.2.3	Flow Description

6.2.4.2.3.1	Message flows with CTF – SCUR

Figure 6.2.4.2.3.1-1: Message flow for ProSe Direct Communication (session based)

1-3. These steps are the same as described in figure 6.2.4.1.3.1-1.

4. When the UE decides that reporting criteria are met, according to the pre-configuration, the UE creates the corresponding usage information report. UE triggers the usage reporting procedure.

5. UE sends the usage information report to the CTF located in ProSe NF (e.g., 5G-DDNFM)..

6ch-a. The NF (CTF) determines the number of units depending on the service requested by the UE, and sends the Charging Data Request[Initial] to the CHF when there is no open charging session.

6ch-b. Based on policies, the CHF opens a CDR related to the service.

6ch-c. The CHF grants authorization to NF (CTF) for the service to start, and returns Charging Data Response.

7.	UE triggers the usage reporting procedure when the reporting criteria are met.

NOTE 1: 	Both UE-1 and UE-2 can decide that reporting criteria are met and trigger the usage reporting procedure.

8.	UE sends the usage information report to the ProSe NF (CTF).

9ch-a.	If there is a charging session for the session of unicast mode direct communication, upon reception of direct communication usage information report for the session, the NF (CTF) triggers the Charging Data Request[Update]. The NF (CTF) sends the Charging Data Request[Update] to the corresponding CHF.

9ch-b. The CDR for the ProSe unicast Direct Communication is updated by CHF for the UE.

9ch-c.	The CHF returns Charging Data Response corresponding to the received Charging Data Request.

NOTE 2: 	The Step 9ch-a to 9ch-c may occur multiple times for update.

10. Upon reception of the Disconnect Request message UE2 responds with a Disconnect Response message and deletes all context data associated with the layer-2 link.

11.	Upon reception of the Disconnect Response from UE-2, the UE-1 triggers the usage reporting procedure.UE1 sends the usage information report to the NF (CTF),

12. UE sends the usage information report to the NF (CTF).

NOTE 3: 	The Step 16 may occur before step 14 and step 15.

13ch-a.	The NF (CTF) decides that the charging session should be closed, and triggers the Charging Data Request[Termination]. The NF (CTF) sends the Charging Data Request[Termination] to the corresponding CHF.

13ch-b. The CDR for the ProSe unicast Direct Communication is closed by CHF for the UE.

13ch-c. The CHF returns Charging Data Response corresponding to the received Charging Data Request.

NOTE 4:	The procedure applies to UE1 to UE2 independently, i.e. each of the UE sends the respective usage information reports to the network using either the under coverage procedure or out of coverage procedure.

6.2.4.2.3.2	Message flows with CEF

TBD

6.2.4.2.4	Solution evaluation

TBD

6.2.4.3	Solution #2.3: ProSe Broadcast mode of Direct Communication charging for Key issues #2.1

6.2.4.3.1	Consideration for Broadcast mode of Direct Communication charging

For broadcast mode of ProSe direct communication over PC5 reference point, the UE is configured with the Destination Layer-2 ID(s) to be used for ProSe applications. The Destination Layer-2 ID for a ProSe direct communication is selected based on the configuration.

The transmitting UE in broadcast communication determines the destination Layer-2 ID for broadcast and assigns itself a source Layer-2 ID. The receiving UE determines the destination Layer-2 ID for broadcast reception. The transmitting UE sends the service data using the source Layer-2 ID and the destination Layer-2 ID.

The 5GS may collect the following charging information in addition to the charging information described in Table 6.2.4.1.1-1:

-	ProSe Source Layer-2 ID

-	ProSe Destination Layer-2 ID

-	Indication whether the UE should use IPv4 or IPv6 for broadcast

- 	Application specific data, e.g. application specific session floor control information

For Broadcast mode ProSe Direct Communication charging, the chargeable events could be:

- 	Received Direct Communication Usage Report (for broadcast).

If event based charging is used, upon this event, a Charging Data Request[Event] for broadcast is generated.

If session based charging is used, either a Charging Data Request[Initial], Charging Data Request[Update], or Charging Data Request[Termination] is generated.

6.2.4.3.2	Architecture Description

The possible solutions for ProSe broadcast mode Direct Communication charging are based on the potential converged charging architectures defined in clause 6.2.4.1.2.

6.2.4.3.3	Flow Description

6.2.4.3.3.1	ProSe broadcast mode of Direct Communication event-based charging - PEC

Figure 6.2.4.3.3.1-1: ProSe broadcast mode of Direct Communication event-based charging – PEC

1. The receiving UE(s) determine the destination Layer-2 ID for broadcast reception. The destination Layer-2 ID is passed down to the AS layer of receiving UE(s) for the reception.

2. The transmitting UE ProSe application layer provides data unit and may provide ProSe Application Requirements to ProSe layer.

3. The transmitting UE determines the destination Layer-2 ID for broadcast, and self-assigns the source Layer-2 ID.

4. The transmitting UE sends the ProSe data using the source Layer-2 ID and the destination Layer-2 ID as defined in TS 23.304 [11].

5. When the UE decides that reporting criteria are met, according to the configuration, and the connection to the network is available, the UE creates the corresponding usage information report.

NOTE 1: 	Both transmitting UE and receiving UE(s) can decide that reporting criteria are met and trigger the usage reporting procedure.

NOTE 2: 	When the UE is out of NR coverage and has no connection to the 5G network, the usage information is stored in a secure environment in the UE, it will trigger the reporting when UE comes back to NR coverage, as described in TS 32.277 [4].

6.	UE triggers the usage reporting procedure. UE (CTF-AMC) sends the usage information report to the ProSe NF（CTF-ADF), according to the configuration.

7ch-a.	Upon reception of Direct Communication usage information report, the NF(CTF) triggers the Charging Data Request [Event]. The NF(CTF) sends Charging Data Request [Event] to CHF.

7ch-b. 	The ProSe broadcast mode Direct Communication CDR is generated by CHF.

7ch-c. The CHF acknowledges by sending Charging Data Response [Event] to the NF(CTF).

6.2.4.3.3.2	ProSe broadcast mode of Direct Communication session-based charging – SCUR

Figure 6.2.4.3.3.2-1: ProSe broadcast mode of Direct Communication session-based charging – SCUR

1-4 These steps are the same as described in Figure 6.2.4.3.3.1-1.

5. When the UE decides that reporting criteria are met, according to the configuration, and the connection to the network is available, the UE creates the corresponding usage information report.

NOTE 1: 	Both transmitting UE and receiving UE(s) can decide that reporting criteria are met and trigger the usage reporting procedure.

NOTE 2: 	When the UE is out of NR coverage and has no connection to the 5G network, the usage information is stored in a secure environment in the UE, it will trigger the reporting when UE comes back to NR coverage, as described in TS 32.277 [4].

6.	UE (CTF-AMC) sends the usage information report to the NF (CTF-ADF).

7ch-a. If the CTF located with ProSe Service is configured to use session based charging, upon reception of direct communication usage information report for a broadcast, the NF (CTF) triggers the Charging Data Request[Init] when there is no open charging session. The NF (CTF) sends the Charging Data Request[Interim] to the corresponding CDF, and starts a charging session.

7ch-b. Based on policies, the CHF opens a CDR related to the service.

7ch-c. 	The CHF grants authorization to NF (CTF) for the service to start, and returns Charging Data Response.

8. UE triggers the usage reporting procedure when the reporting criteria are met.

9ch-a.	If there is a charging session, upon reception of direct communication usage information report, the NF (CTF) triggers the Charging Data Request[Update]. The NF (CTF) sends the Charging Data Request[Update] to the corresponding CHF.

9ch-b. The CDR is updated by CHF for the UE.

9ch-c.	The CHF returns Charging Data Response corresponding to the received Charging Data Request.

NOTE 3: 	The Step 9ch-a to 9ch-c may occur multiple times for update.

10. UE triggers the usage reporting procedure when the reporting criteria are met.

11ch-a.	The NF (CTF) decides that the charging session should be closed, and triggers the Charging Data Request[Termination]. The NF (CTF) sends the Charging Data Request[Termination] to the corresponding CHF.

11ch-b. The CDR is closed by CHF for the UE.

11ch-c. The CHF returns Charging Data Response corresponding to the received Charging Data Request.

6.2.4.3.4	Solution evaluation

TBD

6.2.4.4	Solution #2.4: ProSe Groupcast mode of Direct Communication charging for Key issues #2.1

6.2.4.4.1	Consideration for Groupcast mode of Direct Communication charging

For Groupcast mode of ProSe direct communication over PC5 reference point, the UE is configured with the Destination Layer-2 ID(s) to be used for ProSe applications. The Destination Layer-2 ID for a ProSe direct communication is selected based on the configuration.

The transmitting UE in groupcast communication determines a source Layer-2 ID and a destination Layer-2 ID and the receiving UE determines destination Layer-2 ID. The transmitting UE determines the PC5 QoS parameters for this groupcast. The transmitting UE sends the service data using the source Layer-2 ID and the destination Layer-2 ID.

For Groupcast mode ProSe Direct Communication charging, the chargeable events could be:

- 	Received Direct Communication Usage Report (for Groupcast).

The 5GS could re-use the Service authorization and provisioning mechanism defined in TS 23.304 [11] clause 5.1.3, the UE could be configured with information to be included in the usage information report as per following:

-	Whether the Group Parameters need to be reported;

-	Whether timestamps of the first transmission/reception need to be reported;

-	Whether the amount of data transmitted by UE needs to be reported, and whether with location information;

-	Whether the amount of data received by UE needs to be reported, and whether with location information;

-	Whether the list of locations of the UE when in NR coverage needs to be reported;

-	Whether the list of timestamps of when UE goes in/out of NR coverage need to be recorded.

-	Whether the indicator of radio resources used and radio frequency are to be reported with the amount of data transmitted and received.

The 5GS may collect the following charging information in addition to the charging information described in Table 6.2.4.1.1-1:

-	Application Layer Group ID: Identifies an application layer group that the UE belongs to.

-	ProSe Layer-2 Group ID

-	ProSe Group IP multicast address

-	Indication whether the UE should use IPv4 or IPv6 for that group

- 	Application specific data, e.g. application specific session floor control information, Application layer User ID of group members in the communication.

Editor's note:	It is FFS whether other information elements are needed.

If event based charging is used, upon this event, a Charging Data Request[Event] for Groupcast is generated.

If session based charging is used, either a Charging Data Request[Initial], Charging Data Request[Update], or Charging Data Request[Termination] is generated.

6.2.4.4.2	Architecture Description

The possible solutions for ProSe Group mode Direct Communication charging are based on the potential converged charging architectures defined in clause 6.2.4.1.2.

6.2.4.4.3	Flow Description

6.2.4.4.3.1	ProSe groupcast mode of Direct Communication event-based charging - PEC

The flow is the same as TS 32.277 [4] clauses 5.2.2.3.2 and 5.2.2.3.3 with the following differences:

-	ProSe Function is replaced by ProSe Service which provides PC3ch interface toward UE;

-	CDF is replaced by CHF;

6.2.4.4.3.2	ProSe groupcast mode of Direct Communication session-based charging – SCUR

The flow is the same as TS 32.277 [4] clauses 5.2.2.3.4 and 5.2.2.3.5 with the following differences:

-	ProSe Function is replaced by ProSe Service which provides PC3ch interface toward UE;

-	CDF is replaced by CHF.

6.2.4.4.4	Solution evaluation

TBD

6.2.4.5	Solution #2.5: ProSe charging for ProSe Direct Communication via UE-to-Network Relay for Key issues #2.2

6.2.4.5.1	Consideration for ProSe Direct Communication via UE-to-Network Relay charging

The 5G ProSe Layer-3 UE-to-Network Relay shall provide generic function that can relay any IP, Ethernet or Unstructured traffic, while the 5G ProSe Layer-2 UE-to-Network Relay provides forwarding functionality that can relay any type of traffic over the PC5 link.

Both Layer-2 and Layer-3 UE-to-Network Relay entity provides the relaying functionality to support connectivity to the network for Remote UEs. It can be used for both public safety services and commercial services (e.g. interactive service).

For ProSe Direct Communication via UE-to-Network Relay charging, the chargeable events could be:

- 	Received Direct Communication Usage Report which is triggered by CTF(ADF) receiving a usage information report from the ProSe UE-to-Network relay UE.

The 5GS may collect the following charging information in addition to the charging information described in Table 6.2.4.1.1-1:

-	Identities of the transmitters in the direct communication via 5G ProSe UE-to-Network relay, e.g. Source L2 ID and IP address, ProSe UE-to-Network Relay UE L2 ID and IP address.

- 	List of non-zero amount of data relayed by UE, e.g. List of amount of data relayed by a ProSe UE-to-Network Relay at each location, with NCGI and the corresponding timestamps, and indicator of radio resources used and the radio frequency used;

- 	Application specific data, e.g. application specific session floor control information, Application layer User ID.

6.2.4.5.2	Architecture Description

See clause 6.2.4.1.2.

6.2.4.5.3	Flow Description

6.2.4.5.3.1	Message flow for ProSe Direct Communication via Layer-3 UE-to-Network Relay

Figure 6.2.4.5.3.1-1: Message flow for ProSe Direct Communication via Layer-3 UE-to-Network Relay

1-7. These steps are the same as message flow for ProSe UE-to-Network Direct Communication via Layer-3 procedures described in TS 23.304 [11] clause 6.5.1.1.

8. When the UE decides that reporting criteria are met, according to the pre-configuration, the UE creates the corresponding usage information report. UE triggers the usage reporting procedure.

9.  The Remote UE sends the usage reporting to ProSe UE-to-Network Relay UE. Then Relay UE sends the usage information report to the CTF located in ProSe NF (e.g., 5G-DDNFM).

9ch-a. Upon reception of Direct Communication usage information report, the CTF(ADF) triggers the Charging Data Request [Event]. The CTF(ADF) sends Charging Data Request [Event] to CHF.

9ch-b.  The 5G ProSe Direct communication via UE-to-Network Relay CDR is generated by CHF for the Remote UE.

9ch-c. The CHF acknowledges by sending Charging Data Response [Event] to the CTF(ADF).

10.	5G ProSe UE-to-Network Relay UE triggers the usage reporting procedure and creates the corresponding usage information report when the reporting criteria are met.

11.	5G ProSe UE-to-Network Relay UE sends the usage information report to the ProSe NF (CTF).

NOTE 1:	Step 10 and Step 11 can occur before Step 8 and Step 9.

11ch-a. Upon reception of Direct Communication usage information report, the CTF(ADF) triggers the Charging Data Request [Event]. The CTF(ADF) sends Charging Data Request [Event] to CHF.

11ch-b.  The 5G ProSe Direct communication via UE-to-Network Relay CDR is generated by CHF for the Relay UE.

11ch-c. The CHF acknowledges by sending Charging Data Response [Event] to the CTF(ADF).

NOTE 2:	The procedure applies to Remote UE to UE-to-Network Relay UE independently, i.e. each of the UE sends the respective usage information reports according to different reporting criteria.

6.2.4.5.3.2	Message flow for ProSe Direct Communication via Layer-2 UE-to-Network Relay

Figure 6.2.4.5.3.2-1: Message flow for ProSe Direct Communication via Layer-2 UE-to-Network Relay

1-8. These steps are the same as message flow for ProSe UE-to-Network Direct Communication via Layer-2 procedures described in TS 23.304 [11] clause 6.5.2.2.

9-11.  These steps are the same as message flow for Layer-3 UE-to-Network Relay in figure 6.2.4.x.3.1-1.

6.2.5	Evaluation

6.2.6	Conclusion

6.3	Scenario 3: 5GS charging for ProSe Direct Communication based on QoS flow

6.3.1	General description and assumptions

For NR based PC5, a QoS model similar to that defined in TS 23.501 [7] for Uu reference point is used, i.e. based on 5QIs, a PC5 QoS Flow is associated with a PC5 QoS Rule and the PC5 QoS parameters as defined in TS 23.287 [2] clauses 5.4.2, 5.4.3 and 5.4.4.

The UE may be configured with a set of default PC5 QoS parameters, as defined in TS 23.287 [2] clause 5.1.2.1. For NR based unicast, groupcast and broadcast mode communication over PC5, Per-flow QoS model for PC5 QoS management could be applied. This may need to be charged differently based on the monitored QoS for them respectively.

For Direct communication via ProSe UE-to-Network Relay, PC5 QoS handling to support end-to-end QoS is defined in TS 23.304 [11].

The end-to-end QoS for a relay service handling is supported as shown in figure below.

Figure 6.3.1-1: End-to-End QoS translation for Layer 3 UE-to-Network Relay

In case the QoS Flows setup are initiated by network, PCF or SMF decides the Uu part QoS parameters.

In case that the Remote UE requested dedicated PC5 QoS Flows when establishing the L2 Link over PC5, the Remote UE gets the QoS mapping on the Relay UE via PC5 and decides the PC5 part QoS parameters (i.e. PQI).

6.3.2	Potential charging requirements

REQ-CH_PROSE_5GS_QBC-01: The 5GS should support collect charging information per PC5 Qos Flow.

6.3.3	Key issues

6.3.3.1	Key issue #3.1: QoS flow Based Charging for ProSe Direct Communication

This key issue is for investigating how to support QoS flow Based Charging for ProSe Direct Communication considering REQ-CH_PROSE_5GS_QBC-01.

This investigation covers the following:

-	determination of which entity/entities in the 5G system are suitable to provide the charging information for QoS flow Based Charging;

-	identification of the triggers for flow Based Charging for ProSe Direct Communication, and Direct Communication via UE-to-Network Relay.

6.3.4	Possible solutions

6.3.4.1	Solution #3.1: QoS flow Based Charging for ProSe Direct Communication for Key issues #3.1

6.3.4.1.1	Consideration for ProSe QoS flow Based Charging

The 5GS provides QoS handling mechanism defined in TS 23.304 [11] to support ProSe Direct Communication, the one-to-one mapping of standardized PQI values that are additionally defined to PC5 QoS characteristics. 5G ProSe UE could collect charging information related to data volumes categorized per PC5 QoS Flow identified by PQI.

The possible solutions to support the potential requirements REQ-CH_PROSE_5GS_QBC-01 and Key Issue #3.1 could be the enhancements or extensions to the ProSe Direct Communication and/or ProSe Direct Communication via UE-to-Network relay defined in clause 6.2.

The 5GS could re-use the Service authorization and provisioning mechanism defined in TS 23.304 [11] clause 5.1.3, the UE could be configured with information to be included in the usage information report as per following:

-	Whether the QoS flow infomation need to be reported;

If QoS flow information need to be reported in usage information report, the UE may start to collect QoS information by some trigger conditions, e.g. Start/End of a QoS flow, Start/End of PC5 layer-2 link, PC5 QoS parameters change of charging condition in the ProSe UE. When the UE decides that reporting criteria are met, according to the pre-configuration, the UE creates the corresponding usage information report with QoS information to NF(CTF) over PC3ch or to ProSe UE-to-Network Relay UE.

For ProSe QoS flow Based Charging, the chargeable events could be:

- 	Received Direct Communication Usage Report (for Broadcast, Groupcast, Unicast, including UE-to-Network Relay) with QoS information.

The 5GS may collect the QoS flow information in addition to the charging information described for ProSe Unicast/Groupcast/Broadcast mode of Direct Communication, including Direct Communication via UE-to-Network Relay:

-	PC5 QoS Flow Id;

-	Reprot Time;

-	QoS information;

-	QOS Characteristics;

6.3.4.1.2	Architecture Description

The solutions could reuse the possible architecture defined for ProSe Communication solutions.

6.3.4.1.3	Flow Description

The flow is the same as defined for ProSe Communication solutions with the following differences:

-	QoS flow related information may be reported in Usage information reporting message;

-	QoS flow related information may be included in Charging Data Request message toward CHF for CDR generation;

Editor's note:	It is FFS whether QoS information collection are needed by both remote and relay UE.

6.3.5	Evaluation

6.3.6	Conclusion

7	Conclusions and recommendations

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2020-10 | SA5#133e | - | - | - | - | Initial skeleton | 0.0.0 |
| 2020-10 | SA5#133e | S5-205469 S5-205470 |  |  |  | pCR Add skeleton to draft TR 32.846 pCR 32.846 Add scope and reference | 0.1.0 |
| 2020-11 | SA5#134e | S5-206436 S5-206437 S5-206438 S5-206439 S5-206440 S5-206441 |  |  |  | pCR 32.846 Add terms and abbreviations pCR 32.846 Add concepts and overview of ProSe charging in 5GS pCR 32.846 Add architecture considerations for ProSe charging in 5GS pCR 32.846 Add general description and assumptions pCR 32.846 Add potential charging requirements pCR 32.846 Add key issues | 0.2.0 |
| 2021-02 | SA5#135e | S5-211417 S5-211373 S5-211376 S5-211377 |  |  |  | pCR 32.846 Separating the topic for ProSe direct discovery and communication pCR 32.846 Add usecase for ProSe Direct Discovery pCR 32.846 Add requirement for ProSe Direct Discovery pCR 32.846 Add possible solution for ProSe Direct Discovery | 0.3.0 |
| 2021-03 | SA5#136e | S5-212469 S5-212134 S5-212135 S5-212136 |  |  |  | pCR 32.846 Add possible solution for ProSe Unicast mode Direct Communication pCR 32.846 Add topic for charging for ProSe Direct Communication based on QoS flow pCR 32.846 Update requirement and key issue pCR 32.846 Update solution for Direct Discovery charging | 0.4.0 |
| 2021-05 | SA5#137e | S5-213150 S5-213608 S5-213609 S5-213153 S5-213154 S5-213155 S5-213610 |  |  |  | pCR 32.846 Updating on Direct Discovery charging over PC5 reference point pCR 32.846 Updating on Direct Discovery charging with 5G DDNMF pCR 32.846 Add possible solution for ProSe Broadcast mode Direct Communication pCR 32.846 Add possible solution for ProSe Groupcast mode Direct Communication pCR 32.846 Correction on solution for unicast Direct Communication charging pCR 32.846 Updating description and requirements for Direct Communication charging pCR 32.846 Adding CTF based architecture for direct communication Additional changes based on EditHelp review | 0.5.0 |
| 2021-06 | SA#92e | SP-210394 |  |  |  | Presented for information | 1.0.0 |
| 2021-08 | SA5#138e | S5-214693 S5-214694 S5-214695 |  |  |  | pCR 32.846 Updating descriptions on 5G ProSe pCR 32.846 Add possible solution for ProSe UE to Network relay pCR 32.846 Add possible solution for ProSe Direct Communication charging based on QoS flow | 1.1.0 |