3GPP TSG-SA5 Meeting #139-e	S5-215132

# Online, , 11th Oct 2021 - 20th Oct 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.659 | CR | 0039 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-17 CR TS 28.659 Provide YAML solution set for EUTRAN NRM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NSA_SBMA |  |  |  |  |  | Date: |  |  | 2021-09-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | A YAML solution set is needed for EUTRAN NRM,but haven’t been defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A YAML solution set is needed for EUTRAN NRM is provided,mainly including following basic IOCs: - ENBFunction - EUtranGenericCell - EUtranCellTDD - EUtranCellFDD Other IOCs in TS 28.658 will be added in future SA5 meetings |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2,4,Annex X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge Link: https://forge.3gpp.org/rep/sa5/MnS/tree/28.658_Rel17_CR0039_Provide_YAML_solution_set_for_EUTRAN_NRM |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 21.905: “Vocabulary for 3GPP Specifications”.

[2]	3GPP TS 32.153: “Telecommunication management; Integration Reference Point (IRP) technology specific templates, rules and guidelines”.

[3]	3GPP TS 32.600: "Telecommunication management; Configuration Management (CM); Concept and high-level requirements".

[4]	3GPP TS 28.658: "Telecommunications management; Evolved Universal Terrestrial Radio Access Network (E-UTRAN) Network Resource Model (NRM) Integration Reference Point (IRP): Information Service (IS)".

[5]	3GPP TS 32.300: "Telecommunication management; Configuration Management (CM); Name convention for Managed Objects".

[6]	3GPP TS 32.606: "Telecommunication management; Configuration Management (CM); Basic CM Integration Reference Point (IRP); Solution Set (SS) definitions".

[7]	3GPP TS 32.616: "Telecommunication management; Configuration Management (CM); Bulk CM Integration Reference Point (IRP); Solution Set (SS) definitions".

[8]	3GPP TS 28.623: “Generic network resources Integration Reference Point (IRP); Solution Set (SS) definition”.

[9]	W3C REC-xml11-20060816: "Extensible Markup Language (XML) 1.1 (Second Edition)".

[10]	W3C XML Schema Definition Language (XSD) 1.1 Part 1: Structures.

[11]	W3C XML Schema Definition Language (XSD) 1.1 Part 2: Datatypes.

[12]	W3C REC-xml-names-20060816: "Namespaces in XML 1.1 (Second Edition)".

[13]	3GPP TS 28.541: " Management and orchestration of 5G networks; Network Resource Model (NRM); Stage 2 and stage 3".

[Y]	3GPP TS 32.160: "Management and orchestration; Management Service Template".

2nd Change

4	Solution Set Definitions

This specification defines the following 3GPP E-UTRAN NRM IRP Solution Set Definitions:

-	3GPP E-UTRAN NRM IRP CORBA SS (Annex A).

-	3GPP E-UTRAN NRM IRP XML Definitions (Annex B).

-	3GPP E-UTRAN NRM YAML Definitions (Annex X)

3rd Change

Annex X (normative):
OpenAPI definition of the E-UTRAN NRM

X.1	General

This annex contains the OpenAPI definition of the E-UTRAN NRM in YAML format.

The Information Service (IS) of the E-UTRAN NRM is defined in 3GPP TS 28.658[4].

Mapping rules to produce the OpenAPI definition based on the IS are defined in TS 32.160 [Y].

X.2	OpenAPI document "EutranNrm.yaml"

openapi: 3.0.1

info:

title: EUTRAN NRM

version: 17.0.0

description: >-

OAS 3.0.1 specification of the EUTRAN NRM

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.659; EUTRAN NRM

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.659/

paths: {}

components:

schemas:

# Editor's Note in SA5#139emeeting: Following only include partial IOCs in TS 28.658, which needs further work for the whole EUTRAN NRM

#-------- Definition of types-----------------------------------------------------

CellLocalId_Type:

type: integer

minimum: 0

maximum: 255

CellLocalIdList_Type:

type: array

items:

$ref: '#/components/schemas/CellLocalId_Type'

CellSize_Type:

type: string

enum:

- verysmall

- small

- medium

- large

Mnc_Type:

type: string

pattern: '[0-9]{3}|[0-9]{2}'

PlmnId_Type:

type: object

properties:

mcc:

$ref: 'genericNrm.yaml#/components/schemas/Mcc'

mnc:

$ref: '#/components/schemas/Mnc_Type'

PlmnIdList_Type:

type: array

items:

$ref: '#/components/schemas/PlmnId_Type'

CellAccessInfo_Type:

type: object

properties:

plmnId:

$ref: '#/components/schemas/PlmnId_Type'

tac:

$ref: '#/components/schemas/Tac_Type'

cellId:

type: string

managementServiceExposed:

type: string

CellAccessInfoList_Type:

type: array

items:

$ref: '#/components/schemas/CellAccessInfo_Type'

Tac_Type:

type: integer

minimum: 0

maximum: 16777215

PCI_Type:

type: integer

minimum: 0

maximum: 503

PCIList_Type:

type: array

items:

$ref: '#/components/schemas/PCI_Type'

NbIoTcellFlag_Type:

type: string

enum:

- yes

- no

PartOfSectorPower_Type:

type: string

minimum: 1

maximum: 100

CellResvInfo_Type:

type: string

enum:

- reservedCell

- nonReservedCell

AllowedAccessClasses_Type:

type: string

enum:

- EmergencyCall

- ForPLMNUse

- SecurityServices

- PublicUtilities

- EmergencyServices

- PLMNStaff

IsChangeForEnergySavingAllowed_Type:

type: string

enum:

- yes

- no

NgranCellFlag_Type:

type: string

enum:

- yes

- no

IntraANRSwitch_Type:

type: string

enum:

- on

- off

IRATANRSwitch_Type:

type: string

enum:

- on

- off

ENBId_Type:

type: integer

minimum: 0

maximum: 268435455

X2IpAddress_Type:

type: string

X2IpAddressList_Type:

type: array

items:

$ref: '#/components/schemas/X2IpAddress_Type'

TceIDMappingInfo_Type:

type: object

properties:

tceID:

type: integer

tceIPAddr:

type: string

TceIDMappingInfoList_Type:

type: array

items:

$ref: '#/components/schemas/TceIDMappingInfo_Type'

SharNetTceMappingInfo_Type:

type: object

properties:

plmnId:

$ref: '#/components/schemas/PlmnId_Type'

tceIDMappingInfo:

$ref: '#/components/schemas/TceIDMappingInfo_Type'

SharNetTceMappingInfoList_Type:

type: array

items:

$ref: '#/components/schemas/SharNetTceMappingInfo_Type'

RS_pattern_Type:

type: string

enum:

- CRS

- CRS_PRS

Number_of_CRS_ports_Type:

type: integer

enum:

- 1

- 2

Periodicity_Type:

type: integer

enum:

- 1280

- 2560

- 5120

- 10240

Offset_Type:

type: integer

minimum: 0

maximum: 10239

NetListeningRSForRIBS_Type:

type: object

properties:

rS_pattern:

$ref: '#/components/schemas/RS_pattern_Type'

number_of_CRS_ports:

$ref: '#/components/schemas/Number_of_CRS_ports_Type'

periodicity:

$ref: '#/components/schemas/Periodicity_Type'

offset:

$ref: '#/components/schemas/Offset_Type'

LWIPSeGWIpAddress_Type:

type: string

LWIPSeGWList_Type:

type: object

properties:

LWIPSeGWId_Type:

type: string

LWIPSeGWIpAddressList_Type:

$ref: '#/components/schemas/LWIPSeGWIpAddress_Type'

#-------- Definition of abstract IOCs --------------------------------------------

EUtranGenericCell-Attr:

type: object

properties:

cellLocalId:

$ref: '#/components/schemas/CellLocalId_Type'

cellLocalIdList:

$ref: '#/components/schemas/CellLocalIdList_Type'

cellSize:

$ref: '#/components/schemas/CellSize_Type'

plmnIdList:

$ref: '#/components/schemas/PlmnIdList_Type'

cellAccessInfoList:

$ref: '#/components/schemas/CellAccessInfoList_Type'

tac:

$ref: '#/components/schemas/Tac_Type'

pci:

$ref: '#/components/schemas/PCI_Type'

pciList:

$ref: '#/components/schemas/PCIList_Type'

maximumTransmissionPower:

type: integer

nbIoTcellFlag:

$ref: '#/components/schemas/NbIoTcellFlag_Type'

referenceSignalPower:

type: integer

pb:

type: integer

partOfSectorPower:

$ref: '#/components/schemas/PartOfSectorPower_Type'

relatedTmaList:

$ref: 'genericNrm.yaml#/components/schemas/DnList'

relatedAntennaList:

$ref: 'genericNrm.yaml#/components/schemas/DnList'

relatedSector:

$ref: 'genericNrm.yaml#/components/schemas/Dn'

cellResvInfo:

$ref: '#/components/schemas/CellResvInfo_Type'

allowedAccessClasses:

$ref: '#/components/schemas/AllowedAccessClasses_Type'

isChangeForEnergySavingAllowed:

$ref: '#/components/schemas/IsChangeForEnergySavingAllowed_Type'

operationalState:

$ref: 'genericNrm.yaml#/components/schemas/OperationalState'

administrativeState:

$ref: 'genericNrm.yaml#/components/schemas/AdministrativeState'

availabilityStatus:

type: string

ngranCellFlag:

$ref: '#/components/schemas/NgranCellFlag_Type'

plmnInfoList:

$ref: 'nrNrm.yaml#/components/schemas/PlmnInfoList'

# TBD: other IOCs in TS 28.658 name-contained by EutranGenericCell will be added later when they are defined

#-------- Definition of concrete IOCs --------------------------------------------

SubNetwork-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

$ref: 'genericNrm.yaml#/components/schemas/SubNetwork-Attr'

- $ref: 'genericNrm.yaml#/components/schemas/SubNetwork-ncO'

- type: object

properties:

SubNetwork:

$ref: '#/components/schemas/SubNetwork-Multiple'

ManagedElement:

$ref: '#/components/schemas/ManagedElement-Multiple'

# TBD: other IOCs in TS 28.658 name-contained by SubNetwork will be added later when they are defined

ManagedElement-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

$ref: 'genericNrm.yaml#/components/schemas/ManagedElement-Attr'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedElement-ncO'

- type: object

properties:

ENBFunction:

$ref: '#/components/schemas/ENBFunction-Multiple'

# TBD: other IOCs in TS 28.658 name-contained by ManagedElement will be added later when they are defined

ENBFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

intraANRSwitch:

$ref: '#/components/schemas/IntraANRSwitch_Type'

iRATANRSwitch:

$ref: '#/components/schemas/IRATANRSwitch_Type'

eNBId:

$ref: '#/components/schemas/ENBId_Type'

x2BlackList:

$ref: 'genericNrm.yaml#/components/schemas/DnList'

x2WhiteList:

$ref: 'genericNrm.yaml#/components/schemas/DnList'

x2IpAddressList:

$ref: '#/components/schemas/X2IpAddressList_Type'

tceIDMappingInfoList:

$ref: '#/components/schemas/TceIDMappingInfoList_Type'

sharNetTceMappingInfoList:

$ref: '#/components/schemas/SharNetTceMappingInfoList_Type'

netListeningRSForRIBS:

$ref: '#/components/schemas/NetListeningRSForRIBS_Type'

lWIPSeGWList:

type: integer

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

EUtranCellTDD:

$ref: '#/components/schemas/EUtranCellTDD-Multiple'

EUtranCellFDD:

$ref: '#/components/schemas/EUtranCellFDD-Multiple'

# TBD: other IOCs in TS 28.658 name-contained by ENBFunction will be added later when they are defined

EUtranCellTDD-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- $ref: '#/components/schemas/EUtranGenericCell-Attr'

- type: object

properties:

earfcn:

type: integer

sfAssignment:

type: integer

specialSfPatterns:

type: integer

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-ncO'

# TBD: other IOCs in TS 28.658 name-contained by EUtranCellTDD will be added later when they are defined

EUtranCellFDD-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- $ref: '#/components/schemas/EUtranGenericCell-Attr'

- type: object

properties:

earfcnDl:

type: integer

earfcnUl:

type: integer

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-ncO'

# TBD: other IOCs in TS 28.658 name-contained by EUtranCellFDD will be added later when they are defined

#-------- Definition of arrays for name-contained IOCs ----------------------

SubNetwork-Multiple:

type: array

items:

$ref: '#/components/schemas/SubNetwork-Single'

ManagedElement-Multiple:

type: array

items:

$ref: '#/components/schemas/ManagedElement-Single'

ENBFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/ENBFunction-Single'

EUtranCellTDD-Multiple:

type: array

items:

$ref: '#/components/schemas/EUtranCellTDD-Single'

EUtranCellFDD-Multiple:

type: array

items:

$ref: '#/components/schemas/EUtranCellFDD-Single'

#-------- Definitions in TS 28.663 for TS 28.532 ---------------------------------

resources-grNrm:

oneOf:

- $ref: '#/components/schemas/ManagedElement-Single'

- $ref: '#/components/schemas/ENBFunction-Single'

- $ref: '#/components/schemas/EUtranCellTDD-Single'

- $ref: '#/components/schemas/EUtranCellFDD-Single'

End of Changes