c3GPP TSG-SA5 Meeting #139-e 	S5-215086

e-meeting, 11 - 20 October 2021	(revision of S5-214171)

Source:	Huawei, China Telecom, CATT, AsiaInfo, China Unicom, China Mobile, Deutsche Telekom

Title:	New WID on Enhanced intent driven management service for mobile networks

Document for:	Approval

Agenda Item:	6.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: 	Enhanced intent driven management services for mobile networks

## Acronym: eIDMS_MN

## Unique identifier:

Potential target Release: Rel-18.

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No | X |  |  |  | X |
| Don’t know |  |  |  |  |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

### 2.2	Parent Work Item

| Parent Work Items |  |
| --- | --- |
| Unique ID | Title |

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 810027 | Intent driven management service for mobile networks | SA5 work item |

Dependency on non-3GPP (draft) specification: none.

## 3	Justification

Several typical scenarios including scenarios related to intent expectation for coverage assurance, intent expectation for throughput assurance, intent expectation for delivering radio network, and intent expectation for delivering radio service are specified in IDMS_MN WI. However, to use intent driven management approach to support more scenarios, such as intent expectation for capacity assurance, intent expectation for energy saving, intent for performance monitoring (consumer’s expression is for monitoring the performance (e.g. coverage performance, capacity performance) of the network. Then the MnS producer report the abstract network performance (e.g. Weak Coverage, Good Coverage) to MnS consumer instead of detailed measurements), more intent expectation needs to be described.

Management operations and information are also specified based on the defined scenarios. For the other scenarios to be specified, the management information needs to be defined accordingly.

When there are multiple intents in the MnS producer, the conflict between multiple intents is inevitable. Conflicting intents cannot be executed simultaneously, how to detect and resolve the conflict needs to be considered.

In order to allow the intent driven MnS consumer to get more information of the intent execution status, then the consumer can adjusts the intent based on the information, e.g. modify the intent target or constraint. Defining intent feedback information for different scenarios also needs to be considered.

Also there are discussion in other SDOs (e.g. TM Forum, ETSI ZSM) for intent driven management, the alignment/collaboration work for the intent driven management needs to be considered.

## 4	Objective

The objective of this work item is to enhance the intent driven management services of 5G networks by specifying the following aspects:

- Specify the management scenario and corresponding requirements related to network and service management (including but not limited to intent expectation related to radio network performance monitoring , radio network fault management, radio network energy saving, and radio network capacity).

- Enhance the intent information model to support the newly defined requirements for intent expectation related to corresponding management scenarios.

- Specify the new capabilities for intent driven management, which may include:

- Intent confliction detection and resolution, e.g. the intent confliction between energy saving intent and existing optimization intent.

- Intent capability obtaining, allowing MnS consumer to obtain which intent expectation capability (e.g. coverage target and corresponding value range, RAN UE throughput target and corresponding value range, recommendtations for values of the targets) can be fulfilled by MnS producer.

- Intent report (including the monitoring performance of targets in the intent expectation, recommendations when the intent cannot be fulfilled) from MnS producer to MnS consumer.

- Collaboration/Alignment for intent driven management with other SDOs (e.g. ETSI ZSM, TM Forum) can be considered

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 28.312 | -Add management scenario and corresponding requirements -Specify the new capabilities for intent driven management -Enhance the intent information model | June 2023(SA#100) |  |

## 6	Work item Rapporteur(s)

Xu Ruiyue, Huawei, Xuruiyue@huawei.com (Responsible for intent information model and new capabilities for intent driven management): primary Rapporteur

Lv Feibi,China Unicom : lvfb@chinaunicom.cn (Responsible for typical management scenario and corresponding requirements)

## 7	Work item leadership

SA WG5

## 8	Aspects that involve other WGs

Co-ordination with SA2, RAN3, ETSI ZSM where appropriate

## 9	Supporting Individual Members

Supporting IM name

Huawei

China Telecom

CATT

AsiaInfo

China Unicom

China Mobile

Deutsche Telekom