3GPP TSG-SA5 Meeting #139-e 	S5-215104

- e-meeting, 11 - 20 October 2021

- Source:	Huawei

- Title:	Add key issue on modelling of MnF

- Document for:	Approval

Agenda Item:	6.5.6

# 1	Decision/action requested

Discuss and approve on the proposal.

# 2	References

[1]	3GPP TS 28.622 Generic Network Resource Model (NRM). Integration Reference Point (IRP); Information Service (IS)

[2]	3GPP TS 28.533 Management and orchestration; Architecture framework

[3]	3GPP TR 28.925 enhancement of service based management architecture

[4]	S5-212088 TD Modeling of Managed Management Functions

[5]	S5-214399 Add key issue on modelling of MnF

# 3	Rationale

Observation#1: Management Function Definition and Concept in TS 28.533[2].

A Management Function (MnF) is a logical entity playing the roles of MnS consumer and/or MnS producer.

Management Function can be deployed as a separate entity or embedded in Network Function to provide MnS(s). Following figure 1 shows an example (on the left) which MnF deployed as a separate entity to provide MnS(s) and another example (on the right) which MnF is embedded in Network Function to provide MnS(s):

Figure 1：Examples of MnS deployment scenario

Observation#2: Network Element definition in TR 21.905.

Network Element: A discrete telecommunications entity which can be managed over a specific interface e.g. the RNC.

Observation#3: Network Function definition in TS 28.533.

Network Function (NF): defined in TS 23.501[31].

TS 23.501:

Network Function: A 3GPP adopted or 3GPP defined processing function in a network, which has defined functional behaviour and 3GPP defined interfaces.

NOTE 1:	A network function can be implemented either as a network element on a dedicated hardware, as a software instance running on a dedicated hardware, or as a virtualised function instantiated on an appropriate platform, e.g. on a cloud infrastructure.

Observation#3: ManagedElement, ManagedFunction IOC definition in TS 28.622.

MaangedElement: This IOC represents telecommunications equipment or TMN entities within the telecommunications network providing support and/or service to the subscriber. A ManagedElement IOC is used to represent a Network Element defined in TS 32.101[1] including virtualization or non-virtualization scenario.

ManagedFunction: This IOC is provided for sub-classing only. It provides attribute(s) that are common to functional IOCs. Note that a ManagedElement may contain several managed functions, a managed function may contain other managed functions as specified for the specific subclass.This IOC can represent a telecommunication function either realized by software running on dedicated hardware or realized by software running on NFVI.

Observation#4: SA5 has specified some management related features like DANRManagementFunction, DESManagementFunction, MDA, COSLA etc. Such features also need to be managed in the same way as managing network node in 3GPP management system.

Observation#5: The ManagedElement IOC is used to represent a Network Element. So we think the ManagedElement should contain at least one NetworkFunction.

Observation#6: we think it is needed to differentiate the model for  NF and MnF, we propose to keep the existing ManagedFunction for modelling Network Function, and add new ManagementFunction for modelling Management Function.

To summarize, the SBMA related concepts do not contain a generic corresponding IOCs to represent Management Function. And based on the figure 1 above, Management Function takes an important role to build the links with Network Element, Network Function and other Management Functions. Management Function is also needed to be managed from 3GPP management system using the same approach as managing network node. We propose to add corresponding ManagementFunction IOC to show the completeness of SBMA concept.

- 4	Detailed proposal

It is proposed to add a new Key issue on modelling for MnF. This document proposes the following changes in TR 28.925 [1].

1st Change

## 4.x	Issue #x: Use of Models in SBMA

### 4.x.1	Description

The following existing concepts are related to SBMA:

- TS 28.533 Management Function: A Management Function (MnF) is a logical entity playing the roles of MnS consumer and/or MnS producer. Management Function can be deployed as a separate entity or embedded in Network Function to provide MnS(s).

Figure 4.x-1: Examples of MnS deployment scenario

- TR 21.905 Network Element: A discrete telecommunications entity which can be managed over a specific interface e.g. the RNC.

- TS 28.533 / TS 23.501 Network Function: A 3GPP adopted or 3GPP defined processing function in a network, which has defined functional behaviour and 3GPP defined interfaces.

- The following IOCs are specified:

- TS 28.622 ManagedElement IOC: This IOC represents telecommunications equipment or TMN entities within the telecommunications network providing support and/or service to the subscriber. A ManagedElement IOC is used to represent a Network Element defined in TS 32.101[1] including virtualization or non-virtualization scenario.

- TS 28.622 ManagedFunction IOC: This IOC is provided for sub-classing only. It provides attribute(s) that are common to functional IOCs. Note that a ManagedElement may contain several managed functions, a managed function may contain other managed functions as specified for the specific subclass.This IOC can represent a telecommunication function either realized by software running on dedicated hardware or realized by software running on NFVI.

| TR 21.905 Network Element |  |
| --- | --- |

4.x.2	Potential solutions

This clause provides the description of using related NRM to represent the relations between management function, network function, and nework element.

Management function could be deployed in following different deployment scenarios:

- The Management Function can be deployed in management system, including

  - The Management Function deployed in domain management system.

  - The Management Function deployed in cross domain management system.

- The Management Function deployed in Network Element.

Management functions also need to be managed and modeled in the same approach as managing network node in 3GPP management system.

The following table shows the relation between the concepts and related models:

| Concepts | Illustration of Related Management Models |
| --- | --- |
| TS 28.533 Management Function | A ManagementFunction IOC is used to represent a generic management function which to be managed in 3GPP Management system. The following IOCs are example of specific ManagementFunction IOC. TS 28.541 DANRManagementFunction IOC TS 28.541 DESManagementFunction IOC TS 28.541 CESManagementFunction IOC |
| TR 21.905 Network Element | A network element is represented by ManagedElement IOC defined in TS 28.622. |
| TS 28.533 / TS 23.501 Network Function | A Network function is represented by ManagedFunction IOC defined in TS 28.622. |

End of change