3GPP TSG SA WG5 Meeting 139-e		S5-215162

- Electronic meeting, online, 11 - 20 October 2021

- Source:	Huawei

- Title:	Add MDA common attributes

- Document for:	Approval

Agenda Item:	6.4.18

# 1	Decision/action requested

This contribution is for approval.

# 2	References

[1]	TR 28.809 Management and orchestration; Study on enhancement of Management Data Analytics (MDA)

[2]	3GPP TS 28.104-000 “Management and orchestration; Management Data Analytics”

# 3	Rationale

Multiple MDA capabilities are described in [1]. To trigger MDA procedures and subscribe desired MDA reports by the MDAS consumer, the MDA service has the capability to support the consumer to set input parameters. And for different MDA types, some common attributes including input and output can be defined.

This contribution is proposed to define the common element information for MDA.

# 4	Detailed proposal

It is proposed to apply following changes in [2].

First of changes

## X.Y	Common information elements of MDA reports

### X.Y.1	Descriptions

MDA provider supports multiple capabilities of network and service analyses and provides the desired analytics reports to MDA comuser. For different network issues, besides the specific information elements provides by the analytics reports, some common information also exsits.

Clause X.Y.2 provides the common information elements of MDA reports.

### X.Y.2	Common output elements

The common information elements of MDA reports is provided in table X.Y.2:

Table X.Y.2x-1:  Common information elements of MDA reports

| Information | Description | Support qualifier | Properties |
| --- | --- | --- | --- |
| MDAIdentifier | MDAIdentifier indicates the MDA type of the analytics | M | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| TimeStamp | TimeStamp indicates the time of the Analytics report generation. | M | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| AnalyticsStartTime | StartTime indicates the start time of the information which are used for analytics. | O | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| TimeDuration | TimeDuration indicates the duration time of the information which are used for analytics. | O | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| AffectedObjects | The affected objects of the MDA, e.g. the subnetworks, S-NSSAI. | M | type: DN multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

End of changes