- 3GPP TSG-SA5 Meeting #139-e 	S5-215166

- e-meeting, 11 - 20 October 2021

- Source:	China Mobile, Huawei, AsiaInfo, China Unicom

- Title:	Update the description for generic functional requirements

- Document for:	Approval

Agenda Item:	6.4.9

# 1	Decision/action requested

The group is asked to discuss and approval.

# 2	References

[1]	3GPP 	draft TS 28.100: "Management and orchestration; Levels of autonomous network 0.6.0".

# 3	Rationale

This contribution proposes to update the following generic function requirements to generic autonomy capability description:

-	Clause 7.1.3

-	Clause 7.2.3

-	Clause 7.3.3

# 4	Detailed proposal

It proposes to make the following changes to TS 28.100[1].

1st Change

### 7.1.3 Generic autonomy capability description for management system

##### Level 1 for Network Optimization: The 3GPP management system has the following autonomy capabilities:

- Adjust network based on the specified network adjustment solution.

- Collect network related information (including network performance data, network configuration data and environment data) based on specified collection rules.

##### Level 2 for Network Optimization: The 3GPP management system has following autonomy capabilities:

- Identify the network issue based on the specified network issue identification rule.

- Demarcate the network issue (including network issue categories based on the specified network issue demarcation rule.

- Analyse the root cause of network issue based on the specified network issue analysis rule.

##### Level 3 for Network Optimization:  The 3GPP management system has the following autonomy capabilities:

- Analyse and generate the recommended network adjustment solutions based on specified network adjustment policies.

- Evaluate the recommended network adjustment solutions and determine the network adjustment solutions to be executed based on specified network adjustment decision policies.

- Predict network performance deterioration.

##### Level 4 for Network Optimization: The 3GPP management system has the following autonomy capabilities:

- Determine or update network optimization policies according to network optimization intent based on specified intent translation policies.

- Evaluate network optimization intent fulfilment result based on specified intent evaluation policies.

##### Level 5 for Network Optimization: The 3GPP management system has the following autonomy capabilities:

- Generate the network optimization intent translation and evaluation policies.

2nd Change

### 7.2.3 Generic autonomy capability description for management system

##### Level 1 for RAN NE deployment: The 3GPP management system has the following autonomy capabilities:

-	Download and activate the specified RAN NE network configuration data and software.

- Collect RAN NE information based on the specified RAN NE information collection rule.

##### Level 2 for RAN NE deployment: The 3GPP management system has the following autonomy capabilities:

- Analyse the network configuration data for the RAN NE based on the specified network configuration data generation rules.

##### Level 3 for RAN NE deployment: The 3GPP management system has the following autonomy capabilities:

- Analyse and determine the network configuration data for the RAN NE based on the specified network configuration data generation policies.

- Perform the RAN NE commissioning test based on specified commissioning test policies.

-	Perform the RAN NE dialing test based on specified dialing test policies.

##### Level 4 for RAN NE deployment: The 3GPP management system has the following autonomy capabilities:

-	Determine or update RAN NE deployment policies according to RAN NE deployment intent based on specified intent translation policies.

-	Evaluate RAN NE deployment intent fulfilment result based on specified intent evaluation policies.

##### Level 5 for RAN NE deployment: The 3GPP management system has the following autonomy capabilities:

- Generate the RAN NE deployment intent translation and evaluation policies.

3nd Change

### 7.3.3 Generic autonomy capability description for management system

##### Level 1 for Fault Managemene: The 3GPP management system has the following autonomy capabilities:

- Execute the network recovery actions (including reset the hardware/software, switch to the backup hardware, rollback to the backup software/firmware) with corresponding network elements based on the specified fault recovery rules.

- Obtain fault related data (including alarms, events, signaling data, performance data, configuration data and environment data) based on specified collection rules.

- Filter the alarms based on the specified filtering rules.

##### Level 2 for Fault Management: The 3GPP management system has the following autonomy capabilities:

- Identify the fault based on specified fault recognition rules.

- Demarcate the fault type and corresponding affected managed object (e.g. NE) based on specified fault demarcation rules.

##### Level 3 for Fault Management: The 3GPP management system has the following autonomy capabilities:

- Analyse the root cause of the network fault based on specified fault root cause analysis policies.

- Analyse and generate the recommended fault recovery mechanism and determine the fault recover actions to be executed based on specified fault recovery mechanism analysis and decision policies.

- Predict the potential fault.

##### Level 4 for Fault Management: The 3GPP management system has the following autonomy capabilities:

- Generate or update fault management policies according to fault management intent based on specified intent translation policies.

- Evaluate fault management intent fulfilment based on specified intent evaluation policies.

##### Level 5 for Fault Management: The 3GPP management system has the following autonomy capabilities:

- Generate the fault management intent translation and evaluation policies.

End of changes