3GPP TSG-SA5 Meeting #139-e 	S5-215204rev1

- e-meeting, 11 - 20 October 2021

- Source:	Huawei

- Title:	pCR Add alarm incident analysis solution

- Document for:	Approval

- Agenda Item:	6.4.18

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1]	3GPP TR 28.809 Management and orchestration; Study on enhancement of Management Data Analytics (MDA)

[2]	3GPP TS 28.104-000 “Management and orchestration; Management Data Analytics”

# 3	Rationale

This contribution is proposed to add MnS component for alarm incident analysis capability of MDAS in [2].

# 4	Detailed proposal

It is proposed to apply following changes in [2].

1st modified section

## 8.4	Data definitions per MDA capability

### 8.4.X	Fault management related issues

#### 8.4.X.1	Alarm Incident Analysis

##### 8.4.X.1.1	MDA type

The MDA type for Alarm incident analysis is: FaultManagement. AlarmIncidentAnalysis.

##### 8.4.X.1.2	Enabling data

##### 8.4.X.1.3	Analytics output

The specific information elements of the analytics output (MDA report) for Alarm incident analysis, in addition to the common information elements of the analytics outputs (see clause 8.3), are provided in table 8.4.X.1.3-1.

Table 8.4.X.1.3-1: Analytics output for Alarm incident analysis

| Information | Description | Support qualifier | Properties |
| --- | --- | --- | --- |
| Incident ID | Indicates the identifier of incident. | M | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| Affected Objects | The affected objects (e.g., the MOIs of cells affected by the alarm incident) | O | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| Incident Raisedtime | Indicates the date and time when the incident is first raised. | M | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| Incident Clearedtime | Indicates the date and time when the incident is cleared. | M | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| Root Cause | Indicates the root cause of incident. | O | type: FFS multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| Severity Level | It indicates the severity level of the alarm incident. Values: Critical, Major, Minor, Warning, Indeterminate, Cleared | O | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

MnS component type C

End modified section