3GPP TSG-SA5 Meeting #139-e 	S5-215242

- e-meeting, 11 - 20 October 2021

- Source:	Ericsson

- Title:	Addition of overview

- Document for:	Approval

Agenda Item:	7.5.6

# 1	Decision/action requested

Include the proposed changes in TR 28.826.

# 2	References

[1]		3GPP TR 28.826: " Study on Nchf charging services phase 2 improvements and optimizations"

# 3	Rationale

Addition of overview.

# 4	Detailed proposal

First change

# 4	Overview

The Nchf converged charging services was first released in Rel-15. This first iteration was to a large extent, with some minor exceptions, copying and combining its predecessors, Ro and Rf, based on Diameter to a service based interface (SBI) supporting converged charging service. The depiction of the internal architecture of the Converged Charging System (CCS) was to a large extent based on the Online Charging System (OCS).

The combination of the Diameter Ro and Rf interfaces into one converged charging service together with the move of the integration point for CDRs have led to that all the information previously reported and transported over two or more interfaces now needs to be transported over a single interface. This makes it essential that the converged charging interface is optimized in regard to the amount of information and number of reports sent, or at least allows for such an optimization if needed.

Low latency requirements increased by new business cases may need enhancements to the non-blocking mechanism.

Charging of new service together with converged charging have increased the need for more information in the input to rating.

The use of event-based charging has led to a need for cancelling the charging for the chargeable event, e.g. refund, for unsuccessful scenarios or cases not possible to fulfil.

The document structure of the actual service, i.e. yaml, followed the handling of Ro and Rf, based on Diameter, which may need improvements.

End of changes